3GPP TSG-RAN WG4 Meeting # 106-bis-e											         R4-2306353

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.23.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for LPHAP

Document for:	Approval

- Sub-Topic #1: eDRX cycle beyond 10.24s in INACTIVE

Issue 1-1-1: PRS measurement requirements

- Agreement (from GTW)

  - Define requirements for normal (non-RedCap) and RedCap type of devices

  - Define measurement requirements when eDRX is larger than 10.24s. FFS whether to define requirements for eDRX cycle less or equal to 10.24s

- Wayforward

  - FFS whether to define requirements for eDRX <= 10.24s

    - Option 1: Define requirements for eDRX <= 10.24s

    - Option 2: Not to define requirements for eDRX <= 10.24s

    - Option 3: Define requirements for eDRX <= 10.24s for RedCap UE, and FFS for non-RedCap UE

    - Option 4: other

Issue 1-1-2: Baseline for new PRS measurement requirements

- Wayforward

  - FFS how to define requirements for eDRX <= 10.24s, if requirements are to be defined

    - Option 1: the requirements for RRC_INACTIVE state in R17 can be reused as baseline

    - Option 2: other

  - FFS how to define requirements for eDRX cycle larger than 10.24s.

    - Option 1: the requirements for RRC_INACTIVE state in R17 are reused as baseline

    - Option 2: the requirements for eDRX cycle > 10.24 sec in Rel-18 eRedCap WI are considered

    - Option 3: other

Issue 1-1-3: Adaptations for new PRS measurement requirements

- Agreement

  - The following R17 enhancements for latency reduction are supported for PRS measurement with eDRX in RRC_INACTIVE

    - Reduced number of PRS samples

    - parallelPRS-MeasRRC-Inactive-r17 capability

    - Lower Rx beam sweeping in FR2

- Wayforward

  - FFS whether ONLY reduced number of samples is considered with a higher SINR side condition

  - FFS whether and how to take the alignment between eDRX and PRS configuration into account based on RAN2 progress

  - FFS whether to define a scaling factor related to eDRX in the period requirement calculation

  - FFS whether and how to update Tavailable in the requirements when eDRX cycle is much larger than positioning interval

  - Other adaptations are not precluded

Issue 1-1-4: RRM measurement requirements

- Wayforward

  - FFS whether and how to define RRM measurement requirements for positioning specific needs with > 10.24s eDRX cycle in RRC_INACTIVE

    - RRM measurement requirements from eRedCap WI are to be considered as baseline

    - Positioning specific needs are to be considered

    - Note the eDRX cycle can be much larger than the positioning interval

# Sub-Topic #2: SRS positioning validity area

Issue 1-2-1: General impacts of SRS positioning validity area

- Wayforward

  - Subject to progress in RAN1 and RAN2, RAN4 to investigate and specify the RRM impacts, if any, due to the introduction of SRS positioning validity area.

    - The applicability of UE Rx-Tx measurement requirements will be studied, and revisited/updated if necessary, based on RAN1 and RAN2 agreements on SRS positioning validity area, details FFS.

    - Other RRM impacts are not precluded.

Issue 1-2-2: Detailed impacts of SRS positioning validity area

- Wayforward

  - The issue is merged into issue 1-2-1.

# Sub-Topic #3: PRS measurement in IDLE

Issue 1-3-1: New PRS measurement requirements

- Agreement (from GTW)

  - Define requirements for normal (non-RedCap) and RedCap type of devices

  - Requirements for PRS measurement in INACTIVE, both with and without eDRX, are re-used for PRS measurement in IDLE.

Issue 1-3-2: Adaptations for new PRS measurement requirements

- Agreement

  - RAN4 will specify NR positioning requirements in RRC_IDLE for DL-only positioning methods and measurements.

    - Note: UE Rx-Tx is not considered as DL-only measurement, and no requirement would be defined for UE Rx-Tx in RRC_IDLE.

  - For UEs capable of performing RRM measurement and PRS measurement in parallel to each other in RRC_IDLE mode Kcarrier_PRS = 1.

  - For UEs that are not capable of performing RRM measurement and PRS measurement in parallel to each other in RC_IDLE mode then Kcarrier_PRS is calculated as:

    - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4 of 38.133.

    - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7 of 38.133.

- Wayforward

  - FFS whether the reporting delay for PRS measurements performed in RRC_IDLE needs to account for extra delay to transition to RRC_CONNECTED state.

  - FFS whether to reuse values from Rel. 17 for NRx,TEG,i and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state.

  - FFS whether reduced sample number are considered in requirements for PRS measurement in RRC_IDLE

  - FFS whether Rel. 17 accuracy requirements in RRC_INACTIVE state for Nsample = 4 also apply to positioning measurements in RRC_IDLE.

  - FFS impact of paging cycle on positioning measurement delay requirement

  - Other adaptations are not precluded