3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306566

Electronic Meeting, 17 – 26 April, 2023

Title: 	WF on UE RF requirements for 5G broadcast

Agenda Item:	6.3.3

Source: 	Qualcomm Incorporated

Document for:	Approval

# 1. Specification of Channel bandwidth

Channel bandwidth of 10 MHz for the UE has been assumed, but there are differing views on whether this detail needs to be included in the specification.

-	Option 1: Include 10 MHz channel bandwidth and 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth (a la R4-2305824)

-	Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail.  The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths

-	Option 3: Include 10 MHz channel bandwidth, but instead of referring to 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth, refer to PMCH-bandwidth of 30, 35, and 40 RB’s as specified in 36.331.

Way forward:  FFS these options

# 2. Refsens

-	The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with

Option 1.  6, 7, and 8 MHz PMCH for reference sensitivity

Option 2.  10 MHz PDSCH for reference sensitivity

Way forward:  FFS these options

# 3. ACS

Option 1:  33 dB (for all 6, 7, and 8 MHz bandwidths)

Option 2:  29 dB (for all 6, 7, and 8 MHz bandwidths)

Option 3:

| Channelization | Power in transmission bandwidth configuration | PInterferer | ACS |
| --- | --- | --- | --- |
| 10 MHz (baseline) | REFSENS + 14 dB | REFSENS + 45.5 | 33 dB |
| 6 MHz | REFSENS + 14 dB | REFSENS + 28.5 | [16] dB |
| 7 MHz | REFSENS + 14 dB | REFSENS + 30.0 | [17.5] dB |
| 8 MHz | REFSENS + 14 dB | REFSENS + 35.0 | [22.5] dB |

Way forward:  FFS these options