3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2306579

Online, April 17 – April 26, 2023

Agenda item:			6.6.6

Source:	MediaTek Inc.

Title:	WF on IoT_NTN_FDD_LS_band

Document for:	Approval

- Proposals relevant to Work Plan and Draft TR

Issue 1-1: Work plan for WI IoT_NTN_FDD_LS_band core part

  - Option 1: Approve revised R4-2304496 with following modification:

4)	RAN4#108bis (October 2023, F2F)

  - Option 2: Other

WF:

- Agree Option 1

Issue 1-2: Draft TR for IoT NTN bands

WF:

The Draft TR skeleton R4-2304497 is endorsed

- Proposals relevant to Sytem parameter, UE RF and CR

Issue 2-1: Operating band

- Proposal 1: Band 254 revised to Band [254]

  - To revise Draft CR R4-2304803 and R4-2304805

WF:

- Agree Proposal 1.

Issue 2-2: Channel bandwidth

- Proposals

  - Option 1: The addition of Band [254] does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2.

Agreement in the 1st round

Agree Option1.

Issue 2-3-1: Channel spacing

- Proposals

  - Option 1: Channel spacing sub-clauses keep untouched with the addition of Band [254] for category M1 and category NB1/NB2.

Agreement in the 1st round

Agree Option1.

Issue 2-3-2: Channel raster

- Proposals

  - Option 2:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [254] | 100 | 2483.5 | 228571 | 228571-<1>-228735 | 1610 | 261339 | 261339-<1>-261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Agreement in the 1st round

Agree Option2.

Issue 2-3-3: Tx-Rx frequency separation

- Proposal 1: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz as starting point

- Proposal 2: To discuss variable duplex and asymmetric UL/DL operation next RAN4#107 meeting if there are more inputs.

WF:

- Agree Proposals 1 and 2.

Issue 3-1-1: UE maximum output power

Option 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively.

Table: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [254] |  |  | 23 | +/-2 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [254] | 23 | +/-2 |  |  |

WF:

- Agree Option 1.  The inclusion of PC5 is under study.

Issue 3-1-2: UE maximum output power reduction

- Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 as the starting point

WF:

- Agree Option 1.

Issue 3-2-1: In-band blocking

- Proposals:

  - Option 1: Reuse the 36.102 in-band blocking requirements

Agreement in the 1st round

Agree Option1.

Issue 3-2-2: Narrow-band blocking

- Proposals:

  - Option 1: Reuse the 36.102 narrow-band blocking requirements

Agreement in the 1st round

Agree Option1.

Issue 3-2-3: RX spurious emission

- Proposals:

  - Option 1: Reuse the 36.102 spurious emission requirements

Agreement in the 1st round

Agree Option1.