3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2306589

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Work plan for enhancement for sub-1GHz band combinations

Agenda Item:	5.4.1

Document for:	Approval

Introduction

New SID [1] for enhancement for sub-1GHz band combinations was approved in RAN #99 meeting. Here we provide a work plan based on the objectives for this new SI.

Objectives of the SI

The objectives of core part for enhancement for sub-1GHz band combinations are copied as below:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n28A-n105A | CA_n28A-n105A | DL_n28A-n105A_UL_n28A_BCS0 (New) DL_n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | Telstra |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A | DL_n5A-n28A_UL_n5A-n28A_BCS0 (Ongoing in a separate WI) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n5A-n105A | DL_n5A-n105A_UL_n5A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n28A-n105A | DL_n28A-n105A_UL_n28A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | CA_n28A-n105A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A CA_n28A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

  - The following aspects need be studied

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

NOTE 1: The study of 3 band combination can only start after completion of 2 bands fallbacks

NOTE 2: Check at RAN4#107 whether CA_n5A-n28A-n105A could be included in the SI based on study progress for the fall back combinations listed in Table 1.

Work plan

The TU budget for this SI only include one quarter cited from [1]:

| Topics | 2Q'2023 |  |  |  |
| --- | --- | --- | --- | --- |
|  | RAN4 #106b |  | RAN4 #107 |  |
|  | RF | RD | RF | RD |
| Study on enhancement for sub-1GHz NR band combinations | 0.5 |  | 0.5 |  |

The work plan for this SI is proposed as below.

Proposal: Approve the following work plan for the SI FS_NR_sub1GHz_combo_enh.

RAN4#106bis:

- Study the RF architecture for the following band combinations, i.e. CA_n5A-n105A, CA_n28A-n105A and CA_n26A-n28A for both UL and DL CA.

- Identify the critical issues and potential RF impacts for the band combinations above.

RAN4#107:

- Discuss the potential solutions for the identified issues for each band combination.

- Approve all of the TPs to capture the outcomes of this SI into TR 38.872.

- Check whether CA_n5A-n28A-n105A could be included in the SI based on study progress for the fall back combinations (i.e. CA_n5A-n105A, DL CA_n28A-n105A with UL band in either n28 or n105) and any limits on the uplink fall back combinations.

Note: If the answer is yes, it’s up to RAN plenary whether to extend this SI for further study on CA_n5A-n28A-n105A or just close this SI without conclusion on CA_n5A-n28A-n105A.

- References

[1] RP-230789, Study on enhancement for sub-1GHz NR band combinations, Spark NZ, Huawei, HiSilicon