3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304005

Online Meeting, 17 – 26 April 2023

3GPP TSG RAN WG1 #112	R1-2302087

Athens, Greece, February 27th – March 3rd, 2023

Title:	LS on interference modelling for duplex evolution

Response to:

Release:	Rel-18

Work Item:	FS_NR_duplex_evo

Source:	RAN1, CMCC

To:	RAN4

Cc:

Contact person:	Fei Wang

wangfei@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1		Overall description

In RAN1#112, RAN1 made the following agreements.

Agreement-1

Agree the updated TR for RAN1 in R1-2300997 in principle.

Agreement-2

Inform RAN4 of the updated RAN1 part of the TR. And include the following conclusion in the LS to RAN4.

Conclusion

Regarding the feasibility analysis of SBFD, RAN1 focus on feasibility analysis from performance perspective, specification perspective and impact on legacy operation perspective. The study on implementation feasibility is up to RAN4.

Agreement-3

For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows.

Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB

Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj

- Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj.

- Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale).

- NDLRBadj is the total number of DL RBs in adjacent channel.

- Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel.

- αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector.

  - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

  - FFS the concrete value of αadj, co-site-x

  - ACLRBS and ACSBS are in linear scale.

Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

Agreement-4

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB

where

- IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

- PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

- CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

- NULRB is the total number of UL RBs in the UL subband

- ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

- 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein,

  - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

  - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1

  - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

  - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

    - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

    - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

  - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

  - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

Working assumption:

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity, where,

- Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

  - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

  - NR is the number of Rx chains and NT is the number of Tx chains

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

  - yn=y0n,y1n,…,yRnT,

    - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

    - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

- Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB

  - Pblocker=1NR*m∈Used UL RBsHCLImWsm2

  - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

  - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

    - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

  - NULRB is the total number of UL RBs in the UL subbands,

  - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

# 2		Actions

To: RAN4

ACTION: 	RAN1 respectfully asks RAN4 to provide feedback on the above agreements and working assumption.

# 3		Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #112-bis-e                           17th Apr – 26th Apr, 2023			               E-meeting

TSG RAN WG1 Meeting #113                                    22nd May – 26th May, 2023			               Incheon, KR