3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2306665

Online Meeting, 17 – 26 April 2023

3GPP TSG RAN WG2 Meeting #121bis-e	R2-2304431

# e-meeting, 17-26 April 2023

Title:	Reply LS on intraBandENDC-Support

Response to:	R2-2300060 (R4-2220837)

Release:	Release 16

Work Items:	TEI16

Source:	RAN WG2

To:	RAN WG4

CC:

Contact Person:

Name:		Masato KITAZOE

E-mail Address:	mkitazoe@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 would like to thank RAN4 for their LS on intraBandENDC-Support.

RAN2 discussed RAN4’s suggestion regarding the interpretation of the UE capability parameter intraBandENDC-Support set to “both”. In RAN2, there were concerns raised on the forward compatibility of RAN4’s solution. RAN2’s understanding for “both” is that the UE supports both contiguous and non-contiguous intra-band EN-DC for the same band entries. Since there is no differentiation between DL and UL in the existing UE capability parameter intraBandENDC-Support, RAN4’s solution was considered non-backward compatible. RAN2 therefore agreed on a new signalling solution as outlined below, with pending confirmation from RAN4.

Principles

- A new UE capability parameter intraBandENDC-Support-UL, with the values {non-contiguous, both}, is introduced.

- The UE capability parameter intraBandENDC-Support-UL is only signalled when UL capability is different from the one indicated by the existing parameter intraBandENDC-Support.

- If the new parameter intraBandENDC-Support-UL is not signalled. the existing UE capability parameter intraBandENDC-Support indicates:

  - either the UE capability only for DL (when the UE supports intra-band EN-DC only in DL)

  - or the common capability for DL and UL (when the UE supports intra-band EN-DC in DL and UL)

- Support for non-contiguous intra-band EN-DC in DL and contiguous in UL is considered an invalid case.

- Early implementation from Release-15 shall be supported.

Solution description

With this solution, different intra-band EN-DC scenarios can be signalled as follows.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 | Non-contiguous | Absent | - Non-contiguous/Non-contiguous |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 5 | Both | Non-contiguous | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 7 | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

RAN2 would like to ask RAN4 to review the solution above and inform RAN2 whether it is agreeable from RAN4’s perspective.

# 2	Actions

To RAN4

ACTION:	RAN2 would like to ask RAN4 to review the UE capability signalling solution outlined in this LS and inform RAN2 whether it is agreeable from RAN4’s perspective.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN WG2 Meeting #122	22 – 26 May 2023	Seoul, Korea

TSG-RAN WG2 Meeting #123	21 – 26 August 2023	Toulouse, France