3GPP TSG-RAN WG4 Meeting #107		R4-2307484

# Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	NR-U PC3 UE RF requirements

Agenda item:	7.31.3

Document for:	Discussion

Introduction

In this paper, we would like to discuss the remained issue for Rel-18 NR-U enhancement.

Discussion

Based on the agreed WF[1], the issue is as follows.

Full and Partial RB - Further discuss streamlining of definitions for Full and Partial RB allocations and use the following as starting point - Full RB allocation: - when all RB’s in a 20 MHz channel, or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2 - Partial RB allocation: - when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted or when sub-bands are transmitted according to configurations in Table 6.2F.2-2

Full/Partial RB allocation for NR-U

In the last RAN4 meeting, the following comments were provided.

- Further improvement for Full/Partial RB allocation for NR-U is needed to solve the inconsistency between MPR and A-MPR.

- It should not alter the requirement of MPR and A-MPR.

Based on the comments, we propose the rewording of Full/Partial RB allocation for further clarification as follows.

    - Full RB allocation MPR/A-MPR applies when

  - all RBs in a 20 MHz channel are allocated and the sub-band is transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

    - Partial RB allocation MPR/A-MPR applies when

  - one or more RBs in a 20MHz channel are not allocated and the sub-band is transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and all sub-bands are transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

Proposal 1: Reword Full RB allocation and Partial RB allocation for NR-U MPR and NR-U A-MPR as follows

    - Full RB allocation MPR/A-MPR applies when

  - all RBs in a 20 MHz channel are allocated and the sub-band is transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

    - Partial RB allocation MPR/A-MPR applies when

  - one or more RBs in a 20MHz channel are not allocated and the sub-band is transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and all sub-bands are transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

Conclusion

The following is proposed as a rewording of Full/Partial RB allocation for NR-U MPR/A-MPR for further clarification. If the proposal is agreed, the related CRs are going to be submitted in the next meeting.

Proposal 1: Reword Full RB allocation and Partial RB allocation for NR-U MPR and NR-U A-MPR as follows

    - Full RB allocation MPR/A-MPR applies when

  - all RBs in a 20 MHz channel are allocated and the sub-band is transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or

  - all RBs in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

    - Partial RB allocation MPR/A-MPR applies when

  - one or more RBs in a 20MHz channel are not allocated and the sub-band is transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and all sub-bands are transmitted, or

  - one or more RBs in all sub-bands for wideband operation are not allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

- Reference

[1] R4-2306557, “WF on NR-U enhancement”, Apple Inc.