3GPP TSG-RAN WG4 Meeting #107	R4-2307936

# Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on TRP concept and test method for UL MIMO and TxD

Source:	Samsung

Agenda item:	8.16.2.1

Document for:	Discussion

1.	Introduction

In RAN4#106bis-e meeting, TRP concept was extensively discussed when UE is involved with multi-antenna and non-static antenna pattern. There was no conclusion but the group agreed to conclude this issue no later than RAN4#108 meeting [1]. Test method for single layer UL MIMO and TxD also depends on the conclusion for TRP concept. In this contribution, we share our views on above-mentioned issues to move forward.

2. 	Discussion

2.1	TRP concept

The main controversial issue of TRP concept is whether TRP could be based on dynamic pattern. Looking into previous definition, one definition is as following (TS37.544):

6.1.1.1 Definition and applicability The Total Radiated Power (TRP) is a measure of how much power the DUT actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere: Where is the solid angle describing the direction, is frequency. and are the orthogonal polarizations. and are the actually transmitted power-levels in corresponding polarizations. Thus In these formulas and are the number of sampling intervals for theta and phi. and are the measurement angles. The sampling intervals are discussed further in Subclause 4.4.

Another definition is as following (TS37.544):

6.1.5.1 Definition and applicability The Total Radiated Power (TRP) is a measure of how much power the antenna actually radiates, when non-idealities such as mismatch and losses in the antenna are taken into account. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere: Using to denote either or , is the -polarization component of the gain pattern for the handset antenna measured at the frequency , where is the solid angle describing the direction. is the transmit power level of the handset so that is the actually transmitted power-level, also known as EIRP, in the -polarization and in the direction for frequency . The above equation may be written in "gain" form, that is, the TRP given by PTRP is normalized to the transmitted power . This is the total radiation efficiency, which can also be denoted as Total Radiated Power Gain, TRPG, In practice discrete samples of are measured and used to approximate the integral so that the TRP is computed as Or, by using the relation : In gain form the TRP can be expressed as: In these formulas and are the sampling intervals for the - and -angles, respectively, and the number of samples in the - and -angles are given by and , respectively. The sampling points of the sphere are given by and . The sampling intervals are discussed further in Section 4.4. When measuring power radiated by active devices, expressing the data in terms of EIRP is more appropriate. The upper form of the TRP formulas (which includes EIRP terms) will be used in the data processing.

There seems no clear restriction for TRP definition to be applied for dynamic radiation pattern. However, the highlighted context above also indicates that the TRP definition was defined as an enhancement with active test to address the practical antenna radiation efficiency when non-idealities such as mismatch and losses in the antenna are taken into account.

Observation 1:	legacy TRP concept was defined as an enhancement with active test to address the practical antenna radiation efficiency when non-idealities such as mismatch and losses in the antenna are taken into account.

Without specific mention of dynamic radiation pattern, it should be understood as radiated power of static radiation pattern, otherwise, the radiation efficiency reflected by TRP could be larger than 100%. In FR2, TRP concept is used as radiated power of fixed beam, if TRP is applicable for dynamic beams, the TRP value would be possible larger than maxTRP requirement beyond the regulation.

Observation 2:	In existing specifications and regulations, TRP is based on static radiation pattern in practice.

On the other hand, there is demand to reflect the enhanced performance for UE supporting dynamic radiation pattern, and thus there may be new TRP variant in the future. To avoid inconsistency, it is better to make clear that legacy TRP is based on static radiation pattern, and potential new TRP concept should not bring contradiction with FR2 TRP definition. So we have following proposals:

Proposal 1:	TRP definition in FR1 TRP TRS should avoid inconsistency with FR2 TRP practice

Proposal 2:	‘TRP’ term if using alone should be based on static radiation pattern, which should be the basis of Rel-18

Proposal 3:	If TRP variant for dynamic radiation pattern would be introduced, a full name like ‘combined TRP’ or ‘envelop TRP’ and so on should be used without omitting, which can be further discussed in future release

Based on above proposals, our consideration for TRP of multi-antenna UE can be summarized as below table

| Multi-TX scenarios | Legacy TRP |  | Envelop combined TRP |  |
| --- | --- | --- | --- | --- |
| TAS | TRP = max (TRP1, TRP2) | TAS off | Can be defined in future | TAS on |
| TxD | To be defined in R18 | Test mode | Can be defined in future | TxD on |
| UL MIMO | To be defined in R18 | TPMI index 2 | Can be defined in future | Dynamic TPMI |
| FR2 | TRP with fixed beam | Beam locked | Out of scope | Beam unlocked |

Detailed discussion for UL MIMO and TxD can be found in section 2.2 and 2.3 respectively.

2.2	UL MIMO

The test method for TRP of single layer UL MIMO depends on the TPMI configuration during test. The options in WF [1] are as following:

Issue 2-2-1: TPMI-index configuration for singe-layer UL-MIMO TRP OTA test Proposals: - Option 1: Surface integral of measured EIRP, given fixed TPMI = 2 (NOTE: this metric is TRP-like if normalized by the radiated power of an ideal isotropic radiator) - Option 2: Surface integral of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum. - Option 3: Surface integral of measured EIRP for each TPMI swept over all applicable TPMI according to the UE capability to obtain TRP-like metric for each TPMI and then average the TRP-like metrics. - Option 3a: Define TRP for one-layer UL MIMO with TPMI 2-5 as the average of two TRP values with TPMI 2 and 3, or 4 and 5. - Option 4: Spherical coverage CDF of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum.

Option 4 is spherical coverage metric than TRP so it is obviously out of cope.

For Option 3, in theory, it is not necessary to test all the 2TX TPMIs. Different TPMI index means different relative phase between antennas, however, for each antenna, its TRP does not depend on its phase relationship with another antenna. Anyhow, total TRP = TRP1 + TRP2, TRP1 for antenna1 and TRP2 for antenna2 do not vary with relative phase. The measurement results shown in [2] also indicate that the TRP difference between different TPMIs small to 0.2dB which is smaller than measurement uncertainty. Note that 2TX test has larger measurement uncertainty with the same measurement grid.

Observation 3:	both theory and measurement demonstrate that it is not necessary to test all TPMIs.

Give some UEs only support TPMI index 2 and do not support TPMI index 3, 4 and 5, so the best choice to use fixed TPMI index 2, that is option 1.

Proposal 4:	adopt fixed TPMI index = 2 for single layer UL MIMO TRP test in Rel-18

For option 2 which is TRP based on dynamic radiation pattern, if new TRP variant will be introduced, it can be discussed later after legacy TRP test method for UL MIMO is completed.

2.3	TxD

Test method for TRP of TxD is more controversial due to phase variation issue. Note that there are two aspects for phase variation issue. One is phase variation versus time, the other is phase variation versus direction. The phase variation versus time for a fixed direction can be considered as unintentional. But for phase variation versus direction, it can be intentional, i.e., depending on implementation. The specification for TxD does not preclude intentional phase variation versus direction to improve the field performance.

Taking the phase variation versus direction into account, UE with TxD on mode actually works with dynamic radiation pattern. According to the discussion in section 2.1 on TRP concept, TxD on mode is not legacy TRP but dynamic TRP.

Observation 4:	TRP of TxD on mode is not legacy TRP but dynamic TRP due to potential phase variation versus direction.

To satisfy legacy TRP concept, static radiation pattern need to be maintained during TRP test, the only way is to apply test mode to disable phase variation versus direction if simultaneous 2TX is measured for TxD UE.

Proposal 5:	apply test mode to disable phase variation versus direction for simultaneous 2Tx TRP test of TxD UE in Rel-18

For TRP with TxD on mode based on dynamic radiation pattern, if new TRP variant will be introduced, it can be discussed later after legacy TRP test method for TxD is completed.

3. 	Conclusion

Observation 1:	legacy TRP concept was defined as an enhancement with active test to address the practical antenna radiation efficiency when non-idealities such as mismatch and losses in the antenna are taken into account.

Observation 2:	In existing specifications and regulations, TRP is based on static radiation pattern in practice.

Proposal 1:	TRP definition in FR1 TRP TRS should avoid inconsistency with FR2 TRP practice

Proposal 2:	‘TRP’ term if using alone should be based on static radiation pattern, which should be the basis of Rel-18

Proposal 3:	If TRP variant for dynamic radiation pattern would be introduced, a full name like ‘combined TRP’ or ‘envelop TRP’ and so on should be used without omitting, which can be further discussed in future release

Based on above proposals, our consideration for TRP of multi-antenna UE can be summarized as below table

| Multi-TX scenarios | Legacy TRP |  | Envelop combined TRP |  |
| --- | --- | --- | --- | --- |
| TAS | TRP = max (TRP1, TRP2) | TAS off | Can be defined in future | TAS on |
| TxD | To be defined in R18 | Test mode | Can be defined in future | TxD on |
| UL MIMO | To be defined in R18 | TPMI index 2 | Can be defined in future | Dynamic TPMI |
| FR2 | TRP with fixed beam | Beam locked | Out of scope | Beam unlocked |

Observation 3:	both theory and measurement demonstrate that it is not necessary to test all TPMIs.

Proposal 4:	adopt fixed TPMI index = 2 for single layer UL MIMO TRP test in Rel-18

Observation 4:	TRP of TxD on mode is not legacy TRP but dynamic TRP due to potential phase variation versus direction.

Proposal 5:	apply test mode to disable phase variation versus direction for simultaneous 2Tx TRP test of TxD UE in Rel-18

4. 	References

- R4-2305904    “WF on FR1 TRP TRS”, vivo

- R4-2305701    “Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP”, MediaTek Inc. vivo, CAICT