3GPP TSG-RAN WG4 Meeting #107	R4-2310156

Incheon, KR, 22nd ‒ 26th May, 2023

Title: 	WF on NR BWP operation without restriction

Agenda Item:	8.11.3

Source: 	vivo

Document for:	Approval

- Topic #1: General aspects

#### Applicable conditions of existing RLM/BFD/BM requirements for Option B-1-1, B-1-2 and option C

<Agreement >:

- Conclusion: Wait for RAN1 conclusion before discussing the applicable condition for the L1 requirements for option B-1-1, B-1-2 and option C

## Issue 1-1: How to handle L3 related requirements impact

<Agreement >:

- Send LS to RAN plenary on RAN4 agreements on L3 related requirements impact for the options for supporting BWP operation without restriction.

#### Issue 1-2: Requirements for handover for Option A, C, B-1-1 and B-1-2

  - Option 1:

    - It is beneficial to support handover to a BWP without CD-SSB directly for UE supporting option A, B-1-1 and option B-1-2.

    - It is beneficial to support handover to a BWP with NCD-SSB directly for UE supporting option C.

  - Option 2:

    - Subject to RANP approval, RAN4 to support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap.

  - Option 3:

    - Other options are not precluded.

- Note: The issue can only be discussed unless there is guidance from RANP that it is in the scope of the WI.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option B-1-1 and C

  - Option 1:

    - RAN4 shall discuss the following scenario: If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS).

  - Option 2:

    - If UE supports both option B-1-1 and C, the UE works with option B-1-1 or option C mutually exclusively, and the UE meets the requirements based on option B-1-1 or option C for BM/RLM/BFD, respectively.

    - If the UE is configured with NCD-SSB within the active DL BWP, the UE works and meets the requirements based on Option C i.e., NCD-SSB for BM/RLM/BFD; Otherwise, the UE works and meets the requirements based on Option B-1-1.

  - Option 3:

    - For UE supporting both option B-1-1 and C, clarification on UE behaviour can be considered, for example: SSB within the active BWP should be used for L1 measurements, either CD-SSB or NCD-SSB

  - Option 4:

    - RAN4 not to define additional requirements for UE supporting multiple options.

  - Option 5:

    - Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

  - Option 6:

    - Other options are not precluded.

#### Issue 1-4: Requirements/UE behaviour for UE supporting both option B-1-1 and A

  - Option 1: (MTK)

    - RAN4 shall discuss the following scenario: If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS).

  - Option 2: (vivo)

    - If UE working with option A and option B-1-1 simultaneously is supported, UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

  - Option 3: (CMCC, Huawei, CATT, E///)

    - For UE supporting both option B-1-1 and A, it is not necessary to specify new requirements.

  - Option 4: (Nokia)

    - Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

  - Option 5:

    - Other options are not precluded.

#### Issue 1-5: Requirements/UE behaviour for UE supporting both option C and A

  - Option 1:

    - UE working with option A and option C simultaneously should be supported and UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

  - Option 2:

    - RAN4 not to define additional requirements for UE supporting multiple options.

  - Option 3:

    - Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

  - Option 4:

    - Other options are not precluded.

#### Issue 1-6: Requirements/UE behaviour for UE supporting both option B-1-1 and B-1-2

  - Option 1:

    - RAN4 shall discuss the following scenario: If-else the UE is operating in an active BWP with no NCD-SSB, no CD-SSB and NO CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option B-1-2.

  - Option 2:

    - RAN4 not to define additional requirements for UE supporting multiple options.

  - Option 3:

    - Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

  - Option 4:

    - Other options are not precluded.

- Topic #2: Impact of Option A

#### Issue 2-1: Any clarification on FR2 CSI-RS based RLM/BFD/BM requirements for Option A

<Agreement >:

- Do not introduce further enhancements for CSI-RS based RLM/BFD/BM requirements for Option A

#### Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP

  - Option 1: (Ericsson, Apple, Nokia, Huawei, CMCC, CATT, MediaTek)

    - No clarifications on existing timing requirements are needed.

  - Option 2: (vivo)

    - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

    - FFS from which release the clarification is made.

  - Option 3: (Ericsson)

    - The condition to configure gaps to meet the existing UE transmission timing error requirements in clause 7.1 of TS 38.133, when the UE is performing BM/RLM/BFD based on option A, is NOT needed.

    - A possible compromise is to clarify in clause 7.1.2 of TS 38.133, that the availability of the SSB at the UE is for the purpose of acquiring the timing of the reference cell

- Topic #3: Impact of Option B-1-1

#### Issue 3-3: Clarification on existing timing requirements for supporting Option B-1-1

- Follow conclusion of issue 2-2.

#### Issue 3-4: Clarification/modification on intra-frequency measurement requirements for supporting option B-1-1

- FFS based on RAN progress.

#### Issue 3-7: Whether to define additional requirements for supporting option B-1-1

<Agreement >:

- Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported.

- Do not introduce a new section for RRM requirements for option B-1-1.

- Topic #4: Impact of Option C

#### Issue 4-3: Clarification on existing timing requirements for supporting Option C

<Agreement >:

- The current SSB-based timing requirement can be also applied to non-RedCap UE with NCD-CCB support.

- FFS whether/what additional clarification are needed.

- Topic #5: Impact of Option B-1-2

#### Issue 5-2: Interruption requirements for supporting option B-1-2

<Agreement >:

- Define the following interruption requirements for Option B-1-2

  - Interruption length

    - Option 1: [0.5ms] for FR1 and [0.25ms] for FR2

  - FFS: Interruption ratio

  - FFS: Interruption location

#### Issue 5-4: Applicable of conclusions for Option B-1-1 to Option B-1-2

<Agreement >:

- When CD-SSB is outside the active BWP, the requirements for SSB-based RLM/BFD/BM for UE supporting option B-1-2 with interruptions are FFS.

#### Issue 5-6: Clarification on existing timing requirements for supporting Option B-1-2

<Agreement >:

- Follow conclusion of Issue 3-3 for option B-1-1.

#### Issue 5-7: Clarification/modification on intra-frequency measurement requirements for supporting option B-1-2

- FFS based on RAN progress.

----EoD---