3GPP TSG-RAN WG4 Meeting # 108                                                                          R4-2311058

Incheon, 21 - 25 August, 2023

Title: 	on test time reduction for TxD and UL MIMO

Source: 	Huawei, HiSilicon

Agenda item:	8.15.2.4

Document for:	discussion

# Introduction

Text proposal in R4-2308824 [1] has been approved for test time reduction. As TxD and UL MIMO are also within the scope of this work item, it would be prudent to repeat the same simulations to see if the current gird arrangement and estimated MU are also applicable to TxD and UL MIMO.

# Discussion and Proposals

The background for this discussion is that both TxD and UL MIMO with TPMI index 2 to 5 use two antennas to transmit and effectively form a new radiation pattern. For TxD and UL MIMO under non-coherence, the relative phase between the two antennas may vary over time and the impact of such phase variation is currently under investigation. Nevertheless, the two antennas still form a new radiation pattern during the transmission occasion. Such a radiation pattern could be formed constructively or destructively depending on the spatial direction, resulting in a “narrower beam” or faster variation compared with a single antenna radiation pattern. In other words, coarse grid, which is sufficient for a single antenna, may introduce additional error or uncertainty when applied to two antenna transmission such as TxD or UL MIMO with TPMI index 2 to 5.

Proposal 1: repeat the same simulations as in R4-2308824 for two antenna transmission

The radiation pattern from two antennas can be simulated following the same process used in [2] and [3]

# Conclusion

This contribution makes the following proposal.

Proposal 1: repeat the same simulation as in R4-2308824 for two antenna transmission

# References

[1] R4-2308824 TP on Measurement Uncertainty Associated with Grid Size

[2] R4-2218848 Antenna patterns for TRP and TRS measurement grid analysis

[3] R4-2215539 Test Time Reduction using Coarser TRP/TRS Measurement Grids