3GPP TSG-RAN4 Meeting #108	R4-2311067

# Toulouse , FR, Aug.21sd - 25th, 2023

Source:			China Telecom

Title:			Discussion on the location of SIB1 CBW and UE-specific CBW

Agenda item:	8.5.2.1

Document for:	Discussion

Introduction

In this contribution, we further discuss the location of SIB1 CBW and UE-specific CBW based on the WF on channel raster enhancements last meeting [1].

Discussion

UE-specific channel bandwidth

In CR RP-182896, UE-specific channel bandwidth is introduced to allow UE to set the channel bandwidth in order to be able to operate in a regulatory compliant way when UE is configured with a BWP smaller than the cell-specific channel bandwidth, which is advertised in the SIB1. In CR R2-1902778, It is clarified that UE-specific channel bandwidth must be the values that are defined in TS38.101-1 and TS 38.101-2. Furthermore, it is pointed that location and bandwidth of a BWP is always determined by offsetToCarrier contained in common signaling and not the one configured with dedicated signaling. There is no restriction on the location of UE-specific channel bandwidth except the bandwidth.

Observation 1: No restriction on the location of UE-specific channel bandwidth.

Considering that some vendors claim that there might be NBC issue if network puts UE-specific channel bandwidth not on the channel raster, a UE capability can be introduced to inform the network whether or not to support flexible location for UE-specific channel.

Proposal 1：UE capability can be introduced to support flexible location of UE-specific channel bandwidth.

Proposal 2：UE capability can be per UE.

Cell channel bandwidth (SIB1 CBW)

For the cell specific channel bandwidth indicated in SIB1, it should be supported that channel raster based on the global raster can be used by network if no coexistence issue is concerned. Based on the discussion in previous meetings, the granularity of 10MHz can be considered.

Proposal 3: The granularity of 10MHz can be considered for SIB1 channel bandwidth.

Summary

Observation 1: No restriction on the location of UE-specific channel bandwidth.

Proposal 1：UE capability can be introduced to support flexible location of UE-specific channel bandwidth.

Proposal 2：UE capability can be per UE.

Proposal 3: The granularity of 10MHz can be considered for SIB1 channel bandwidth.

Reference

[1] R4-2310269 WF on channel raster enhancements, RAN4#107.