3GPP TSG-WG4 Meeting #108 	R4-2311078

# Toulouse, France, Aug 21 - Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101 - inclusion of 8Rx Applicability Rule |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of PDSCH, PDCCH and PBCH applicability rules for 8Rx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated tables in Clause 5.1.1.2, introduction of 8Rx Applicability rules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 8Rx applicability rules will not be included in TS 38.101-4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

#### 5.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2, 4 or 8 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exceptions |  |
| --- | --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |  |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |  |
|  | PBCH | All tests in Clause 5.4.2 |  |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |  |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2) | If UE passes test in 5.2.4 UE can skip Clause 5.2.3 Table 5.2.3.2.1-4 (Test 2-1 and Test 2-2) and Table 5.2.3.2.1-6 (Test 4-1) |  |
| or | PDCCH | All tests in Clause 5.3.3.(Note 2,3) |  |  |
| UE supports only 8Rx and 4Rx | PBCH | All tests in Clause 5.4.3(Note 1) |  |  |
| UE supports only 8Rx and 2Rx | PDSCH | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2) | If UE passes test in 5.2.4 UE can skip Clause 5.2.2 Table 5.2.2.2.1-4 (Test 2-1 and Test 2-2) |  |
|  | PDCCH | All tests in Clause 5.3.2.(Note 2,4) |  |  |
|  | PBCH | All tests in Clause 5.4.2 |  |  |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.4.(Note 2) |  |  |
|  | PDCCH | N/A |  |  |
|  | PBCH | N/A |  |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

<End of change 1>