3GPP TSG-RAN WG4 Meeting #108 	R4-2311083

Toulouse, France, Aug 21 – Aug 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting Simulations for 256QAM UL Demodulation

Agenda item:	8.6.4.1

Document for:	Discussion

Introduction

At RAN#98 meeting, the WI was approved to study the feasibility to support UL256 QAM [1] with the following objectives of the performance part.

UL 256QAM - Specify the BS demodulation performance

In this paper, we present Nokia’s initial simulation results for 256QAM UL demodulation.

Discussion

Background

This paper presents the simulation results to support our corresponding discussion paper for RAN4#108 regarding UL256QAM [5]. The simulation results are shown broken down by channel type, focusing on AWGN, TDLA 30-10 and TDCL 300-100, to represent a variety of operating environments where 256 QAM UL may be employed.

Phase noise models used in these simulations are either from TS 38.808 [3] (Set 1) or from R1-162885 [4].

AWGN

The following section presents results for AWGN channel, representing a benign Line of Sight (LOS) channel.

The following tables show results for the case with 1 DMRS and PTRS for MCS 20-27, demonstrating the performance with various Phase Noise models. Results are shown for 70% and 95% TPUT, with a CBW of 50 MHz.

Table 1 : AWGN 256QAM UL, 50 MHz, 1 DMRS + PTRS

| PN Model | No PN |  | 38.808 Set1 |  | R1162885 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR [dB] @ TP | 70% | 95% | 70% | 95% | 70% | 95% |
| MCS 20 | 16.6 | 16.9 | 17.3 | 17.7 | 17.4 | 18.1 |
| MCS 21 | 17.1 | 17.4 | 17.9 | 18.3 | 18.0 | 18.7 |
| MCS 22 | 18.5 | 18.9 | 19.6 | 20.1 | 19.9 | 21.1 |
| MCS 23 | 19.3 | 19.7 | 20.8 | 21.5 | 21.2 | 23.0 |
| MCS 24 | 20.3 | 20.6 | 22.3 | 22.9 | 22.8 | 26.0 |
| MCS 25 | 22.1 | 22.4 | 25.5 | 26.7 | 27.5 | NaN |
| MCS 26 | 22.7 | 23.1 | 27.8 | 29.4 | 31.2 | NaN |
| MCS 27 | 25.5 | 26.2 | NaN | NaN | NaN | NaN |

TDLA 30-10

The following section presents results for TDLA 30-10 channel, representing a Line of Sight (LOS) channel with some multipath and very limited mobility.

Rank 1

The following tables show results for 1 DMRS and PTRS for MCS 20-27, demonstrating the performance with various Phase Noise models. Results are shown for 70% and 95% TPUT, with a CBW of 50 MHz and a Rank of 1

Table 2 : TDLA 30-10 256QAM UL, 100 MHz, 1 DMRS + PTRS

| PN Model | No PN |  | 38.808 Set1 |  | R1162885 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR [dB] @ TP | 70% | 95% | 70% | 95% | 70% | 95% |
| MCS 20 | 17.8 | 20.6 | 18.6 | 21.6 | 18.9 | 22.0 |
| MCS 21 | 18.7 | 21.5 | 19.8 | 22.7 | 20.2 | 23.5 |
| MCS 22 | 19.6 | 22.5 | 21.0 | 24.0 | 21.6 | 25.4 |
| MCS 23 | 20.8 | 23.8 | 22.7 | 25.8 | 23.8 | 30.3 |
| MCS 24 | 22.5 | 25.6 | 25.3 | 28.8 | 27.3 | NaN |
| MCS 25 | 23.9 | 27.1 | 28.0 | 32.1 | 32.0 | NaN |
| MCS 26 | 26.1 | 30.0 | 34.8 | NaN | NaN | NaN |
| MCS 27 | NaN | NaN | NaN | NaN | NaN | NaN |

Table 3 : TDLA 30-10 256QAM UL, 50 MHz, 1 DMRS + PTRS

| PN Model | No PN |  | 38.808 Set1 |  | R1162885 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR [dB] @ TP | 70% | 95% | 70% | 95% | 70% | 95% |
| MCS 20 | 17.3 | 21.4 | 18.1 | 22.4 | 18.3 | 22.7 |
| MCS 21 | 17.8 | 21.9 | 18.7 | 23.0 | 18.9 | 23.4 |
| MCS 22 | 19.2 | 23.4 | 20.6 | 25.0 | 21.1 | 25.8 |
| MCS 23 | 20.2 | 24.5 | 22.0 | 26.5 | 22.7 | 28.0 |
| MCS 24 | 21.3 | 25.6 | 23.6 | 28.4 | 24.8 | 31.5 |
| MCS 25 | 23.2 | 28.1 | 27.5 | 33.1 | 30.5 | NaN |
| MCS 26 | 24.1 | 29.2 | 29.9 | 36.5 | 35.3 | NaN |
| MCS 27 | 28.0 | 35.4 | NaN | NaN | NaN | NaN |

Rank 2

Table 4 : TDLA 30-10 256QAM UL, 100 MHz, 1 DMRS + PTRS

| PN Model | No PN |  | 38.808 Set1 |  | R1162885 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR [dB] @ TP | 70% | 95% | 70% | 95% | 70% | 95% |
| MCS 20 | 25.9 | 30.2 | 28.0 | 34.3 | 29.7 | NaN |
| MCS 21 | 27.6 | 32.8 | 30.6 | NaN | 32.3 | NaN |
| MCS 22 | 29.1 | 35.5 | 33.6 | NaN | 37.5 | NaN |
| MCS 23 | 31.5 | NaN | NaN | NaN | NaN | NaN |
| MCS 24 | 37.9 | NaN | NaN | NaN | NaN | NaN |
| MCS 25 | NaN | NaN | NaN | NaN | NaN | NaN |
| MCS 26 | NaN | NaN | NaN | NaN | NaN | NaN |
| MCS 27 | NaN | NaN | NaN | NaN | NaN | NaN |

Table 5 : TDLA 30-10 256QAM UL, 50 MHz, 1 DMRS + PTRS

| PN Model | No PN |  | 38.808 Set1 |  | R1162885 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR [dB] @ TP | 70% | 95% | 70% | 95% | 70% | 95% |
| MCS 20 | 25.9 | 31.7 | 27.8 | 37.3 | 28.5 | 43.7 |
| MCS 21 | 26.5 | 32.7 | 28.8 | 41.5 | 29.5 | NaN |
| MCS 22 | 28.8 | 37.3 | 32.7 | NaN | 34.8 | NaN |
| MCS 23 | 31.2 | NaN | 40.1 | NaN | NaN | NaN |
| MCS 24 | 34.1 | NaN | NaN | NaN | NaN | NaN |
| MCS 25 | NaN | NaN | NaN | NaN | NaN | NaN |
| MCS 26 | NaN | NaN | NaN | NaN | NaN | NaN |
| MCS 27 | NaN | NaN | NaN | NaN | NaN | NaN |

TDLC 300-100

Simulations were conducted for TDLC 300-100 with Rank1, Rank2 with both 2 DMRS and 1 DMRS+PTRS, for both 100 MHz and 50 MHz channels, for MCS 20-27 and it was found that no MCS provided achievable results.

UL 256QAM was not able to achieve 70% TPUT in TDLC300-100

Conclusion

Within this contribution we discuss the demodulation requirements for UL 256 QAM.

In the paper, the following Observations were made:

Observation 1: UL 256QAM was not able to achieve 70% TPUT in TDLC300-100

- References

- RP-222909 Revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, RAN#98-e, December 2022

- TS 38.104 v18.1.0. Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 18). March 2023

- TS 38.808 v17.0.0. Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17). March 2021

- R1-162885, On the phase noise model for 5G New Radio evaluation, RAN1#84, Busan, Korea, April 2016

- R4-2311082 “Discussion on 256QAM UL Demodulation”, Nokia, Nokia Shanghai Bell, RAN WG4 #108, Toulouse, Aug 21 – Aug 25, 2023