3GPP TSG-RAN WG4 Meeting # 108		R4-2311084

# Toulous,FR, Aug 21st – Aug 25th, 2023

Agenda item:	8.4.1.2

Source: 	Spreadtrum Communications

Title: 	Discussion on indication of delta TRXSRS

Document for:	Discussion

Introduction

In RAN4#107, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows:

- Sub-topic 2-1: DL channel estimation at gNB depends whether or not actual ΔTRXSRS are compensated at UE • Issue 2-1-1: Whether or not actual ΔTRXSRS are compensated at UE <Way forward>: - Further discuss the UE behaviour whether or not UE compensates losses to achieve the same power across different ports during SRS antenna switching. - Option 1: UE compensates losses to achieve the same power across ports (no power imbalance approach) - Option 2: UE does not compensate losses to achieve the same power across ports (fixed power imbalance approach). - Option 3: Other • Issue 2-1-2:Solutions for the issue that DL channel estimation at gNB depends whether or not actual ΔTRXSRS are compensated at UE <Way forward>: - Further discuss in next meeting. - Sub-topic 2-2 Applicability to 2RX/4RX • Issue 2-2-1: Applicability to 2RX/4RX <Way forward>: - If ΔTRxSTS indication from UE to NW is introduced for 8Rx, - Option 1: It can also apply to 2Rx/4Rx case. - Option 2: It does not apply to 2Tx/4Rx case. - Sub-topic 2-3 Effect of loss imbalance across RX paths • Issue 2-3-1: Whether or not to consider effect of loss imbalance across RX paths <Way forward>: - Further discuss the effect of loss imbalance across Rx paths in next meeting.

Discussion

Whether or not actual ΔTRXSRS are compensated at UE

RAN4 sent a LS [2] to RAN1 in RAN1#112-bis-e meeting, asking RAN1 to consider the issues which are mentioned in LS. Until the ending of RAN1#113, there was no conclusion about whether to support reporting SRS IL imbalance or not. Most companies support to report, but some companies have concerns about this issue. It will be continued to discuss in RAN1# 114 meeting.

For the TDD bands, uplink and downlink are reciprocal, gNB can measure SRS sent by UE, evaluate the downlink channel quality based on the uplink, and calculate the weight of the downlink beamforming. Downlinks can be thought of balanced when the signal has good performance because we focus more on SNR and we can align that with AGC. In order to ensure the accuracy of downlink channel estimation, output power of SRS between main branch and diversity branch needs to be kept balanced. Otherwise, it will impact calculate the weight of downlink beamforming which causes DL throughput loss. It is necessary to eliminate the insertion loss between main branch and diversity branch through compensation by UE or gNB.

When UE power is sufficient, diversity branch SRS antenna switch port can reach up to same power with main branch SRS antenna switch port by calibration compensation and aligning the parameters of PA and RFIC. In this case, there is no need to indicate delta TRXSRS to NW. But when UE power is limited, UE could not keep power balanced between main branch SRS antenna switch port and diversity branch by self-compensation, UE needs to have approaches to solve this issue.

Proposal1: UE needs to have approaches to solve this issue when UE power is limited which UE could not keep power balanced between main branch SRS antenna switch port and diversity branch by self-compensation.

Solutions for the issue that DL channel estimation at gNB depends whether or not actual ΔTRXSRS are compensated at UE

There are three possible solutions mentioned in LS. The first solution is to enable UE to report on the actual IL imbalance for each diversity branch used for SRS. About reporting is static or dynamic which is not sure. If the UE did not do self-calibration compensation and not adjust PA and RFIC parameters, the imbalance which is the actual IL would be fixed. It is suitable to report it statically. At the same time, some companies mentioned that changes in the mapping relationship between physical and logical antenna ports will affect the accuracy of actual IL. As far as we know, the serial number of physical antenna port is unique and fixed which can be known to network.  GNB can get the information based on the logical port corresponding to the actual physical antenna port. Under different power levels, actual IL is different. Therefore, if the UE considers calibration compensation and parameters adjustment, reporting statically is not reasonable. Or rather it needs to report actual output of power instead of actual IL where power imbalances across antenna ports are derived by comparing the output of power. But in some cases, self-calibration compensation is sufficient and it is not necessary to report the gNB for compensation. Therefore, it is recommended that reporting capability is optional.

As for the second possible solution mentioned in LS which utilizes PCMAX, f, c via PHR type 3, where power imbalances across antenna ports are derived by comparing the maximum configured power of main branch to those of diversity branches, which needs to further discuss because Pcmax,f,c  indicates the maximum output power, and when the power is not at the maximum output power level, Pcmax,f,c could not be accurately indicated.

As for the third possible solution mentioned in LS defines UE measurements of downlink channels which are reported in order to assist the network in determining the difference between the UE insertion losses for two given antenna ports. It belongs to a new way to measure which looks more complicated and needs to be further discussed.

Proposal 2: Considering that UE reports on the actual IL imbalance for each diversity branch used for SRS per band.

Applicability for 2RX/4RX

For TDD low frequency cells, gNB supports the adaptive function of downlink SRS weight and PMI weight, so that the weight value can more accurately reflect the quality of channel and improve the downlink user throughput of edge users. 2RX/4RX may also have a large SRS IL, therefore, it is necessary to report SRS IL imbalance when the actual IL is large (e.g., band n79).

Proposal 3: The reporting mechanism of 8RX is also applicable to 2RX/4RX.

Whether or not to consider effect of loss imbalance across RX paths

For the TDD bands, uplink and downlink are reciprocal, gNB can measure SRS sent by UE, evaluate the downlink channel quality based on the uplink, and calculate the weight of the downlink beamforming. Therefore, by SRS antenna switching to measure uplink channel quality could reflect the quality of downlink channel. But SRS cannot reflect the PCB level conducted insertion loss of RX paths. There is always existing loss imbalance across RX paths. But RX paths can be thought of balanced when the signal is good performance because we focus more on SNR and we can align that with AGC. We could think that RX paths are balanced and we do not need to consider effect of loss imbalance across RX paths.

Proposal4: We do not need to consider the effect of loss imbalance across RX paths.

Conclusions

In this paper, we provide our views on 8RX UE RF requirements, we have made the following proposals:

Proposal1: Proposal1: UE needs to have approaches to solve this issue when UE power is limited which UE could not keep power balanced between main branch SRS antenna switch port and diversity branch by self-compensation.

Proposal 2: Considering that UE reports on the actual IL imbalance for each diversity branch used for SRS per band.

Proposal 3: The reporting mechanism of 8RX is also applicable to 2RX/4RX.

Proposal4: We do not need to consider the effect of loss imbalance across RX paths.

- Reference

- R4-2310279, WF on 8RX UE RF requirement, NTT DOCOMO, INC.TSG-RAN WG4#107

- R4-2303519, LS on the UE SRS IL imbalance issue, Huawei, Hisilicon. TSG-RAN WG4#106