3GPP TSG-RAN WG4 Meeting #108	R4-2311089

Toulouse, France, August 21 – August 25, 2023

Source:	China Telecom

Title:	Discussion on PDSCH requirements for UE with multiple Rx

Agenda Item:	8.4.3.1.2

Document for:	Discussion

- Introduction

In RAN4 #107 meeting, the 8Rx demodulation are discussed and the WF is approved in [1]. In this paper, we give our views on the remaining open issues.

- Discussion

In the last meeting, we have agreed the baseline selected MCS for rank 2 and rank 4 tests for companies to check if there are any testability issues.

MCS for Rank 2 and Rank 4 test

Status in the WF in [1]:

Rank2: Agree [MCS 20] (Table 2) for this meeting, if any issues are figured out for next meeting, MCS 19 (Table 1) will be selected

Rank4: Agree [MCS 26] (Table 1) for this meeting, if any issues are figured out for next meeting, MCS 17 (Table 1) will be selected

From our simulation results, we observe that for Rank 2, we observe SNR range 19~20dB under 70% max TP with MCS 20(Table 2). For Rank 4, SNR range 18~19 under 70% max TP with MCS 26(Table 1). Considering the impairments, the highest final SNR requirement will be around 22dB, thus no testability issue is observed. Therefore, we support MCS 20(Table 2) for Rank 2 and MCS 26(Table 1) for Rank 4.

Proposal 1: Use MCS 20(Table 2) for Rank 2 and MCS 26(Table 1) for Rank 4.

- Conclusion

Proposal 1: Use MCS 20(Table 2) for Rank 2 and MCS 26(Table 1) for Rank 4.

- References

- [1] R4-2309806, WF for 8Rx UE performance requirements, Huawei, HiSilicon, RAN4 #107, May, 2023