3GPP TSG-RAN4 Meeting #108	R4-2311090

Toulouse, France, August 21 – August 25, 2023

Source:	China Telecom

Title:	Discussion on PDSCH requirements for UE with multiple Rx: Simulation results

Agenda Item:	8.4.3.1.2

Document for:	Discussion

- Introduction

This contribution provides the simulation results about 8Rx demodulation performance for PDSCH, and the assumption is based on the approved WF in [1].

- Simulation Results

Table 1.  Ideal and impairment simulation results for 8Rx PDSCH under TDD 30kHz SCS

| TDD 8 Rx Rank 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | SNR (Alignment results) | SNR (Impairment results) |
| TDD 40 MHz/30 kHz | 2T8R ULA Medium B | Rank 2, MCS20 (Table 2) | TDLC300-100 | 70 | 19.96 | 21.96 |
|  |  | Rank 2, MCS19 (Table 1) |  |  | 11.09 | 13.09 |
| TDD 8 Rx Rank 4 |  |  |  |  |  |  |
| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | SNR (Alignment results) | SNR (Impairment results) |
| TDD 40 MHz/30 kHz | 4T8R ULA Low | Rank 4, MCS17 (Table 1) | TDLA30-10 | 70 | 9.56 | 11.56 |
|  |  | Rank 4, MCS26 (Table 1) |  |  | 19.19 | 21.19 |

Table 2.  Ideal and impairment simulation results for 8Rx PDSCH under FDD 15kHz SCS

| FDD 8 Rx Rank 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | SNR (Alignment results) | SNR (Impairment results) |
| FDD 10 MHz/15 kHz | 2T8R ULA Medium B | Rank 2, MCS20 (Table 2) | TDLC300-100 | 70 | 19.27 | 21.27 |
|  |  | Rank 2, MCS19 (Table 1) |  |  | 10.1 | 12.1 |
| TDD 8 Rx Rank 4 |  |  |  |  |  |  |
| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | SNR (Alignment results) | SNR (Impairment results) |
| FDD 10 MHz/15 kHz | 4T8R ULA Low | Rank 4, MCS17 (Table 1) | TDLA30-10 | 70 | 9.27 | 11.27 |
|  |  | Rank 4, MCS26 (Table 1) |  |  | 18.21 | 20.21 |

- Reference

[1] R4-2309806, WF for 8Rx UE performance requirements, Huawei, HiSilicon, RAN4 #107, May, 2023