3GPP TSG-RAN WG4 Meeting # 108	R4-2311118

# Toulouse, France, August 21 – August 25, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	CA_n71-n85 cross band MSD for 35MHz n71 UL CBW

Agenda Item:	7.1.1.2	Others	[WI code]

Document for:	Approval

# Introduction

In RAN4#107, agreements on how to handle the new 35MHz CBW for n71 were captured in WF [1], which was based on the input in contribution [2]. In this contribution, and within the context of the way forward from previous meeting, we provide our proposals for n71 35MHz UL-related cross-band MSD for CA_n710-n85 based on measured data.

# Discussion

In RAN4#107, WF [1] has captured a set of agreements on CA_n71-n85 MSD and handling of the 35MHz CBW in n71:

<WF1 applicable band n71 cross band MSD due to n85 UL>

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

<WF2 applicable band n85 cross band MSD due to n71 UL applicable to UEs not supporting optional maximum symmetrical CBW >

- Applicable to legacy UEs or UEs supporting optional symmetrical UL/DL CBW for band n71 but not the maximum one

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

<WF3 band n85 cross band MSD due to n71 UL applicable to UEs supporting optional maximum symmetrical CBW to be evaluated for RAN4#108>

- Applicable to UEs supporting the maximum optional symmetrical UL/DL CBW in band n71

- If max CBW is 35MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | TBDX | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

- If max CBW is 30MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 683 | 30 | 15 | 20 (Rbstart=140) | 730.5 | 5 | TBDX | ACLR2 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

Since this way forward, the introduction of the 35MHz UL CBW for band n71 was confirmed in RAN#100. Thus, in this contribution we provide our MSD evaluation for the first test point in WF3 based on measured data.

Separately, although the MSD was evaluated properly and thus the MSD value is valid, the UL RB allocation for the n85 UL cross band MSD into band n71 is not correct as band n85 is above band n71. The RBstart value should be 0 and not 59 (UL allocation closest to the victim DL band).

Proposal for correction of CA_n71-n85 cross band MSD in n71 due to n85 UL:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n85 | n71 | 705.5 | 15 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.8 | >ACLR2 |

## Evaluation of band n85 cross band MSD related to n71 35MHz UL

To precisely evaluate this MSD, we measured the PA output noise in the lower 5MHz of band n85 with a DFT-s-OFDM 20RB168 waveform including transceiver impairments, at 27dBm after calibrating for 30dB ACLR for 100RB0 DFT-s-OFDM at 26dBm (4dB post PA losses assumed). The post-MRC MSD calculation is provided in Table 1. The interference being related to the IMD3 of the wanted 20RBs and their image, as expected the MSD is close to 30dB. but slightly lower thanks the peak of the IMD3 falling just below band n85.

Table 1: post MRC calculations for cross band MSD in n85 DL due to 35MHz n71 UL

| CBW/NRB | [MHz]/# | 5 | 25 |
| --- | --- | --- | --- |
| Meas Tx noise / transmitter noise floor | [dBm/CBW] | -25.6 | -63.5 |
| 5MHz CBW scaled REFSENS | [dBm/CBW] | -97.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -93.0 | -93.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 42 | 38 |
| LNA to antenna insertion loss | [dB] | 4 |  |
| Antenna isolation | [dB] | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -63.6 | -69.6 |
| noise degradation due to Tx noise: Main/Div | [dB] | 29.5 | 23.5 |
| REFSENS degradation after MRC | cor [dB] | 28.1 |  |

Proposal for CA_n71-n85 cross band MSD in n85 due to n71 UL for legacy UEs and UEs supporting the newly introduced 35MHz symmetrical DL/UL configuration:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 28.1Y | ACLR1 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth of 35MHz NOTE Y: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth of 35MHz |  |  |  |  |  |  |  |  |  |

The two test points are necessary to cover all types of UEs in this Release of 38.101-1, but in Release 19, the first test point can be discarded as all UES will have to support all UL CBW.

# Conclusions

In this contribution, we further develop our proposals in [4] in the light of the agreements achieved in WF [1] and address other proposed options.

Proposal for correction of CA_n71-n85 cross band MSD in n71 due to n85 UL:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n85 | n71 | 705.5 | 15 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.8 | >ACLR2 |

Proposal for CA_n71-n85 cross band MSD in n85 due to n71 UL for legacy UEs and UEs supporting the newly introduced 35MHz symmetrical DL/UL configuration:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 28.1Y | ACLR1 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth of 35MHz NOTE Y: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth of 35MHz |  |  |  |  |  |  |  |  |  |

The two test points are necessary to cover all types of UEs in this Release of 38.101-1, but in Release 19, the first test point can be discarded as all UES will have to support all UL CBW.

# References

- R4-2310318 WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support T-Mobile USA, Skyworks Solutions, Inc., RAN4#107

- R4-2309363 New test point for CA_n71-n85 cross band MSD Skyworks Solutions Inc., RAN4#107