# 3GPP TSG-RAN WG4 Meeting #108                                                        	R4-2311120

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5983 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.101 Rel-18 Removal of Cat-M2 REFSENS in B71 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2023-08-02 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | REFSENS is specified for channel bandwidths of 1.4MHz and 3MHz that are not supported by band 71. These cell configurations are not valid and can not be verified. See R4-2309740. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For band 71 1.4MHz and 3MHz CBW: - Table 7.3.1E-8 and Table 7.3.1E-9: remove REFSENS requirements. - Table 7.3.1E-10: remove uplink configurations for REFSENS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | REFSENS requiements can not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## << Start of change >>

### 7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis

<<< Unchanged sections are skipped >>>

Table 7.3.1E-8: Reference sensitivity for FDD /TDD UE category M2 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 2 | -100.2 | -97.2 | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 3 | -99.2 | -96.2 | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 4 | -102.2 | -99.2 | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 5 | -100.7 | -97.7 | -95.7 | -95.7 |  |  | FDD |
| 7 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 8 | -99.7 | -96.7 | -94.7 | -94.7 |  |  | FDD |
| 11 |  |  | -97.7 | -97.7 |  |  |  |
| 12 | -99.2 | -96.2 | -94.7 | -94.7 |  |  | FDD |
| 13 |  |  | -94.2 | -94.2 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 18 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 19 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 20 |  |  | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 21 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 24 |  |  | -96.7 | -96.7 |  |  | FDD |
| 25 | -98.7 | -95.7 | -94.2 | -94.2 | -94.2 | -94.2 | FDD |
| 26 | -100.2 | -97.2 | -95.2 | -95.2 | -95.2 |  | FDD |
| 27 | -100.7 | -97.7 | -95.7 | -95.7 |  |  | FDD |
| 28 |  | -97.7 | -96.2 | -96.2 | -96.2 | -96.2 | FDD |
| 31 | -96.5 | -93.2 | -91.2 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 40 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 41 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | TDD |
| 42 |  |  | -96.7 | -96.7 | -96.7 | -96.7 | TDD |
| 43 |  |  | -96.7 | -96.7 | -96.7 | -96.7 | TDD |
| 48 |  |  | -96.5 | -96.5 | -96.5 | -96.5 | TDD |
| 54 | -103.7 | -99.7 | -97.7 |  |  |  | TDD |
| 66 | -101.7 | -98.7 | -97.2 | -97.2 | -97.2 | -97.2 | FDD |
| … |  |  |  |  |  |  |  |
| 71 |  |  | [-93.4] | [-93.4] | [-93.4] | [-93.4] | FDD |
| 72 | [-96.5] | [-92.5] | [-90.5] |  |  |  | FDD |
| 73 | [-96.5] | [-92.5] | [-90.5] |  |  |  | FDD |
| 85 |  |  | -94.7 | -94.7 |  |  | FDD |
| 87 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| 88 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that supports both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 5: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |

Table 7.3.1E-9: Reference sensitivity for HD-FDD category M2 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -97 | -97 | -97 | -97 | HD-FDD |
| 2 | -101 | -97 | -95 | -95 | -95 | -95 | HD-FDD |
| 3 | -100 | -96 | -94 | -94 | -94 | -94 | HD-FDD |
| 4 | -103 | -99 | -97 | -97 | -97 | -97 | HD-FDD |
| 5 | -101.5 | -97.5 | -95.5 | -95.5 |  |  | HD-FDD |
| 7 |  |  | -95 | -95 | -95 | -95 | HD-FDD |
| 8 | -100.5 | -96.5 | -94.5 | -94.5 |  |  | HD-FDD |
| 11 |  |  | -97 | -97 |  |  | HD-FDD |
| 12 | -100 | -96 | -94 | -94 |  |  | HD-FDD |
| 13 |  |  | -94 | -94 |  |  | HD-FDD |
| 14 |  |  | -94 | -94 |  |  | HD-FDD |
| 18 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 19 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 20 |  |  | -94.5 | -94.5 | -94.5 | -94.5 | HD-FDD |
| 21 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 24 |  |  | -97.5 | -97.5 |  |  | HD-FDD |
| 25 | -99.5 | -95.5 | -93.5 | -93.5 | -93.5 | -93.5 | HD-FDD |
| 26 | -101 | -97 | -95 | -95 | -95 |  | HD-FDD |
| 27 | -101.5 | -97.5 | -95.5 | -95.5 |  |  | HD-FDD |
| 28 |  | -97.5 | -95.5 | -95.5 | -95.5 | -95.5 | HD-FDD |
| 31 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 71 |  |  | -94.2 | -94.2 | -94.2 | -94.2 | HD-FDD |
| 72 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 73 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 85 |  |  | -94 | -94 |  |  | HD-FDD |
| 87 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 88 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

Table 7.3.1E-10: FDD/HD-FDD and TDD UE category M2 Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 2 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 3 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 4 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 5 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 7 |  |  | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 8 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 11 |  |  | 24 | 24 |  |  | FDD/HD-FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD/HD-FDD |
| 13 |  |  | 201 | 201 |  |  | FDD/HD-FDD |
| 14 |  |  | 151 | 151 |  |  | FDD/HD-FDD |
| 18 |  |  | 24 | 24 | 24 |  | FDD/HD-FDD |
| 19 |  |  | 24 | 24 | 24 |  | FDD/HD-FDD |
| 20 |  |  | 24 | 201 | 203 | 203 | FDD/HD-FDD |
| 21 |  |  | 24 | 241 | 241 |  | FDD/HD-FDD |
| 24 |  |  | 24 | 241 |  |  | FDD/HD-FDD |
| 25 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 26 | 6 | 15 | 24 | 24 | 24 |  | FDD/HD-FDD |
| 27 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 28 |  | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 31 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  |  | 24 | 24 | 24 | TDD |
| 40 |  |  |  | 24 | 24 | 24 | TDD |
| 41 |  |  |  | 24 | 24 | 24 | TDD |
| 42 |  |  |  | 24 | 24 | 24 | TDD |
| 43 |  |  |  | 24 | 24 | 24 | TDD |
| 48 |  |  |  | 24 | 24 | 24 | TDD |
| … |  |  |  |  |  |  |  |
| 54 | 6 | 15 | 24 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 71 |  |  | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 72 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 73 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 85 |  |  | 201 | 201 |  |  | FDD/HD-FDD |
| 87 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 88 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: For the UE which supports both Band 11 and Band 21 the uplink configuration for reference sensitivity is FFS. NOTE 3: 3 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 NOTE 4: 4 refers to Bands 31, 72 and 73; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

## << End of change >>