3GPP TSG-RAN WG4 Meeting #108	R4-2311142

Toulouse, France, 21 – 25 August 2023

Source:	Skyworks Solutions, Inc.

Title:	BCS4/5 Cross-band MSD update for n79 10MHz CBW

Agenda item:			7.26.2.2 Necessary MSD requirements for BCS4/BCS5 for CA band combinations

Document for:		Approval

# 1. Introduction

At RAN4 meeting #107, the agreed Way Forward [1] provided guidelines on how to handle the maintenance of MSD test points in case a new smallest or a new largest CBW needs to be supported due to requesting BCS4/5 in certain band combinations. This contribution is a companion document to Change Requests (CRs) [2,3] which updates the band n79 10MHz downlink (DL) channel bandwidth (CBW) MSD needed to account for recently agreed BCS4/5 for CA_n41A-n79A and CA_n78A-n79A.

# 2. Discussion

Table 1 and Table 2 summarize the BCS4/5 requirements for CA_n41-n79 and CA_n78-n79.

Table 1 Rel-18 BCS4/5 for CA_n41-n79 extracted from Table 5.5A.3.1-1j

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | n418 n798 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n79C | CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41C-n79A | CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n79C | CA_n41C CA_n79C CA_n41A-n79A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

Table 2 Rel-18 BCS4/5 for CA_n78-n79 extracted from Table 5.5A.3.1-1n

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A-n79A | n78A8,9 n79A8,9 CA_n78A-n79A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n79C | - | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | CA_n78(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

Table 3 and Table 4 summarize the PC2 and PC1.5 band n79 MSD and downlink configurations. The band n79 MSD is specified for 40MHz CBW, which was technically correct when BCS4/5 was not requested. However, according to WF [1], when BCS4/5 was requested and agreed, these test points should have been updated to reflect that BCS4/5 introduces a new band n79 DL CBW of 10MHz.

Table 3 Rel-18 band n79 PC2 downlink configuration and MSD due to cross band isolation – from Table 7.3A.6-1a

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 3.1 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |

Table 4 Rel-18 band n79 PC1.5 downlink configuration and MSD due to cross band isolation – from Table 7.3A.6-1b

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |

Observation: For CA_n41-n79, and CA_n78-n79, the PC2, PC1.5 MSD test points due to cross-band isolation are specified for n79 40MHz DL CBW. According to WF [1], the request for BCS4/5 introduces a new band n79 lowest CBW of 10MHz. These MSD test points shall be updated.

For CA_n41-n79, the source of interference for the lowest band n79 10MHz DL carrier is ~ ACLR17, i.e., it can be assumed that the band n41 UL transmitter noise power spectral density (PSD) is flat across the n79 DL carrier.

For CA_n78-n79, it can also be assumed that the band n78 transmitter noise PSD is flat since the lowest band n79 10MHz DL carrier is affected by ~ ACLR6. Thus, for the first MSD test point, there is no need to revisit the band n79 MSD level. Only the band n79 downlink configuration needs to be updated. The 2nd optional MSD test point does not need to be updated.

Proposal: For Rel-18, endorse the Rel-18 and Rel-17 CRs [2,3] which capture the corrections to the band n79 MSD test point highlighted in green in Table 5 and Table 6.

Table 5 Rel-18 band n79 PC2 MSD test point corrections for BCS4/5

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 3.1 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |

Table 6 Rel-18 band n79 PC1.5 MSD test point corrections for BCS4/5

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |

# 3. Conclusion

In this contribution, we observe the band n79 MSD test point due to cross-band isolation need to be updated to reflect the introduction of the new smallest 10MHz CBW for band n79 due to BCS4/5 requests for CA_n41-n79 and CA_n78-n79.

Observation: For CA_n41-n79, and CA_n78-n79, the PC2, PC1.5 MSD test points due to cross-band isolation are specified for n79 40MHz DL CBW. According to WF [1], the request for BCS4/5 introduces a new band n79 lowest CBW of 10MHz. These MSD test points shall be updated.

Proposal: For Rel-18, endorse the Rel-18 and Rel-17 CRs [2,3] which capture the corrections to the band n79 MSD test point highlighted in green in Table 5 and Table 6.

Table 5 Rel-18 band n79 PC2 MSD test point corrections for BCS4/5

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 3.1 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |

Table 6 Rel-18 band n79 PC1.5 MSD test point corrections for BCS4/5

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 8 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |

# References

[1]	R4-2310412 WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW, 3GPP TSG RAN4 Meeting #107, Incheon, Republic of Korea, Skyworks Solutions, Inc.

[2]	R4-2311131 CR to TS 38.101-1 Rel-18 Corrections to cross-band MSD and CA_n7-n38 fallback, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[3]	R4-2311132 CR to TS 38.101-1 Rel-17 Corrections to cross-band MSD, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.