3GPP TSG-RAN WG4 Meeting #108	R4-2311144

Toulouse, France, 21 – 25 August 2023

Source:	Skyworks Solutions, Inc.

Title:	Corrections to cross-band MSD for CA_n1-n41, CA_n41-n66 and EN-DC counterparts

Agenda item:			7.1.1.2 Others

Document for:	Approval

# 1. Introduction

This contribution is a companion document to Change Requests [5,6,7,8] in which we propose to cross-align the RF front-end (RF-FE) assumptions used to evaluate the MSD band B1/n1, B4, B66/n66 due to band 41/n41 transmitter cross-band isolation interference. This ensures the MSD levels are cross-aligned between NR-CA and their EN-DC counterpart. It also ensures consistent MSD requirements for bands with near identical downlink frequency range.

# 2. Discussion

The migration to the new MSD table template requires revisiting many NR-CA and EN-DC MSD levels due to change of uplink (UL) and downlink (DL) RF configuration parameters: channel bandwidth (CBW), carrier frequencies (Fc) and UL RB allocation (Lcrb/RBSTART). In preparation to the migration of all Rel-18 EN-DC test points to the new table template adopted for NR-CA in Rel-17, it is necessary to ensure the NR-CA MSD test points are cross-aligned with their EN-DC counterparts.

In this paper, we have identified different assumptions of RF-FE cross-band isolation from n41 Tx to DL n1/B4 for the MSD analysis of CA_n1-n41 and DC_4_n41.

- For band n1, the PC3 MSD was analyzed assuming a total cross-band isolation from n41 Tx to n1 Rx of 30dB: 20dB from the band n41 Tx SAW filter plus 10dB from the diplexer based on RF-FE assumptions [2]. The band n1 5MHz CBW MSD was evaluated at ~18.1 dB as shown in Table 1 below [1].

Table 1: n1 updated MSD level for CA_n1-n41 [1]

- For band 4, the PC3 MSD was analyzed assuming a total cross-band isolation from n41 Tx to n1 Rx of 40dB: 30dB from the band n41 Tx SAW filter plus 10dB from the diplexer based on RF-FE assumptions [3]. The band B4 5MHz CBW MSD was evaluated at ~9.4dB, i.e., nearly 9dB lower than that of band n1 as shown in Table 2 below [4].

Table 2: B41 MSD level for DC_4_n41 [4]

Note that in both analyses, the band 41/n41 interference level is identical to ~-45dBm, so the difference in MSD between band n1 and band B4 is solely due to the difference in RF-FE isolation assumptions between 30dB from [2] and 40dB from [3]. The 10dB difference in isolation results in near 10dB difference in MSD, despite the near similar affected downlink frequency ranges of band n1 and B4.

Observation: For CA_n1-n41, the band n1 PC3 MSD due to band n41 cross-band isolation interference is nearly 10dB higher than the band B4 PC3 MSD for DC_4_n41. This is due to a 10dB difference in RF-FE cross-band isolation assumptions between [2] and [3]. Due to overlapping DL frequency ranges of band 1/n1, B4, and n66/B66, it is necessary to cross-align assumptions and MSD levels.

The different RF-FE assumptions have been the main challenge in migrating test points that were specified at different stages of the TS development from different companies. However, under the new MSD table template, it is necessary to ensure near similar set of assumptions are used to evaluate the MSD requirements of similar downlink frequency ranges.

We therefore make the following correction proposal:

Proposal: For MSD due to band 41/n41 transmitter noise, the MSD test points due to cross-band isolation are cross-aligned between NR-CA and EN-DC by assuming a total of 40dB cross-band isolation: 30dB from Tx Saw filter + 10dB diplexer isolation [3]. It is proposed to endorse the following corrections [5,6,7,8].

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Power class/ CR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 9.4 | PC3 - [5,6] |
| n411 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 9.4 | PC3 - [5,6] |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 12.4 | PC2 - [5,6] |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 15. | PC1.5 - [5,6] |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2167.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 4 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2152.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 661 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2197.5 | 5 | 9.4 | PC3 - [7] |
| 41 | n1 | 2506 | 20 | 15 | 100 (RBSTART=0) | 2167.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 12.4 | PC2 – [8] |
| n41 | 66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 12.4 | PC2 – [8] |

# 3. Conclusion

In this contribution we observe that difference in RF-FE total cross-band isolation [1,2,3,4] from the band n41 Tx frequency range to the downlink band 1/n1, 4, 66/n66 frequency ranges introduces inconsistent MSD levels for near identical affected downlink frequency ranges.

Observation: For CA_n1-n41, the band n1 PC3 MSD due to band n41 cross-band isolation interference is nearly 10dB higher than the band B4 PC3 MSD for DC_4_n41. This is due to a 10dB difference in RF-FE cross-band isolation assumptions between [2] and [3]. Due to overlapping DL frequency ranges of band 1/n1, B4, and n66/B66, it is necessary to cross-align assumptions and MSD levels.

Under the new MSD table template, it is necessary to ensure near similar set of assumptions. We propose to endorse CR [5,6,7,8] which capture the following proposals.

Proposal: For MSD due to band 41/n41 transmitter noise, the MSD test points due to cross-band isolation are cross-aligned between NR-CA and EN-DC by assuming a total of 40dB cross-band isolation: 30dB from Tx Saw filter + 10dB diplexer isolation [3]. It is proposed to endorse the following corrections [5,6,7,8].

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Power class/ CR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 9.4 | PC3 - [5,6] |
| n411 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 9.4 | PC3 - [5,6] |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 12.4 | PC2 - [5,6] |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 15. | PC1.5 - [5,6] |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2167.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 4 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2152.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 661 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2197.5 | 5 | 9.4 | PC3 - [7] |
| 41 | n1 | 2506 | 20 | 15 | 100 (RBSTART=0) | 2167.5 | 5 | 9.4 | PC3 - [7] |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 12.4 | PC2 – [8] |
| n41 | 66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 12.4 | PC2 – [8] |

# References

[1]	R4-2214004 Corrections to NR-CA Cross-band Isolation MSD Test Points, 3GPP TSG RAN4 Meeting #104-e, Online, Skyworks Solutions, Inc.

[2]	R4-1908940 Discussion on MSD and updated TP for TR 37.716-11-11: DC_1_n41, Huawei, HiSilicon, 3GPP TSG-RAN WG4 #92 meeting.

[3]	R4-1907461 TP for TR 37.716-11-11: DC_66A_n41A, 3GPP TSG-RAN WG4 Meeting #91, Ericsson, T-Mobile US, MediaTek Inc.

[4]	R4-2305748 PC3 cross-band isolation MSD for EN-DC simplification, 3GPP TSG RAN4 Meeting #106bis-e, Online, Skyworks Solutions, Inc.

[5]	R4-230xxxx CR to TS 38.101-1 Rel-18 Corrections to cross-band MSD and CA_n7-n38 fallback, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[6]	R4-230xxxx CR to TS 38.101-1 Rel-17 Corrections to cross-band MSD, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[7]	R4-230xxxx Draft CR to TS 38.101-3 Rel-18 New table format for PC3 MSD due to cross-band isolation, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[8]	R4-230xxxx Draft CR to TS 38.101-3 Rel-18 New table format for PC2 MSD due to cross-band isolation, 3GPP TSG RAN4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.