3GPP TSG-RAN WG4 Meeting #108	R4-2311150

Toulouse, France, 21 – 25 August 2023

Source:	Skyworks Solutions, Inc.

Title:	Rx harmonic mixing for SUL

Agenda item:			8.1.3 Simplification of specification and reduction of test burden

Document for:		Approval

# 1. Introduction

This contribution investigates the need for a new MSD table for SUL due to receiver (Rx) harmonic mixing.

# 2. Discussion

A comparison of agreed Rx harmonic mixing test point for NR-CA indicates that the power-class 3 (PC3) test point for CA_n39A-n41A may be applicable to its counter-part SUL_n41A-n98A, due to near identical BCS 0 requirements as shown in Table 1 below. For CA_n39A-n41A, n41A may be affected by UL4/DL3 Rx harmonic mixing. In rel 18.2.0 there are currently no Rx harmonic mixing requirements for SUL configurations.

Table 1: Comparison of CA_n39A-n41A and SUL_n41A-n98A BCS requirements.

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| SUL configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| SUL_n41A-n98A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |

Observation: CA_n39A-n41A and SUL_n41A_n98A have near identical BCS0 requirements. In CA_n39A-n41A, band n41 may be affected by UL4/DL3 Rx harmonic mixing. Yet, currently there are  no Rx harmonic mixing requirements for its SUL counter-part.

The new MSD table template eases cross-alignment of MSD test point across band combinations of different type: NR-CA with EN-DC, etc. We propose to consider aligning the band n41 Rx harmonic mixing requirement for SUL_n41A-n98A with the requirement for CA_n39A-n41a.

Proposal: Consider the introduction of new Rx harmonic mixing MSD table "Table 7.3C.2-6"  as captured in Table 2 below.

Table 2: "New" Table 7.3C.2-6 Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to receiver harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | [8.1] | NOTE 1 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | [3.3] | NOTE 1 | UL4/DL3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

# 3. Conclusion

In this contribution, we observe that CA_n39A_n41A and SUL_n41A-n98A have near similar BCS0 requirements. It is therefore proposed to cross-align the band n41 UL4/DL3 Rx harmonic mixing MSD test point for SUL_n41A-n98A in new Table 7.3C.2-6.

Observation: CA_n39A-n41A and SUL_n41A_n98A have near identical BCS0 requirements. In CA_n39A-n41A, band n41 may be affected by UL4/DL3 Rx harmonic mixing. Yet, currently there are no Rx harmonic mixing requirements for its SUL counter-part.

Proposal: Consider the introduction of new Rx harmonic mixing MSD table "Table 7.3C.2-6"  as captured in Table 2 below.

Table 2: "New" Table 7.3C.2-6 Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to receiver harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | [8.1] | NOTE 1 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | [3.3] | NOTE 1 | UL4/DL3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

# References