3GPP TSG-RAN WG4 Meeting # 108	R4-2311209

# Toulouse, France, August 21 – August 25, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	CA_n5-n105 requirements compared to CA_n5-n71

Agenda Item:	7.30.1.4	CA configuration of CA_n5-n105	[NR_700800900_combo_enh-Core]

Document for:	Approval

# Introduction

In RAN4#107, a few contributions provided input to the CA_n5-n105 requirements which were captured in TR 38.872 via the CR [1]. This included our input in [2] which was based on existing band n105 duplexer performance. We also provided input to the very similar case for CA_n5-n71 in [4] that was captured in [3]. In this contribution, we provide refined proposals based on comparing the two cases.

# Discussion

In RAN4#107, a CR [1] captured inputs from two companies on cross band MSDs for CA-n5-n105. Our proposals in [2], were based on an architecture with two antennas and a dual triplexer approach. However, this proposal was based on the isolation performance of current duplexers and assuming a -125dBm/Hz noise floor from the transmitters. Other companies provided input and found that either the MSD was negligible or higher than our input.

In a separate discussion in RAN4#107, the very similar CA_n5-n71 case was discussed in [4] and consensus captured in [3]. Since the case is the same, similar values can be anticipated for CA_n5-n105 and CA_n5-n71:

- n71 UL is only 5MHz lower in frequency than n105 but at this large distance there is no difference in transmitter noise and n71 implementation may use n105 filter

- n71 and n105 DL bandwidths start at the same distance from n5 and at this large distance there is no difference in transmitter noise.

In fact, it is very likely that CA_n5-n105 and CA_n5-n71 uses the same RF front-end as n105 is a 5MHz extension of n71. Given that the cross-band MSD is linked to the far distance transmitter noise floor, there should be no difference in MSD. The values adopted in [3] are already the result of compromises between companies.

Proposal on architecture: CA_n5-n71 and CA_n5-n105 requirements should be derived from the same implementation and thus have the same specification for DeltaT/R and cross-band MSD. Thus, CA_n5-n105 should adopt the agreed upon requirements in [3].

Proposal on Delta T and Delta R:

Table 1: ΔTIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n105 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n105 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Proposal on cross band MSD:

Table 3: Reference sensitivity exceptions (MSD) due to cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |

# Conclusions

In this contribution, we compared the DeltaT/R and cross band MSD for CA_n5-n105 with CA_n5-n71 and make the following proposals.

Proposal on architecture: CA_n5-n71 and CA_n5-n105 requirements should be derived from the same implementation and thus have the same specification for DeltaT/R and cross-band MSD. Thus, CA_n5-n105 should adopt the agreed upon requirements in [3].

Proposal on Delta T and Delta R:

Table 1: ΔTIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n105 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n105 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Proposal on cross band MSD:

Table 3: Reference sensitivity exceptions (MSD) due to cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |

# References

- R4-2310256 Draft CR for 38.872 to introduce CA_n5-n105 Xiaomi, Huawei, HiSilicon, Skyworks Solutions, Inc., Qualcomm, RAN4#107

- R4-2309299 Input on CA_n5-n105 cross-band MSD and DeltaT/R Skyworks Solutions, Inc., RAN4#107

- R4-2310317 TP for 38.718-02-01 on MSD value due to cross band isolation for CA_n5A-n71A combination Ericsson, Rogers, RAN4#107

- R4-2309298 Input on CA_n5-n71 cross-band MSD and DeltaT/R Skyworks Solutions, Inc., RAN4#107