3GPP RAN WG4 Meeting #108		R4-2311212

Toulouse, France, August 21st – 25th 2023

Agenda item:	6.3.2

Source:	Apple

Title:	Update of the regulatory requirements

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which has been approved recently in different regulatory regions. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI [2], RAN#96 meeting approved a new WI [3] with the main motivation of enabling LPI and VLP in countries not covered by previous efforts.

During the previous RAN WG4 meetings we have been providing overviews of the 6GHz regulations for the LPI and VLP modes identifying which existing NS values can be reused and which new NS values are still needed. There are several countries – South Africa, Argentina, and Singapore – which have recently completed its regulations for the license-exempt operation in the 6GHz band and thus were not covered by the CRs agreed during the previous RAN4#107 meetring. In this paper we present an overview of these regulations suggesting to re-use existing NS values for new countries.

# 2	Overview of the 6GHz regulatory rules

Here we provide a more detailed overview of the 6GHz band requirements adopted by new countries with a summary of which NS flags can be re-used.

-	South Africa. This country allows both LPI and VLP modes in 5925-6425MHz frequency range, with the maximum Tx power of 23dBm and 14dBm. The corresponding regulatory decision also refers to the ETSI EN 303 687 document as a relevant performance standard.

-	Singapore. This country allows both LPI and VLP modes in 5925-6425MHz frequency range, with the maximum Tx power of 24dBm and 14dBm; and the PSD restrictions are set to 11dBm/MHz and 1dBm/MHz accordingly.

-	Argentina. This country allows only the LPI mode in 5925-7125MHz frequency range with the maximum Tx power of 24dBm and -1dBm/MHz PSD restriction. Out-of-band emission requirements are set to -27dBm/MHz.

Both South Africa and Singapore allow LPI and VLP usage in the lower 6GHz band, i.e. they can be supported with the existing 3GPP band n102. As for the exact regulatory parameters, in case of South Africa existing EU/CEPT NS flags can be re-used for that country. In turn, Singapore can be supported with the same set of flags as for e.g. Australia.

Unlike most of the Latin American countries, Argentina allows only the LPI mode in the whole 6GHz band for the indoor usage. Nevertheless, its regulatory parameters for the LPI mode are aligned with other countries in that region. Thus, the existing band n96 and NS_53 can be re-used to support the LPI mode in that country.

As a general summary, no new NS flags are needed to support operation on the unlicensed 6GHz band in South Africa, Argentina, and Singapore.

Proposal 1a:	Re-use 3GPP band n102 for the LPI/VLP operation in South Africa and Singapore.

Proposal 1b:	Re-use 3GPP band n96 for the LPI operation in Argentina.

Proposal 2a:	For the LPI/VLP operation in South Africa, same NS flags as for EU/CEPT can be used.

Proposal 2b:	For the LPI/VLP operation in Singapore, same NS flags as for Australia can be used.

Proposal 2c:	For the LPI operation in Argentina, same NS flag as for Brazil can be used.

Based on the presented considerations, an updated summary of NS values is presented in the Table 2-1 below, where the highlighted parts are new countries and NS values. Same changes are also captured in CR for TR 38.849 [4].

Table 2-1: Updated summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_64 |
| UK | N/A | NS_01 | NS_65 |
| Morocco | N/A | NS_01 | NS_65 |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | NS_68 |
| Qatar | N/A | NS_01 | NS_65 |
| Jordan | N/A | NS_01 | NS_65 |
| Russian Federation | N/A | NS_01 | NS_68 |
| South Africa | N/A | NS_58 | NS_64 |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | [NS_66] |
| Canada | NS_54 | NS_59 | NS_66 |
| Brazil | N/A | NS_53 | NS_67 |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | NS_65 |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | NS_60 | NS_66 |
| Argentina | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_64 |
| Australia | N/A | NS_01 | NS_68 |
| New Zealand | N/A | NS_01 | NS_68 |
| Malaysia | N/A | NS_01 | NS_65 |
| Japan | N/A | NS_63 | NS_69 |
| Singapore | N/A | NS_01 | NS_68 |

Proposal 3:	Update the summary of NS values in TR 38.849.

# 3	Conclusions

In this contribution we have presented summary of the regulatory requirements of the 6GHz band LPI and VLP modes for the countries that were not addressed during the previous meetings. The document also presented the updated summary of the NS values, which can be found in the CR for TR 38.849 [4].

Proposal 1a:	Re-use 3GPP band n102 for the LPI/VLP operation in South Africa and Singapore.

Proposal 1b:	Re-use 3GPP band n96 for the LPI operation in Argentina.

Proposal 2a:	For the LPI/VLP operation in South Africa, same NS flags as for EU/CEPT can be used.

Proposal 2b:	For the LPI/VLP operation in Singapore, same NS flags as for Australia can be used.

Proposal 2c:	For the LPI operation in Argentina, same NS flag as for Brazil can be used.

Proposal 3:	Update the summary of NS values in TR 38.849.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2311213, "Update of the NS values for new countries supporting license-exempt operation on the 6GHz band", Apple Inc.