3GPP RAN WG4 Meeting #108	R4-2311217

# Toulouse, France, August 21 - 25th 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draft | rev | <#> | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the NTN L/S-band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Globalstar Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of the NTN L-/S-band into the 3GPP specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN L-/S-band will not be supported by the 3GPP specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.3.5, 5.3.6(new), 5.4.2.3, 5.4.3.3, 5.4.4, 6.2.1, 6.2.3.1, 6.5.3.2, 6.5.3.3.x1(new), 6.5.3.3.x2(new), 7.3.2, 7.6.2, 7.6.3, 7.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.133 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.2	Operating bands with conducted requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

********** NEXT CHANGED SECTION **********

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n256 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n255 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
| n254 | 15 | 5 | 10 | 15 |  |
|  | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| [n254] | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

********** NEXT CHANGED SECTION **********

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NTN satellite operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NTN satellite operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

********** NEXT CHANGED SECTION **********

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15kHz | Case A | 6215 – <1> – 6244 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1.

Table 5.4.4-1: UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz |
| n255 | -101.5 MHz |
| [n254] | 862 – 885 MHz |

********** NEXT CHANGED SECTION **********

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) |  | Maximum mean power density |
| --- | --- | --- | --- | --- | --- |
| n254 | NS_03N | 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) 15dBm/4kHz (peak limit) |
|  | NS_04N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_05N | 5, 10, 15 | 1610 – 1626.5 |  |

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2.

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in 3GPP TS 38.101-1 [5] clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20 | Table 5.3.2-1 in 3GPP TS 38.101-1 [5] | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.7 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.x1 | n254 | 5 |  | N/A |
| NS_04N | 6.5.3.3.x2 | n254 | 5, 10, 15 |  | N/A |
| NS_05N | 6.5.3.3.x3 | n254 | 5, 10, 15 |  | N/A |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [8] absent is default for all NTN satellite bands.]

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

### 6.2.4	Configured transmitted power

The requirements for configured transmitted power defined in subclause 6.2.4 of 3GPP TS 38.101-1 [5] clause 6.2.4 shall apply to NTN satellite UE.

********** NEXT CHANGED SECTION **********

### 6.5.3	Spurious emission

#### 6.5.3.1	General spurious emissions

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel | BWChannel + 5 |

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |  |

#### 6.5.3.2	Spurious emissions for UE co-existence

This clause specifies the requirements for NR NTN satellite bands for UE coexistence with protected bands.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n254 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n54, n65, n66, n67, n70, n71, n74, n75, n76, n77, n78, n85, n90, n91, n92, n93, n94, n100, n101, n105, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 31, 54, 72, 73, 87, 88, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79, n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n255 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n74, n75, n76, n85, n90, n91, n92, n93, n94, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n256 | NR Band n1, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n26, n28, n29, n30, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n71, n74, n75, n76, n78, n79, n85, n90, n91, n92, n93, n94, n101, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 33, 35, 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |
| NOTE 1: The protected NR or E-UTRA bands are specified in clause 5.2 from 3GPP TS 38.101-1 [5] or 3GPP TS 36.101 [10]. FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in 3GPP TS 38.101-1 [5] or 3GPP TS 36.101 [10]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between n256 and band n2, n25 and n70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

#### 6.5.3.3	Additional spurious emissions

##### 6.5.3.3.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5.3.3.2	Requirement for network signalling value "NS_02N"

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

##### 6.5.3.3.x1	Requirement for network signalling value "NS_03N"

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.x1-1: Additional out-of-band requirements for "NS_03N"

| Frequency band (MHz) |  | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth |  | NOTE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz, 10 MHz, 15 MHz |  |  |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x1-2 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5.3.3.x1-2: Additional in-band requirements for "NS_03N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE: Spectrum emissions are linearly interpolated in dBm versus frequency offset. |  |  |

##### 6.5.3.3.x2	Requirement for network signalling value "NS_04N"

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.x2-1: Additional out-of-band requirements for "NS_04N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x2-2 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5.3.3.x2-2: Additional in-band requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE: Spectrum emissions are linearly interpolated in dBm versus frequency offset. |  |  |

##### 6.5.3.3.x3	Requirement for network signalling value "NS_05N"

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.x3-1: Additional out-of-band requirements for "NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

********** NEXT CHANGED SECTION **********

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2.2 of 3GPP TS 38.101-1 [5], with parameters specified in Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-2.

less than or equal to that specified in Table 7.3.2-2.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  | 24 | 36 | [36] | FDD |
|  | 60 |  | 10 | 18 | [18] |  |
| n254 | 15 | 25 | 50 | 75 |  | FDD |
|  | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1 of 3GPP TS 38.101-1 [5]). |  |  |  |  |  |  |

The minimum requirements specified in Table 7.3.2-1 shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1 of 3GPP TS 38.101-1 [5]) configured.

********** NEXT CHANGED SECTION **********

## 7.6	Blocking characteristics

### 7.6.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

### 7.6.2	In-band blocking

For NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.2-1 and Table 7.6.2-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.2-1: In-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| BWinterferer | MHz | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in 3GPP TS 38.101-1 [5] Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

Table 7.6.2-2: In-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n254, n255, n256 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 |  |  |  |  |

### 7.6.3	Out-of-band blocking

For NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-1: Out-of-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

able 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2580 MHz and FInterferer < 2785 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-1, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  min(BWchannel/2,5) MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

### 7.6.4	Narrow band blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing. The relative throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4-1.

Table 7.6.4-1: Narrow Band Blocking

| Operating Band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 |
| n254, n255, n256 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 3 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |
|  | Fuw (offset SCS= 30 kHz)3 | MHz | NA |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: The PREFSENS power level is specified in clause 7.3.2. NOTE 3: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |