3GPP RAN WG4 Meeting #108		R4-2311231

Toulouse, France, August 21-25, 2023

Agenda item:	5.2.5

Source:	Apple

Title:	Measurement Grids for Optional 6x2 PC3 Antenna Array Configuration

Work Item:	FS_FR2_enhTestMethods

Document for:	Discussion and Decision

Discussion

One of the objectives of the Release-17 Enhanced Testability Item in RAN4 [1] was related to test time reduction.

6. Study testability enhancements to reduce test time - Including RF test method enhancement with reduced test time, and possible test time saving approach for UE Demodulation test and RRM test

There were 2 approaches considered for antenna array configuration e.g., 4x1 [2] and later 8x2 [3].It was found that significant test time reduction was achieved based on the relaxed 4x2 array configuration [4][5].

Similarly, CTIA should also consider 6x2 antenna array configuration to accommodate UEs with more than 4x2 array size. This will help to reduce the test time significantly ~ 30% by reducing the measurement grid test-points from 7.5° (8x2 default) to ~10° (6x2 proposed) based on the analysis for 4x2 antenna config.[6][7]

| Measurement Grid | Default based on 8x2 Antenna Assumption | Optional based on 4x2 Antenna Assumption | Reduction in Test Points |
| --- | --- | --- | --- |
| Beam Peak Search | Number of unique grid Points: 1106 (Dq=Df=7.5°) | Number of unique grid Points: 422 (Dq=Df=12°) | ~0.38 |
| TRP | Number of unique grid Points: 192 (Dq~16.4, Df~18.9°) | Number of unique grid Points: 84 (Dq=Df~24.7°) | ~0.44 |
| Spherical Coverage | Number of unique grid Points: 266 (Dq=Df=15°) | Number of unique grid Points: 266 (Dq=Df=15°) | None |

It is proposed to consider measurement Grids for Optional 6x2 PC3 Antenna Array Configuration an additional test time reduction technique and to capture it in TR38.884.  This can accommodate UEs with different array configurations.

# Proposals

Proposal 1: It is proposed that measurement Grids for Optional 6x2 PC3 Antenna Array Configuration should also be considered for test time reduction and to accommodate UEs with different array config.

Proposal 2:  There is no reduction in measurement uncertainty budget if the OEM opts to use the measurement grid from Proposal 1.

References

- RP-192322, New SID: Study on enhanced test methods for FR2, Apple Inc., CAICT, 3GPP TSG-RAN Meeting #85, September 2019

- R4-1802332, On Constant Density TRP Measurement Grids for mm-wave, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #86, March 2018

- R4-1805995, Way Forward on Measurement Grids for non sparse antenna arrays, Rohde&Schwarz, Anritsu, Intel, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #86bis, April 2018

- R4-2102618, On Test Time Enhancements based on different Antenna Array Assumptions, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #98-e, January 2021

- R5-213812, Measurement Grids for Optional 4x2 PC3 Antenna Array Configuration, Keysight Technologies, 3GPP TSG-RAN5 Meeting #91-e, May 2021

- OTA5GmmW_2023_001_003, Measurement Grids for Alternate 4x2 PC3 Antenna Array Configuration

- OTA5GmmW_2023_002_004, Meas. Grid CR for Alternate 4x2 PC3 Ant. Conf., Keysight