3GPP TSG RAN WG4 Meeting #108		R4-2311239

Toulouse, France, August 21st – 25th 2023

Agenda item:	6.6.1

Source:	Apple

Title:	[NR_unlic_enh-Core] On 100MHz A-MPR for NS_53

WI/SI:	NR_unlic_enh-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

The A-MPR for 100MHz of NS_53 is still in brackets and requires finalisation. This contribution provides additional information on NS_53 A-MPR for 100MHz CBW and proposal for its completion.

# 2	Discussion

The A-MPR for 100MHz of NS_53 is still in brackets and requires finalisation. It has been concluded that from perspective of the out-of-band requirements the NR-U MPR is enough to comply. In this consideration the PSD limit has not been included. The Table 1 provides the required back-off for channel up to 100MHz when considering interlaced allocations and the PSD limit of -1dBm/MHz. It can be observed that 4dB is the minimum required power back-off for a 100MHz channel if occupied with an interlace consisting of 50 RBs. To enable compliance with PSD limits it is proposed to increase DFT-s-OFDM Partial A-MPR to a value of 4dB. A companion CR [1] is provided with the proposed changes.

Table 1: Interlaces – Required power backoff in dB to meet -1dBm/MHz PSD limit

(distance of RBs is assumed to be equal to or larger than 1MHz)

| Number of RBs for the interlace | 10 (20MHz CBW) | 20 (40MHz CBW) | 30 (60MHz CBW) | 40 (80MHz CBW) | 50 (100MHz CBW) |
| --- | --- | --- | --- | --- | --- |
| Backoff required to meet PSD limit | 11.0 | 8.0 | 6.2 | 5.0 | 4.0 |

Proposal: Finalise NS_53 PC5 A-MPR by removing brackets and setting DFT-s-OFDM Partial A-MPR of PI/2 BPSK and QPSK, 16QAM and 64QAM to 4dB.

# 3	Conclusions

This paper contributes to finalising PC5 A-MPR of NS_53. The following proposal is made:

Proposal: Finalise NS_53 PC5 A-MPR by removing brackets and setting DFT-s-OFDM Partial A-MPR of PI/2 BPSK and QPSK, 16QAM and 64QAM to 4dB.

# 4  References

- R4-2311240, “[NR_unlic_enh-Core] CR for TS 38.101-1 Rel-18: Corrections to NS_53 A-MPR for 100MHz”, Apple, 3GPP TSG-RAN WG4 Meeting #108