3GPP TSG RAN WG4 Meeting #108		R4-2311249

Toulouse, France, August 21st – 25th 2023

Agenda item:	8.6.2

Source:	Apple

Title:	On FR2 UL 256QAM

WI:	NR_RF_FR2_req_Ph3-Core

Release:	Rel-18

Document for:	Approval

# 1  Introduction

This contribution discusses certain open topics based on WF [1] from RAN4#107. Simulations results on PTRS based CPE correction for DFT-s-OFDM and CP-OFDM are provided and certain thoughts on dynamic range are made.

# 2  Discussion

#### 2.1 PTRS configuration for DFT-s and CP-OFDM

During RAN4#106bis-e and RAN4#107 the PTRS configuration handling was discussed. One option is to define a fixed PTRS configuration for all devices and the other is to let the UE signal its recommended PTRS configuration for optimum UL performance. The last meetings have not seen fundamental movements between the two main options. To yet no agreement could be reached. The WF [1] captures to collect more data for final decisions making on PTRS with CP-OFDM and for PTRS with DFT-s-OFDM.

The gain of different PTRS configuration were analysed in [2] [3] and it was found that the gain is dependent on the device specific phase noise profile with respect to the applied PTRS configuration. In [5] results indicate that DFT-s-OFDM can provide positive or negative gains dependent on the RB configuration. It seems that larger allocations might not benefit but have negative gains.

#### 2.2 Results on PTRS performance

The WF [1] encourages companies to provide further evaluation on the performance of PTRS with narrow RB allocations and the use of DFT-s-OFDM:

To obtain insights on the performance of 256QAM with PTRS several allocation sizes were simulated with and without PTRS. A 100MHz channel is used with 120kHz SCS. The frequency is set to 29GHz, and the first pole-zero model from Issue 1-1-1 of [1] is used.

In case of CP-OFDM the PTRS information is injected in frequency domain. Every second RB contains one PTRS occurrence. The phase information is averaged across those PTRS occurrences for each symbol and then used to rectify the CPE. Allocations sizes from 2RBs up to 66RBs have been considered and a consistent gain in EVM is observed for all allocations.  Table 1 summarizes the results. These results are in line with previous findings [6]. It is therefore proposed to deploy PTRS for 256QAM and adopt L-PTRS = 1 K-PTRS =2 as fixed configuration.

Table 1: 29GHz, CP-OFDM, 100MHz CBW, 120kHz SCS

|  | No CPE correction | With CPE correction | Delta (Gain of PTRS) |
| --- | --- | --- | --- |
| 2RBs | -33.7 | -34.1 | +0.4 |
| 4RBs | -32.8 | -33.4 | +0.6 |
| 16RBs | -31.6 | -32.4 | +0.8 |
| 64RBs | -29.9 | -30.6 | +0.7 |
| 66RBs | -29.9 | -30.5 | +0.6 |

Observation 1: PTRS performance evaluation with CP-OFDM shows positive gains for small and large allocation sizes. No negative gains are observed. This falls in the line with previous observations from other companies.

Proposal 1: It is proposed to deploy PTRS for 256QAM and adopt L-PTRS = 1 K-PTRS =2 as fixed configuration.

The results for DFT-s-OFDM are captured in table 2. In case of DFT-s-OFDM the PTRS information is injected in time domain. The phase is estimated at each PTRS group and then linearly interpolated between adjacent PTRS groups. Each group consists of four chips and four/eight groups are used per data symbol. In our simulations we found no performance gain with active CPE correction through PTRS. There exist degradations for all RB allocations sizes. This is partly in line with previous findings. It has been found in [6] that larger allocations sizes have negative performance gain with PTRS based CPE correction. Consequently, it was proposed to only consider allocations sizes smaller than 20RBs. Since our simulations also indicate losses with smaller RBs sizes it is therefore proposed to not use PTRS for DFT-s-OFDM.

Table 2: 29GHz, DFT-s-OFDM, 100MHz CBW, 120kHz SCS

|  | No CPE correction | With CPE correction | Delta (Gain of PTRS) |
| --- | --- | --- | --- |
| 2RBs | -33.1 | -32.5 | -0.6 |
| 4RBs | -32.5 | -31.7 | -0.8 |
| 16RBs | -31.3 | -29.8 | -1.5 |
| 64RBs | -29.7 | -28.6 | -1.1 |

Observation 2: PTRS performance evaluation with DFT-s-OFDM shows negative gains for large allocation sizes. Also, smaller allocations sizes have negative gains which makes PTRS not usable for DFT-s-OFDM.

Proposal 2: It is proposed to not deploy PTRS for DFT-s-OFDM.

#### 2.2 MPR and Dynamic Range

During the discussion on potential MPR values for 256QAM it was quickly identified that some power classes and bands might end up with low dynamic range. This section provides an updated estimation for dynamic range based on last meeting agreement [1]. Dynamic range is calculated by using the min peak EIRP and subtracting the minimum EIRP requirements for EVM test. The results are provided in Table 1.

The values in Table 1 are further limited by MPR. The following thought process assumes that the targeted dynamic range would be at least 10-11dB and considers that the MPR might have similar range for CP-OFDM. The color coding in Table 1 shall be an indicator where the range might be considered sufficient (green), marginal (yellow) and deficient (red).

Table 1: Maximum dynamic range (Updated according to last meetings agreement [1])

For PC1 it seems that the high band region has deficient dynamic range which makes deployment challenging. Those bands have been ruled out by link level simulations anyways as it was found that there is no achievable throughput gain for 256QAM with reasonable SNR levels. The mid bands in the range of 37 to 43.5GHz would feature low dynamic range for PC1 and PC5. In contrast PC2 could have sufficient range for all bands due to the considerably lower minimum EIRP requirements.

The issue with low dynamic range has been identified in RAN4#106 and potential solutions were shortly discussed. One option under discussion is to confine the MPR values for 256QAM. Confining MPR is controversial therefore RAN4 considered to first focus on MPR simulations before agreeing further steps. RAN4 agreed the following:

Simply capping MPR/A-MPR to a certain level without technical justification is not part of any RAN4 approach. Hence, it was proposed that advanced UE implementation technologies might be included in the analysis (according to Issue 2-1-4 in [5]). The challenge with advanced UE technologies is that those require clarification as they might reach beyond typical RAN4 assumptions. Detailed analysis and measurements would be needed to support and verify simulated performance improvements. Furthermore, it would need to be guaranteed that implementation is feasible with current technology to not specify requirements which might only be achievable in a certain future.

Observation 3: Considering advanced UE implementation technologies for MPR confinement has the challenge that those require clarification as they might reach beyond typical RAN4 assumptions. Detailed analysis and measurements would be needed to support and verify simulated performance improvements.

# 3  Conclusions

This contribution discusses the EVM budget and breakdown for FR2 256QAM. The following observations and proposals are made:

Observation 1: PTRS performance evaluation with CP-OFDM shows positive gains for small and large allocation sizes. No negative gains are observed. This falls in the line with previous observations from other companies.

Proposal 1: It is proposed to deploy PTRS for 256QAM and adopt L-PTRS = 1 K-PTRS =2 as fixed configuration.

Observation 2: PTRS performance evaluation with DFT-s-OFDM shows negative gains for large allocation sizes. Also, smaller allocations sizes have negative gains which makes PTRS not usable for DFT-s-OFDM.

Proposal 2: It is proposed to not deploy PTRS for DFT-s-OFDM.

Observation 3: Considering advanced UE implementation technologies for MPR confinement has the challenge that those require clarification as they might reach beyond typical RAN4 assumptions. Detailed analysis and measurements would be needed to support and verify simulated performance improvements.

# 4  References

- R4-2310260, “WF on UL 256QAM requirements”, Xiaomi, 3GPP TSG-RAN WG4 Meeting #107

- R4-2216784, “On enabling FR2 UL256QAM”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #104-Bis-e

- R4-2218040, “On enabling FR2 UL256QAM”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #105

- R4-2212371, “Discussion on phase noise and EVM budget for 256QAM”, Apple, 3GPP RAN WG4 Meeting #104-e

- R4-2303709, “WF on FR2-1 UL 256QAM”, Xiaomi, 3GPP TSG-RAN WG4 Meeting #106

- R4-2304601, „On enabling FR2 UL256QAM“, Qualcomm Incorporated, 3GPP TSG-RAN WG4 #106-Bis-e

- R4-2306603, “WF on FR2-1 UL 256QAM”, Xiaomi, 3GPP TSG-RAN WG4 Meeting #106bis-e