3GPP RAN WG4 Meeting #108		R4-2311251

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.22.2

Source:	Apple

Title:	RSD analyses for new PC2 FDD bands

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The Rel-18 basket WID for NR FR1 PC2 FDD bands has been approved in RAN #97-e meeting [1]. The initially proposed bands include n2, n5, n8, n13, n25, n26, n28, n66, n71, and n85 where the RSD analysis for bands subject to further REFSENS degradation due to increased UL interference caused by 3dB higher maximum output power from PC3 to PC2 have mostly been performed. In RAN #100 meeting, three new PC2 FDD bands n7, n14, and n70 were proposed and included in the revised WID [2]. On the other hand, it has also been agreed that the PC2 RSD for the newly proposed n71 UL CBWs 25MHz, 30MHz, and 35MHz for symmetric UL/DL CBWs are to be handled within the PC2 FDD bands basket WID. In this contribution, we provide our RSD analyses for the three newly proposed PC2 FDD bands and 25MHz/30MHz/35MHz symmetric UL/DL CBWs for n71. For n70, since the 300MHz duplex spacing is wider than that of n1 which has been concluded not subject to RSD under PC2, we presume that no RSD is required without further analysis. For n7, n14, and n71, we have performed simulations and link analysis to derived 1Tx and 2Tx RSD respectively.

# Discussion

## 2.1	PC2 RSD with 1Tx configuration

The RSD analysis is based on the simulation assumptions as summarized in Table 2.1-1 where only Tx noise into Rx band was considered.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Tx power | 26 dBm | At antenna port |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |
| n7 channel BW | UL/DL: 5/5, 10/10, 15/15, 20/20, 25/25, 30/30, 35/35, 40/40, 50/50 | MHz |
| n14 channel BW | UL/DL: 5/5, 10/10 | MHz |
| n71 channel BW | UL/DL: 25/25, 30/30, 35/35 | MHz |

Table 2.1-1 Simulation assumptions for RSD analysis

Table 2.1-2 summarizes RSD results from our simulations.

| Band | Channel BW (MHz) / RSD (dB) (symmetric UL/DL CBW) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n7 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | N/A | 2.0 |
| n14 | 0.3 | 0.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n71 | Done | Done | Done | Done | 1.7 | 1.5 | 1.5 | N/A | N/A | N/A |

Table 2.1-2 1Tx PC2 RSD relative to PC3 REFSENS

## 2.2	PC2 RSD with 2Tx configuration

The 2Tx RSD analysis was based on the PC3 REFSENS requirements to first extract the Tx noise, assuming antenna isolation is 10 dB. The Tx noise was then added to thermal noise for both main and diversity Rx paths, followed by MRC to calculate the equivalent total noise for 2Tx configuration. The RSD was then derived by taking the difference between the 2Tx PC2 equivalent total noise and the PC3 equivalent total noise. On the other hand, the PC3 REFSENS was assumed with 1dB thermal noise difference between main Rx path and diversity Rx path to account for the insertion loss difference between using a duplexer and a DL only filter.

Table 2.2-1 summarizes RSD results based on our analyses.

| Band | Channel BW (MHz) / RSD (dB) (symmetric UL/DL BW) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n7 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | N/A | 5.6 |
| n14 | 0.8 | 0.8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n71 | Done | Done | Done | Done | 7.4 | 7.7 | 7.8 | N/A | N/A | N/A |

Table 2.2-1 2Tx PC2 RSD relative to PC3 REFSENS

# 3	Conclusion

In this contribution, we provide our RSD analyses for the three newly proposed PC2 FDD bands, n7, n14, and n70.

Proposal 1: RAN4 to take the RSD values in the table below into consideration for 1Tx PC2 RSD relative to PC3 REFSENS.

| Band | Channel BW (MHz) / RSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n7 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | N/A | 2.0 |
| n14 | 0.3 | 0.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n70 | 0 | 0 | 0 | 0 | 0 | N/A | N/A | N/A | N/A | N/A |
| n71 | Done | Done | Done | Done | 1.7 | 1.5 | 1.5 | N/A | N/A | N/A |

Proposal 2: RAN4 to take the RSD values in the table below into consideration for 2Tx PC2 RSD relative to PC3 REFSENS.

| Band | Channel BW (MHz) / RSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n7 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | N/A | 5.6 |
| n14 | 0.8 | 0.8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n70 | 0 | 0 | 0 | 0 | 0 | N/A | N/A | N/A | N/A | N/A |
| n71 | Done | Done | Done | Done | 7.4 | 7.7 | 7.8 | N/A | N/A | N/A |

# 4	References

RP-222625 “New WID: High power UE for FR1 for FDD single band(s) with power class 2”, China Unicom, 3GPP TSG-RAN Meeting #97-e, Online, September 12th – 16th, 2022

RP-231064 “Revised WID High power UE (power class 2) for NR FR1 FDD single band”, China Unicom, 3GPP TSG-RAN Meeting #100, Taipei, Taiwan, June 12th – 14th, 2023