3GPP TSG RAN WG4 Meeting #108		 R4-2311274

# Toulouse, France, August 21st – 25th, 2023

Source:	DISH Network

Title:	Adding asymmetric channel bandwidth of UL 15/ DL 35 MHz for n66

Agenda item:			7.1  (TEI18: NR_FR1_35MHz_45MHz_BW)

Document for:	Approval

# 1. Introduction

This paper explains the motivation, necessary changes and RF impact to support asymmetric UL15 MHz/DL 35 MHz for n66. This paper also provided the proposed changes to 38.101.

# 2. Discussion

Motivation

The ability to fully utilize the maximum licensed uplink and downlink bandwidth that an operator’s owned is crucial to ensure a competitive end to end service quality. n66 is one of the main bands for DISH. However, as per today’s specification, the asymmetric UL/DL bandwidth combination of UL15/DL35 MHz cannot be utilized because it is not supported in the standards.

Specification status

The following is the status of specification for n66, per 38.101-1 ver. 18.2.0:

- n66 definition

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |

- The 15MHz and 35MHz CBW are defined for UL and DL

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |

- The asymmetric UL and DL channel bandwidth combination for n66 does not have combination for UL15MHz/DL35MHz

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |

RF impact

The separation of Rx-Tx of n66 is 400 MHz according to 38.101 clause 5.4.4, hence no RF impact is foreseen.

Based on the analysis above, it is proposed to add asymmetric UL 15 MHz/ DL 35 MHz to n66.

Proposal 1: RAN4 to agree to add asymmetric UL 15 MHz/ DL 35 MHz for n66

RAN4 specification works related to this proposal

The specification works related to n66, i.e., asymmetric channel bandwidth for n66, additional bandwidth support, are as follows:

- n66 asymmetric bandwidth: 
Agreed in R4-1805444 (DISH Network), Rel-15 WI: NR_newRAT

- adding 25 MHz, 30 MHz channel bandwidth:

Agreed in R4-2002851 (Huawei, HiSilicon), Rel-16 WI: NR_n66_BW

- adding 35 MHz, 45 MHz channel bandwidth:

Agreed in R4-2115101 (Skyworks), Rel-17 WI: NR_FR1_35MHz_45MHz_BW

This proposal builds on the specification works from Rel-15 up to the present release. The affected feature is isolated to operation with asymmetric channel bandwidth for n66. Therefore, we think it is appropriate to treat this proposal as TEI18.

The content of CR for the necessary changes is included below for discussion purpose.

# 3. Conclusion

The motivation, necessary changes and RF impact to support asymmetric UL15 MHz/DL 35 MHz for n66 were explained.

The following is proposed:

RAN4 to agree to add asymmetric UL 15 MHz/ DL 35 MHz for n66

The content of CR for the necessary changes is included below for discussion purpose.

# Reference

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v18.2.0

# Contents of CR for discussion

3GPP TSG-RAN WG4 Meeting #104	DRAFT R4-23xxxxx

# Toulouse, France,  21 - 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101> | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1: Adding asymmetric UL 15/DL 35 MHz for n66 (TEI18: NR_FR1_35MHz_45MHz_BW) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 15MHz and 35MHz channel bandwidth for n66 are already defined, but the asymmetric channel bandwidth combination of UL15 MHz/DL 35 MHz is not defined yet. To fully utilized operator’s licenced spectrum, asymmetric bandwith operation UL15 MHz/DL 35 MHz for n66 is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Asymmetric channel bandwidth of UL 15 MHz/ DL 35 MHz for n66 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operation of asymmetric channel bandwidth of UL 15 MHz/ DL 35 MHz for n66 is not possible, hence operator’s licenced spectrum can not be fully utilized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of changes ---

Unaffected parts ommited

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
|  | 10, 15, 20 | 25, 35 | 1 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n26 | 20 | 25, 30 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
|  | 5, 10, 15 | 20, 25, 30, 35, 40 | 2 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 20 | 25, 30, 35 | 2 |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n105 | 20 | 25, 30, 35 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

--- End of changes ---