3GPP TSG-RAN4 Meeting #108	R4-2311284

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0625 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Feature CR for FR2 multi-Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New feature introduced as 7.3J and called ‘Spherical coverage requirement for simultaneous reception from two angles of arrival’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The same requirement applies for sDCI and mDCI UEs. - Capability list reflect previous agreements. - The requirement applies for downlink signals from the 2 AoAs with overlapping RBs in both time and frequency. - The requirements are derived in context of the agreed test system (R4-2306604). The requirements may not apply for a different positioning system. It is therefore necessary to clarify this detail by including applicable test system detail - The foreseen system has a 2-axis positioner. - The positioner motion and the sources locations are such that the sources both TRPs appear at the same spatial grid points relative to the UE, and that the AoA pairs are always along meridians as viewed from the UE. - Wording can optionally list the types of positioners, but these may need definition of new terms. - DL power condition per TRP is per agreement in R4-2306604 (legacy EIS spherical coverage condition) - The requirement metric is aligned with the baseline assumptoin of R4-2306604. Further detail is included in normative Annex K. - Wording is provided for both, OR and arithemetic mean combining. To be down selected - DL RMCs: - mDCI UEs: FRC reused from legacy sensitivity requirement - sDCI UEs: new FRC assuming 2L single TB. (data rates are roughly 2x legacy data rates) - Requirement is limited to PC3. - Main wording needs to be downselected between one orientation per AoA separation and one orientation for all separation conditions. - Three options for requiements provided for visualization, to be down selected. - UL for this requirement is limited to one of the TCI-states from the pair used for DL. The conditions are replicated from legacy sensitivity test (transmitter set to PUMAX, RMC in annex). The test metric is pass/fail for each AoA pair. The grid is the same as the 2AoA spherical coverage grid, described in Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

7.3J	Spherical coverage requirement for simultaneous reception from two angles of arrival

### 7.3J.0	General

For this release, the requirement applies only to FR2-1 UEs that support the following set of capabilities:

- simultaneousReceptionDiffTypeD-r16

- 4L per CC in DL

- either of:

  - singleDCI-SDM-scheme-r16 or

  - multiDCI-MultiTRP-r16 and either of:

    - overlapPDSCHsFullyFreqTime-r16.

    - overlapPDSCHsInTimePartiallyFreq-r16

The requirement applies for simultaneous reception of rank 2 PDSCH, where each layer uses overlapping RBs in both time and frequency  and is associated with a unique TCI state and AoA. The scheduled TCI states for the rank 2 PDSCH shall be configured with different QCL type-D reference signals respectively. The DL power at the center of quiet zone from each AoA equals the EIS spherical coverage requirement from sub-clause 7.3.4.

For UEs supporting singleDCI-SDM-scheme-r16, the cumulative throughput in the DL associated with both TCI-states shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified A.3.3.2-5 and A.3.3.2-6 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

For UEs supporting multiDCI-MultiTRP-r16, the throughput in the DL associated with each TCI-state shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified A.3.3.2-1 and A.3.3.2-2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to the downlink transmission bandwidth. The UL is assigned to any one of the two TCI-states scheduled for simultaneous DL, with reference measurement channel as specified in Annex A.2.3.2. The transmitter shall be set to PUMAX as defined in clause 6.2.4.

Unless otherwise specified, the minimum requirements shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

### 7.3J.1	(Reserved)

### 7.3J.2	(Reserved)

### 7.3J.3	Power class 3

The requirements apply to the UE when tested in a test system as described in Annex K. The requirement is verified with the test metric of throughput (Link= 2AoA spherical coverage grid, Meas=Link Angle).

The spherical coverage requirement for simultaneous reception from two angles of arrival applies to the probability to support simultaneous reception of rank 2 PDSCH defined in sub-clause 7.11.0. The probability (see Annex K) is defined as the spatial average over the full sphere around the UE of the probability of any one direction to support 2 AoA reception. In the applicable test system (see Annex K), the probability of any one direction of the UE to support 2 AoA reception for any specific AoA separation is [….]

[(Option 1 – arithmetic mean combining) the ratio of the number of unique AoA pairs that include that direction and can support 2 AoA reception to the total number of verified unique AoA pairs that include that direction.]

[(Option 2 – OR combining) 1 if any of the AoA pairs that include that direction and can support 2 AoA reception.]

The requirement applies only for the UE’s declared orientation in the positioner of the test system. The requirement for each AoA separation condition applies only for the UE’s declared orientation in the positioner of the test system for that AoA separation. The minimum required overall probability to support 2 AoA reception for power class 3 UEs for any channel bandwidth is included in table 7.3 J.3-1.

Option 1: Table 7.3J.3-1: Requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| [60] | F60 |
| [150] | F150 |

Option 2: Table 7.3J.3-1: Requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| One value chosen by UE by declaration from {30⁰, 60⁰, 90⁰, 120⁰, 150⁰, 180⁰} | F |

Option 3: Table 7.3J.3-1: Requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| One value chosen by UE by declaration from {30⁰, 60⁰, 90⁰} | FlowAoAoffset |
| One value chosen by UE by declaration from {120⁰, 150⁰, 180⁰} | FhighAoAoffset |

### 7.3J.4	(Reserved)

### 7.3J.5	(Reserved)

## <<< END OF CHANGES >>>

## <<< START OF CHANGES >>>

#### A.3.3.2-5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2-5: Fixed Reference Channel for Receiver Requirements (SCS 60 kHz, TDD)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 23 | 23 |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 8456 | 16896 | 33816 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | CBs | 1 | 2 | 2 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 28512 | 57024 | 114048 |
| Max. Throughput averaged over 1 frame | Mbps | 19.449 | 38.861 | 77.777 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {3,4,5,6,7} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {0,1,2} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |

#### A.3.3.2-6	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2-6: Fixed Reference Channel for Receiver Requirements (SCS 120 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 47 | 47 | 47 | 47 |
| MCS index |  | 4 | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 4096 | 8456 | 16896 | 33816 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | CBs | 1 | 1 | 2 | 2 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 13824 | 28512 | 57024 | 114048 |
| Max. Throughput averaged over 1 frame | Mbps | 19.660 | 40.589 | 81.101 | 162.317 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {7,…,15} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {0,…,6} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |  |

## <<< END OF CHANGES >>>

## <<< START OF CHANGES >>>

Annex K (normative): Requirement metric for 2AoA reception

# K.1	Spherical coverage grid for 2AoA reception

For the calculation of the probability metric, the reference coordinate system grid is defined as follows: The grid is the set of locations described by a source/probe from the UE’s perspective when the TE completes a 3D spherical scan with a constant step size for each rotational axis of its positioner. The grid comprises intersection points of circles of fixed polar angle (geographic analog: ‘latitude’) and meridians of fixed azimuthal angle (geographic analog: ‘longitude’) and completely covers the sphere around the UE. A valid set of locations for the source/probes are those that describes the same constant step size grid from the UE’s perspective over a complete spherical scan. For 2AoA connectivity, this condition is satisfied when the active probes/sources are always contained in a plane of constant azimuthal angle ‘’ in this reference system. The reference coordinate system with example UE orientations are shown in figures in tables J.2-x.

# K.2	Probability to support 2AoA reception

For UEs required to fulfil a requirement on the probability for 2AoA reception, the metric for a given AoA separation is the spatial average:

Pspatial_av= θ1φ1Pdirectional  (θ1,φ1).AW(θ1)

Where:

(1,1) are coordinates of any grid point in the UE-centered reference system described above.

AW(1) is the area weight associated with each grid point with polar angle ‘1’, and:

θ1φ1AW(θ1)=1

Pdirectional(1,1) is the probability that the direction given by coordinates (1,1) can support 2 AoA reception under the agreed UERF test conditions. Pdirectional(1,1)  is given by:

[(Option 1 – arithmetic mean combining)

Pdirectional(θ1,φ1)= {PFAoApair#1+PFAoApair#2+…PFAoApair#nn}

]

[(Option 2 – OR combining)

Pdirectional(θ1,φ1)= OR{PFAoApair#1,PFAoApair#2,…PFAoApair#n}

]

Where:

AoApair#x represents an AoApair with the chosen AoA separation of test that has one source at coordinates (1,1).

‘n’ is the maximum number of unique AoApairs that the TE can generate with one source at coordinates (1,1)

PF(AoApair#x) is the pass/fail outcome (1/0 respectively) of the throughput verification  test

## <<< END OF CHANGES >>>