3GPP TSG-RAN WG4 Meeting #108	 R4-2311303

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0015 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS36.102: Corrections to IoT-NTN requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are squre brackets in current specification - The bandwidth for Test 3 in Table 8.2.1.1.1.1-2 is wrong - The SNR requirements for Test 2 in Table 8.2.1.1.1.1-2 is wrong (R4-2311524) - No Annex C.3.2 in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove squre brackets - Correct the bandwith from 10MHz to 1.4MHz for Test 3 in Table 8.2.1.1.1.1-2 - Correct SNR requirements for Test 2 in Table 8.2.1.1.1.1-2 (R4-2311524) - Correct the Annex numbering for B.3.2 and B.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Specification includes brackets - The bandwidth for Test 3 in Table 8.2.1.1.1.1-2 is wrong - The SNR requirements for Test 2 in Table 8.2.1.1.1.1-2 does not follow the simulation reults - Wrong Annex numbering |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1, 8.3.1.1.1.1, B.3.2, B.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 8.2.1.1.1	Single-antenna port performance

8.2.1.1.1.1	Minimum Requirements

The requirements are specified in Table 8.2.1.1.1.1-2, with the addition of the parameters in Table 8.2.1.1.1.1-1, and the downlink physical channel setup according to Annex B.3.2. The purpose is to verify the performance of single antenna port configuration.

Table 8.2.1.1.1.1-1: Test Parameters for single antenna port (FRC)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | ρA |  | -3 | -3 | -3 |
|  | ρB |  | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  | σ |  | 0 | 0 | 0 |
|  | δ |  | 3 | 3 | 3 |
| Noc at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode B |
| PDSCH transmission mode |  |  | 1 | 1 | 1 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 1 | 8 | 64 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | N/A | N/A | N/A |
| Frequency hopping interval (interval-FDD) |  | ms | N/A | N/A | N/A |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 1 | 8 | 64 |
| MPDCCH repetition number |  |  | 1 | 8 | 64 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | N/A | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | 4 | 2.5 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 | 0 |
| MPDCCH aggregation level |  |  | 8 | 24 | 24 |
| Note 1: PB=1. Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [3] or/and TS 36.213 [12] as appropriate. |  |  |  |  |  |

Table 8.2.1.1.1.1-2: Minimum performance for single antenna port (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

END OF CHANGE 1

START OF CHANGE 2

8.3.1.1.1.1	Minimum Requirements for standalone

The requirements are specified in Table 8.3.1.1.1.1-2, with the addition of the parameters in Table 8.3.1.1.1.1-1 and the downlink physical channel setup according to Annex B.3.3. The purpose of these tests is to verify the performance.

Table 8.3.1.1.1.1-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 128 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

END OF CHANGE 2

START OF CHANGE 3

## B.3.2	Measurement of Performance requirements

Table B.3.2-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table B.3.2-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | EPRE Ratio |
| --- | --- |
| PBCH | PBCH_RA = A+ |
|  | PBCH_RB = B+ |
| PSS | PSS_RA = 0 (Note 3) |
| SSS | SSS_RA = 0 (Note 3) |
| MPDCCH | MPDCCH_RA = A+δ |
|  | MPDCCH_RB = B+δ |
| PDSCH | PDSCH_RA = A |
|  | PDSCH_RB = B |
| OCNG | OCNG_RA = A+ |
|  | OCNG_RB = B+ |

NOTE 1:	A= B = 0 dB means no RS boosting.

NOTE 2:	OCNG are not defined downlink physical channels in [3].

NOTE 3: Assuming PSS and SSS transmitted on a single antenna port.

NOTE 4: A, B, , and δ are test specific.

Table B.3.2-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Total transmitted power spectral density Ior | dBm/15 kHz | Test specific | 1. Ior shall be kept constant throughout all OFDM symbols |
| Cell-specific reference signal power ratio ERS/Ior |  | Test specific | 1. Applies for antenna port p |
| Energy per resource element EPRE |  | Test specific | 1. The complex-valued symbols yp(i) and ak,l(p) defined in TS 36.211 [3] shall conform to the given EPRE value. |

## B.3.3	Measurement of Receiver Characteristics for Narrowband IoT

For the performance requirements for Narrowband IoT, the power allocation for the physical channels is listed in Table B.3.3-1

Table B.3.3-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | EPRE Ratio for one NRS antenna port | EPRE Ratio for two NRS antenna ports |
| --- | --- | --- |
| NPBCH | 0 dB | -3 dB |
| NPDCCH | 0 dB | -3 dB |
| NPDSCH | 0 dB | -3 dB |
| NPSS | 0 dB | 0 dB |
| NSSS | 0 dB | 0 dB |

NOTE 1:	Assuming NPSS and NSSS transmitted on one NRS antenna port.

Table B.3.3-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Transmitted power spectral density Ior | dBm/15 kHz | Test specific | Ior shall be kept constant throughout all OFDM symbols |
| Narrowband reference signal power ratio ECRS/Ior |  | 0 dB | Applicable for Stand-alone operation |
| Narrowband reference signal power over cell-specific reference signal power ENRS/ERS |  | 0 dB | Applicable for In-band operation |

END OF CHANGE 3