3GPP TSG-RAN WG4 Meeting # 108		R4-2311322

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.7.3.4

Source:	Apple

Title:	On scheduling/measurement restrictions for multiple Rx chains

Document for:	Discussion

# Introduction

In RAN4#107 meeting, the WF[1] for WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1) was agreed. In this contribution, we discuss the open issues for scheduling and measurement restriction for UE with FR2 multi-Rx chain DL reception capability.

# Discussion on scheduling restriction

3.1 Scheduling restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP)

Followings were the agreements in last meeting,

### Issue 3-1-1: Conditions/cases that scheduling restriction for L1 measurements can be relaxed for multi-Rx <Agreement >: - In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and CSI-RS is transmitted from anyone of the TRPs, scheduling restriction relaxation can be made for CSI-RS based L1 measurements with multi-Rx when following conditions are met - The CSI-RS is not in a CSI-RS resource set with repetition ON - The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time - CSI-RS has same QCL source as the active TCI state of any one of the PDCCH/PDSCHs - Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair - UE is activated with multi-Rx operation - Note: The wording on the 2nd and 3rd bullet can be further polished. ### Issue 3-1-2: Conditions/cases that scheduling restriction can be relaxed for SSB based measurements for multi-Rx <Agreement >: - For SSB based L1 measurements, scheduling restrictions relaxation is not considered for UE supporting Rel-18 multi-Rx capability.

The scenario for scheduling restriction relaxation is like in the following figure: the CSI-RS for L1 measurement is colliding with PDCCH1/PDSCH1 and PDCCH2/PDSCH2 on time domain, and CSI-RS is QCLed typeD with the active TCI of PDCCH1/PDSCH1. Since CSI-RS1 and PDCCH1/PDSCH1 has no scheduling restriction as specified in legacy requirement, CSI-RS1 will not cause scheduling restriction on PDCCH2/PDSCH2 if resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair.

Figure 1. Scenario for scheduling restriction relaxation

The 2nd bullet and 3rd bullet in the last agreement is not very clear for the scenario interpretation. Thus, we propose to revise the last agreement to the following:

- In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and CSI-RS is transmitted from anyone of the TRPs, scheduling restriction relaxation can be made for CSI-RS based L1 measurements with multi-Rx when following conditions are met

  - The CSI-RS is not in a CSI-RS resource set with repetition ON

  - The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time  The CSI-RS and both of the PDCCH/PDSCHs are transmitted at the same time

  - CSI-RS has same QCL source as the active TCI state of any one of the PDCCH/PDSCHs  CSI-RS has same QCL source as the active TCI of the PDCCH/PDSCH from the same TRP

  - Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

  - UE is activated with multi-Rx operation

Proposal 1: the last agreement on scheduling restriction for L1 measurement shall be revised as:

- In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and CSI-RS is transmitted from anyone of the TRPs, scheduling restriction relaxation can be made for CSI-RS based L1 measurements with multi-Rx when following conditions are met

  - The CSI-RS is not in a CSI-RS resource set with repetition ON

  - The CSI-RS and both of the PDCCH/PDSCHs are transmitted at the same time

  - CSI-RS has same QCL source as the active TCI of the PDCCH/PDSCH from the same TRP

  - Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

  - UE is activated with multi-Rx operation

3.2 Scheduling restriction for L3 measurement

The options for scheduling restriction for L3 measurement were captured as:

### Issue 3-1-3: Conditions/cases that scheduling restriction for L3 measurements can be relaxed for multi-Rx <Way forward >: - Proposals - Option 1: Scheduling restriction for L3 measurements is not enhanced for multi-RX operation - Option 2: Scheduling restriction for L3 measurements is enhanced for multi-RX operation

For SSB based L3 measurement for mobility, the beam sweeping is always assumed on the whole spherical area to detect the potential neighbor cells’ SSBs, and on those SSBs for L3 measurement, the scheduling restriction is applied. However, since the multi-Rx chain UE now has capability of simultaneous reception on different AoAs, it would be possible to avoid scheduling restriction on some of the L3 measurement occasions. As shown in the following figure 2 as an example, the rough Rx beam sweeping on right panel will not cause interruption to the data/control channel reception on the left panel.

Figure 2. Example for scheduling restriction on L3 measurement

However, the only difficult part is that network is not aware of which L3 measurement occasions can be used for scheduling. We do observe the benefit to enhance the throughput for the UE to mute the scheduling restriction on some of the L3 measurement occasions, e.g., in the figure 2 case, 50% of throughput loss can be saved compared with legacy requirement. Since L3 measurement is periodic and quite static (RRC configuration based), RAN4 can further discuss how to indicate the scheduling restriction information to network, and network can use the some of the L3 measurement occasions for scheduling. Details of indication and signaling can be FFS, e.g., UE may indicate a periodic pattern to network to tell which L3 occasion can have no scheduling restriction, but we also agree that when the scheduling active TCI is changed, UE may need to indicate a new pattern then.

Proposal 2: Scheduling restriction for L3 measurements is enhanced for multi-RX operation.

However, due to the last meeting discussion, it has not sufficient time to discuss all kinds of enhancements in this WI, and therefore we can also compromise to not consider the scheduling restriction relaxation for L3 measurement in this release.

# Discussion on measurement restriction for L1 measurement

The agreements in last meeting was:

### Issue 3-2-1: Conditions/cases that measurement restriction for L1 measurements can be removed for multi-Rx <Way forward >: - Proposals - Option 1: In multi-TRP operation scenario, when the CSI-RS for RLM/BFD/L1-RSRP measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, measurement restriction relaxation can be made for the CSI-RS based L1 measurements with multi-Rx when following conditions are met - The CSI-RS for RLM/BFD/L1-RSRP or the other CSI-RS in a resource set configured with repetition ON - [The two CSI-RSs are transmitted through different TRPs at the same time] - [QCL source of] the two CSI-RSs have been reported via GBBR in a pair - UE is indicated/configured with multi-Rx operation - [Other conditions are not precluded] - Option 2: measurement restriction relaxation is up to UE implementation - Option 3: Do not introduce measurement restriction relaxations for L1 measurements for multi-Rx chain UEs

The existing measurement restriction is specified as followings (using RLM as one example):

#### 8.1.2.3 Measurement restrictions for SSB based RLM [unrelated part omitted] For FR2, when the SSB for RLM measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined. [unrelated part omitted]

#### 8.1.3.3 Measurement restrictions for CSI-RS based RLM [unrelated part omitted] For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined. For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, - In the following cases, UE is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined. - The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or - The other CSI-RS is configured in q1 and beam failure is detected, or - The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE, - Otherwise, UE shall be able to measure the CSI-RS for RLM without any restriction. [unrelated part omitted]

Based on the existing measurement restriction requirement, three categories of RS shall be considered when defining measurement restriction requirement:

(1) The SSB for RLM, for BFD/CBD/L1-RSRP are all with beam sweeping

(2) The CSI-RS with beam sweeping: CSI-RS for CBD, CSI-RS for L1-RSRP with repetition on.

(3) The CSI-RS without beam sweeping: CSI-RS for RLM, CSI-RS for BFD, CSI-RS for L1-RSRP with repetition off.

The existing measurement restriction is applied as shown in following table 1. “Yes” means measurement restriction is applied, otherwise “No” means no measurement restriction is applied and parallel measurement shall be performed.

Table 1. Summary for existing measurement restriction applicability

| Scheduling Restriction Applicability | SSB RLM/BFD/CBD/L1-RSRP | CSI-RS RLM/BFD/L1-RSRP(repetition OFF) | CSI-RS CBD/L1-RSRP(repetition ON) |
| --- | --- | --- | --- |
| SSB RLM/BFD/CBD/L1-RSRP | NA | Yes | Yes |
| CSI-RS RLM/BFD/L1-RSRP(repetition OFF) | Yes | QCLed typeD: No Non-QCLed typeD: Yes | Yes |
| CSI-RS CBD/L1-RSRP(repetition ON) | Yes | Yes | Yes |

Thus, the current measurement restriction applies based on the following main principles:

- At least one of the RS needs beam sweeping (e.g., SSB, or CSI-RS for CBD, or CSI-RS for L1-RSRP with repetition ON), or

- These two RSs are not QCLed type D

Observation: the existing measurement restriction applies based on the following main principles:

- At least one of the RS needs beam sweeping (e.g., SSB for RLM/BFD/CBD/L1-RSRP, or CSI-RS for CBD, or CSI-RS for L1-RSRP with repetition ON), or

- These two RSs are not QCLed type D

As shown in figure 3, if the previous GBBR measurement is using the resource sets with repetition ON, UE was using beam sweeping to find the best beam pair for GBBR, e.g., beam 3 and 4, and such beam pair reported in GBBR is to indicate UE can support simultaneous reception with such CSI-RS1 TCI and CSI-RS2 TCI, but for individual L1-RSRP measurement, the beam 1 and beam 2 are the best beams to achieve the best L1-RSRP, respectively. Beam 3 and beam 4 might have low interference for simultaneous reception to each other, however, beam 1 and beam 2 may have high interference and cannot support simultaneous reception. Thus, as long as previous GBBR is based on resource set with repetition ON, measurement restriction shall be applied for the non-GBBR L1 measurement even though the RSs for non-GBBR L1 measurement is reported in a GBBR pair or RSs for non-GBBR L1 measurement is QCLed typed D with the RSs reported in a GBBR pair. Since SSB based L1 measurement is like repetition ON case (Rx beams sweeping is always assumed in both GBBR and non-GBBR L1 measurement), the non-GBBR L1 measurement based on SSB shall be always with measurement restriction.

Figure 3. spatial separation between RS for non-GBBR L1 measurement (previous GBBR is repetition ON)

If the previous GBBR is using resource sets both with repetition OFF, UE used only one Rx beam to perform the GBBR measurement, and if the RSs in non-GBBR L1 measurement is QCLed type D to or same as the RSs in reported GBBR, UE will use the same Rx beams to perform non-GBBR L1 measurement. Even when the non-GBBR L1 measurement is based on resource set with repetition ON, UE will only refine the Rx beams for non-GBBR L1 measurement inside the beam pair of previous GBBR measurement, like shown in figure 4. Thus, if previous GBBR is based on resource set with repetition OFF, measurement restriction shall not be applied for the non-GBBR L1 measurement when the RSs for non-GBBR L1 measurement is reported in a GBBR pair or RSs for non-GBBR L1 measurement is QCLed typed D with the RSs reported in a GBBR pair.

Figure 4. spatial separation between RS for non-GBBR L1 measurement (previous GBBR is repetition OFF)

Proposal 3: Measurement restriction shall be applied for SSB based L1 measurement (RLM/BFD/CBD/L1-RSRP), i.e., no relaxation.

For CSI-RS based L1 measurement, the similar scenario as for scheduling restriction can be considered for possible enhancement. For instance, as shown in figure 5,

Figure 5. Scenarios for CSI-RS based L1 measurement restriction relaxation

In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and two CSI-RS are transmitted from different TRPs, L1 measurement restriction relaxation can be made for CSI-RS based RLM/BFD/L1-RSRP measurements with multi-Rx when following conditions are met:

- both CSI-RS are not in any CSI-RS resource set with repetition ON

- both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time

- Each CSI-RS has same QCL source as the active TCI state of the PDCCH/PDSCH from the same TRP

- Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

- UE is activated with multi-Rx operation

In the example of figure 5, the logic behind can be clarified based on the scheduling restriction agreements:

- CSI-RS1 can be received simultaneously with PDCCH2/PDSCH2

- CSI-RS2 can be received simultaneously with PDCCH1/PDSCH1

- PDCCH1/PDSCH1 can be received simultaneously with PDCCH2/PDSCH2, due to GBBR report

- Therefore, according to (1)(2)(3), CSI-RS1 can be received simultaneously with CSI-RS2.

Proposal 4: In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and two CSI-RS are transmitted from different TRPs, L1 measurement restriction relaxation can be made for CSI-RS based RLM/BFD/L1-RSRP measurements with multi-Rx when following conditions are met:

- both CSI-RS are not in any CSI-RS resource set with repetition ON

- both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time

- Each CSI-RS has same QCL source as the active TCI state of the PDCCH/PDSCH from the same TRP

- Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

- UE is activated with multi-Rx operation

# Conclusion

In this contribution, we discuss the measurement and scheduling restriction for multi-Rx chain capable UE.

Proposal 1: the last agreement on scheduling restriction for L1 measurement shall be revised as:

- In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and CSI-RS is transmitted from anyone of the TRPs, scheduling restriction relaxation can be made for CSI-RS based L1 measurements with multi-Rx when following conditions are met

  - The CSI-RS is not in a CSI-RS resource set with repetition ON

  - The CSI-RS and both of the PDCCH/PDSCHs are transmitted at the same time

  - CSI-RS has same QCL source as the active TCI of the PDCCH/PDSCH from the same TRP

  - Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

  - UE is activated with multi-Rx operation

Proposal 2: Scheduling restriction for L3 measurements is enhanced for multi-RX operation.

Proposal 3: Measurement restriction shall be applied for SSB based L1 measurement (RLM/BFD/CBD/L1-RSRP), i.e., no relaxation.

Proposal 4: In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and two CSI-RS are transmitted from different TRPs, L1 measurement restriction relaxation can be made for CSI-RS based RLM/BFD/L1-RSRP measurements with multi-Rx when following conditions are met:

- both CSI-RS are not in any CSI-RS resource set with repetition ON

- both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time

- Each CSI-RS has same QCL source as the active TCI state of the PDCCH/PDSCH from the same TRP

- Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

- UE is activated with multi-Rx operation

# References

[1] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, RAN4 #107.