3GPP TSG-RAN WG4 Meeting # 108	R4-2311352

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.18.1.1

Source:	Apple

Title:	On advanced receiver to cancel intra-user interference for MU-MIMO

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#107 PDSCH demodulation requirements for MU-MIMO with advanced receiver were discussed and way forward [1] was agreed.  In this contribution we present our views on receiver assumptions for advanced receiver considered for mitigating inter- user interference in MU-MIMO.

# 2. Discussion

## Reference Receiver

For reference receiver the following options were captured in [1]:

Reference receiver

Candidate options:

  - Option 1: Down select to R-ML as the reference receiver

  - Option 2: Make decision later

  - Option 3: Keep open in case requirements are to be defined for up to 4 total layers and with high modulation orders

In our companion paper [2] we present simulation results for Phase 1 performance evaluation with advanced receivers – E-MMSE-IRC and R-ML. We observe that the performance with E-MMSE-IRC is not significantly better than baseline MMSE-IRC receiver across all the cases considered. Performance with R-ML receiver is significantly better than MMSE-IRC especially when the modulation order of co-scheduled UE is smaller than the target UE. The performance evaluation was with genie knowledge of all the parameters for co-UE needed for R-ML.

- Large gains with R-ML are observed when modulation order of co-UE is smaller than target UE.

- Performance evaluation results are based on genie knowledge of co-UE’s parameters.

In RAN4#107 RAN4 agreed to introduce DCI based assistance information for modulation order signaling and UE assistance in modulation order blind detection and sent LS to RAN1 [3]. Blind detection of co-UE parameters significantly increases UE complexity, especially if no assistance information is provided for modulation order of co-UE.  With the assumption that request from RAN4 will be honored and the suggested DCI signaling can be introduced, then R-ML receiver can be selected as the reference receiver for phase 2 for requirements definition.

- RAN4 agreed DCI based signalling for NWA on modulation order of co-scheduled UE.

- R-ML receiver with blind detection on co-UE modulation order increases complexity.

- With the assumption that DCI based signaling for NWA on modulation order is agreed, RAN4 can select R-ML as reference receiver for requirements definition in Phase 2.

To limit the UE processing and complexity, the R-ML receiver for MU-MIMO should be limited to up to 4 MIMO layers of target and co-UE and for DMRS type1 configuration with length 1.

- Define R-ML receiver for maximum 4 layers across target and co-UE, with DMRS configuration type 1 with length 1.

## Required Information for Advanced Receivers

Other required information of the co-scheduled UE for both R-ML and E-IRC

In RAN4#107, the following was discussed and agreed :

| Information | RAN4 Default assumption (If N/A, how could be obtained by the UE) | Signalling if RAN4 default assumption not valid | Way forward on the signalling details if introduced |
| --- | --- | --- | --- |
| The DMRS port information for the co-scheduled UE | N/A (Obtained by UE blind detection) | N/A | FFS whether additional RRC based assistant signalling can be considered. |
| PRB bundling size for the co-scheduled UE Frequency domain resource allocation for the co-UE within each PRG of the target UE | UE assume in each its PRG, the resource allocation and precoding of the potential DMRS sequence aligned co-scheduled UE(s) in other DM-RS ports of different CDM group are aligned with PRG=2 or 4. | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| DMRS power boosting for the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| Time domain resource allocation information of the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not | FFS separate UE capability corresponding the dedicated RRC signaling needed or not |
| Frequency domain resource allocation for the co-UE across different PRGs of the target UE: | N/A (Obtained by UE blind detection) | N/A | No signalling on frequency domain resource allocation information. |
| CSI-RS location of co-scheduled UE (Only required for R-ML) | UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE | Down-select to one of the below options in the next meeting: Option 1: No RRC signalling is needed Option 2: 1-bit RRC signaling |  |

FDRA

It was agreed that UE obtains the FDRA of co-UE(s) by blind detection. DCI based indication of FDRA of co-UE is not practical. The resource allocation type can be type 0, type 1 or dynamic.   The UE needs to do blind detection on FDRA and the DMRS ports of co-UEs. It would be helpful to introduce RRC based signaling to indicate to the UE the resource allocation type of the co-UE(s). If we go with the default assumption that the resource allocation type is the same as target UE, at a minimum if signaling can indicate if the default assumption is not valid, it would help with the blind detection of DMRS ports and FDRA.

- Knowledge of PDSCH resource allocation type of co-UE can assist with blind detection of DMRS ports and FDRA .

- Default assumption on PDSCH resource allocation type of co-UEs same as target UE

- Introduce 1-bit RRC signaling to indicate if default assumption on PDSCH resource allocation type is not valid.

CSI-RS location of co-scheduled UE

For CSI-RS location of co-scheduled UE, the default assumption is that the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE. In case this doesn’t hold, it should be indicated by RRC signaling

- Introduce 1-bit RRC signaling to indicate if default assumption on CSI-RS location of co-scheduled UE is not valid.

The modulation order information of the co-scheduled UE (Only required for R-ML)

In some cases the DCI based NWA for modulation order will not be able to provide the modulation order of the co-UEs and UE if capable might choose to do blind detection of modulation order of co-scheduled layers. RRC signaling to indicate the MCS table or maximum modulation order of the co-UEs would assist UE in modulation order blind detection, if UE is capable of blind detection of modulation order. A 2 bit RRC signaling can be introduced with the following mapping for maximum modulation order:

00: 64QAM

01: 256QAM

10: 1024QAM

- Introduce 2-bit RRC signaling to indicate MCS table or maximum modulation order of co-UEs.

In case DCI based NWA for modulation order is not agreed in RAN1, RAN4 should discuss if MAC Ce based signalling can be used.

- In case DCI based NWA for modulation order is not agreed in RAN1, RAN4 should further discuss possibility of indicating modulation order NWA via MAC-CE.

In summary the following need to be indicated by RRC signaling:

- 2-bit signaling for maximum modulation order

- 1 bit to indicate if default assumption on precoding granularity is not valid

- 1 bit to indicate if default assumption on DMRS power boosting for the co-scheduled UE

- 1 bit to indicate if default assumption on Time domain resource allocation information of the co-scheduled UE is not valid

- 1 bit to indicate if default assumption on CSI-RS location of the co-scheduled UE is not valid

- 1-bit RRC signaling if default assumption on PDSCH resource allocation type is not valid.

- Introduce the following RRC signaling:
-      2-bit signaling for maximum modulation order

- 1 bit to indicate if default assumption on precoding granularity is not valid

- 1 bit to indicate if default assumption on DMRS power boosting for the co-scheduled UE

- 1 bit to indicate if default assumption on Time domain resource allocation information of the co-scheduled UE is not valid

- 1 bit to indicate if default assumption on CSI-RS location of the co-scheduled UE is not valid

- 1-bit RRC signaling if default assumption on PDSCH resource allocation type is not valid.

## UE Capability

If R-ML is agreed as the reference receiver for R18 MU-MIMO, we propose to introduce the following UE capabilities:

- UE supporting R-ML without modulation order blind detection of co-scheduled layers

- UE supporting R-ML with modulation order blind detection

- Maximum number of layers of co-UE or total number of layers for joint detection

- UE capability on maximum number of DMRS ports for blind detection

- Introduce the following UE capabilities:
-      UE supporting R-ML without modulation order blind detection of co-scheduled layers 
-      UE supporting R-ML with modulation order blind detection 
-      Maximum number of layers of co-UE or total number of layers for joint detection
-      UE capability on maximum number of DMRS ports for blind detection

- Send LS to RAN2 on RRC signaling for NWA and UE capabilities

# 3. Conclusion

In this paper, we provide our views on open issues on on receiver assumptions for advanced receiver considered for mitigating inter- user interference in MU-MIMO. Our observations and proposals are captured below:

Reference Receiver

- Large gains with R-ML are observed when modulation order of co-UE is smaller than target UE.

- Performance evaluation results are based on genie knowledge of co-UE’s parameters.

- RAN4 agreed DCI based signalling for NWA on modulation order of co-scheduled UE.

- R-ML receiver with blind detection on co-UE modulation order increases complexity.

- With the assumption that DCI based signaling for NWA on modulation order is agreed, RAN4 can select R-ML as reference receiver for requirements definition in Phase 2.

- Define R-ML receiver for maximum 4 layers across target and co-UE, with DMRS configuration type 1 with length 1.

Information required for E-MMSE-IRC and R-ML

- Knowledge of PDSCH resource allocation type of co-UE can assist with blind detection of DMRS ports and FDRA .

- Default assumption on PDSCH resource allocation type of co-UEs same as target UE

- Introduce 1-bit RRC signaling to indicate if default assumption on PDSCH resource allocation type is not valid.

- Introduce 1-bit RRC signaling to indicate if default assumption on CSI-RS location of co-scheduled UE is not valid.

- Introduce 2-bit RRC signaling to indicate MCS table or maximum modulation order of co-UEs.

- In case DCI based NWA for modulation order is not agreed in RAN1, RAN4 should further discuss possibility of indicating modulation order NWA via MAC-CE.

- Introduce the following RRC signaling:
-      2-bit signaling for maximum modulation order

- 1 bit to indicate if default assumption on precoding granularity is not valid

- 1 bit to indicate if default assumption on DMRS power boosting for the co-scheduled UE

- 1 bit to indicate if default assumption on Time domain resource allocation information of the co-scheduled UE is not valid

- 1 bit to indicate if default assumption on CSI-RS location of the co-scheduled UE is not valid

- 1-bit RRC signaling if default assumption on PDSCH resource allocation type is not valid.

UE capability

- Introduce the following UE capabilities:
-      UE supporting R-ML without modulation order blind detection of co-scheduled layers 
-      UE supporting R-ML with modulation order blind detection 
-      Maximum number of layers of co-UE or total number of layers for joint detection
-      UE capability on maximum number of DMRS ports for blind detection

- Send LS to RAN2 on RRC signaling for NWA and UE capabilities

# Reference

- R4-2309892, “WF on advanced receiver for MU-MIMO scenario”, China Telecom.

- R4-2311353, “On test parameters and simulation results for MU-MIMO”, Apple.

- R4-2309895, “LS on required DCI signalling for advanced receiver on MU-MIMO scenario”, RAN4