3GPP TSG-RAN WG4 Meeting #108									              R4-2311363

Toulouse, France, August 21 – 25, 2023

Agenda item:	8.9.2.1

Source:	Apple

Title:	Discussion on scope and general issues of R18 gap enhancement

Document for:	Discussion

# Introduction

The scope and general issues of R18 gap enhancement was discussed during the previous RAN4 and RANP meetings. The last agreements can be found in [1], duplicated here for information:

## Sub-topic 2-1: Scope and general issues ### Issue 2-1-1: Which Type of MG is considered together with Pre-MG/NCSG in the WI? < Way forward >: - Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI. - Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR. ### Issue 2-1-2: If Type-1 MG is considered in this WI, which RRM requirements shall be applied? < Agreement from [R4-2306330]>: - If Type-1 MG is considered in this WI, similar to that of Rel-17 Concurrent MG WI, “no requirements” could be applied for the scenario when Type-1 MG is colliding with Pre-MG/ NCSG in the same FR.

In this contribution, we continue discussing the open issue.

# Discussion

The remaining issue is whether Type-1 MG can be considered with Pre-MG/NCSG in the WI. Take type-1 MG in case 1 for example, i.e. Gap(s) configured via GapConfig without suffix plus Pre-MG, when the Pre-MG is activated, the configuration is effectively same as ‘concurrent gap + legacy gap’. Note that this issue has already been discussed in R17 in RAN4#101-bis-e. RAN4 agreed in the approved LS to RAN2.

Q1 – Can Rel-17 concurrent gaps be configured together with legacy gap? If ‘yes’, what would be the UE behavior?

- RAN4 response: From RAN4 requirement perspective, RAN4 would like to ensure that the association of frequency layers or dedicated use cases to measurement gaps shall be clearly understood by both UE and Network for all configured measurements. How the association is up to RAN2.

Since network can configure Pre-MG or NCSG and association between MO and gap, network shall be able to provide configuration of type-2 MG as well. In other word, the network has been upgraded to R17 (at least for this feature). Therefore, we don’t see too much value for network to configure type-1 MG in this case. On the other hand, existing concurrent gaps requirements in R17 only applies when each frequency layer is only associated with one concurrent measurement gap. Besides, implicit association is being discussed in case 1 requirements. It would be more challenging for UE and network to align understanding on which MO shall be measured within each MG occasion if both implicit association and type-1 MG are considered.

From requirements perspective, the following two options are quite similar:

- Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI.

- Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR.

RAN4 doesn’t need to define requirements for neither of them.

Observation 1: option 1 and 2a are quite similar from RAN4 requirement point of view.

- Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI.

- Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR.

Proposal 1: not define RRM requirements when type 1 MG is colliding with pre-MG or NCSG in the same FR.

# Conclusion

In this contribution, we provide discussion on scope and general issues of R18 gap enhancement. After discussion, the following conclusions are provided:

Observation 1: option 1 and 2a are quite similar from RAN4 requirement point of view.

- Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI.

- Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR.

Proposal 1: not define RRM requirements when type 1 MG is colliding with pre-MG or NCSG in the same FR.

# References

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation