3GPP TSG-RAN WG4 Meeting #108									              R4-2311373

Toulouse, France, August 21 – 25, 2023

Agenda item:	8.10.2.4

Source:	Apple

Title:	Discussion on RRM core requirements for option B-1-2

Document for:	Discussion

# Introduction

RRM core requirement for option B-1-2 was extensively discussed in previous RAN4 meetings. The latest agreements can be found in [1]. However, there are still some remaining issues. Besides, in the last RANP#100, it was agreed to include L3 intra-frequency measurement without gaps for option B-1-1 and option C, and there is a note to further check option B-1-2

2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2

In this contribution, we provide our view on remaining issues and L3 measurement related impact for option B-1-2.

# Discussion

- Remaining issues to support option B-1-2

#### Issue 5-2: Interruption requirements for supporting option B-1-2

<Agreement >:

- Define the following interruption requirements for Option B-1-2

  - Interruption length

    - Option 1: [0.5ms] for FR1 and [0.25ms] for FR2

  - FFS: Interruption ratio

  - FFS: Interruption location

Regarding detailed interruption requirements, we see two alternatives:

- Define explicit interruption length without mentioning interruption ratio.

- Define interruption ratio in a general way.

For alterative 1), typical example could be 0.5ms in FR1 and 0.25ms in FR2. Note that how frequently UE shall perform L1 operation is configurable from network point of view. Once the configuration of L1-RS, SMTC and measurement gap is provided, the L1 operation interval is determined, at least from RAN4 requirement point of view network shall know the interruption ratio. Therefore, it is feasible to only define explicit interruption length without mentioning interruption ratio.

Alternatively, RAN4 can also clearly specify interruption in the format of equation so that it can cover all the configurations. For instance, the ratio can be calculated by X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

Proposal 1: the following options can be considered to define interruption for option B-1-2:

- Option 1: define explicit interruption length without mentioning interruption ratio. (preferred)

- Option 2: X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

- L3 measurement

We don’t support additional L3 measurement without gap for UE supporting option B-1-2. The reason is 1) gap based L3 measurement can still work. 2) performing L3 measurement outside active BWP would result in additional interruption, especially when UE is configured with MG for other inter-frequency measurement, which is quite typical to our understanding.

Observation 1: performing L3 measurement outside active BWP would result in additional interruption, especially when UE is configured with MG for other inter-frequency measurement, which is quite typical.

Proposal 2: L3 measurement without gap is NOT considered for option B-1-2.

# Conclusion

In this contribution, we provide discussion on RRM core requirement for option B-1-2. After discussion, the following conclusions are provided:

Proposal 1: the following options can be considered to define interruption for option B-1-2:

- Option 1: define explicit interruption length without mentioning interruption ratio. (preferred)

- Option 2: X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

Observation 1: performing L3 measurement outside active BWP would result in additional interruption, especially when UE is configured with MG for other inter-frequency measurement, which is quite typical.

Proposal 2: L3 measurement without gap is NOT considered for option B-1-2.

# References

- R4-2310176, WF on NR BWP operation without restriction, vivo

- RP-231486, Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo