3GPP TSG-RAN WG4 Meeting #108									              R4-2311381

Toulouse, France, August 21 – 25, 2023

Agenda item:	8.24.5

Source:	Apple

Title:	Discussion on Enhanced CHO configurations

Document for:	Discussion

# Introduction

Enhanced CHO configurations have been discussed widely for several meetings. The latest agreement can be found in the approved WF [1]. There are still quite many remaining issues that need to be addressed. In this contribution, we provide further discussion on the open issues.

# Discussion

### 2.1 CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

Issue 3-2-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: (CATT, ZTE)

    - consider DCHOwithCPAC = Max [DCHO,  DCPAC] as baseline. TEvent_DU needs to be updated.

      - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

      - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

      - TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change

  - Option 1a: (Ericsson)

    - DCHOwithCPAC = Max [DCHO,  DCPAC].

      - DCHO = TRRC + TEvent_DU + Tcell_index_identify + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - DCPAC = TRRC_delay + TEvent_DU + Tcell_index_identify + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow legacy except from update of TEvent_DU and  Tcell_index_identify

  - Option 2: (CMCC)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution if

      - CPC/CPA and CHO conditions are satisfied simultaneously

      - CPC/CPA condition is fulfilled firstly

  - Option 3: (MTK, Apple, HW, QC)

    - similar as obj.3, i.e., DCHOwithPCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

      - Tprocessing = 25 ms

      - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

  - Option 4: (vivo)

    - if CHO and CPC conditions are met simultaneously or CPC conditions is met firstly and CHO condition is met later, DCHOwithCPC_PCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + Tinterrupt + TCHO_execution

According to RAN2 agreement, UE needs to wait until both CHO and CPC conditions are met. When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions:

Agreement from RAN2#122: P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions. For CHO+CPC we only consider execution when BOTH conditions are met. - (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

However, it is not crystal clear to us whether network will always provide CHO-only or R17 CHO with SCG configuration, when provide CHO with candidate CPA/CPC. RAN4 requirements would be different for these two cases.

- If CHO-only or R17 CHO with SCG configuration is NOT provided (depends on RAN2)

For PCell latancy, it is almost same as that agreed in obj.3 with exception that TEvent_DU + Tmeasure need to be extended to the later one between PCell and PSCell. Therefore, we propose the same equation from obj.3 with the following modification highlighted in yellow:

Proposal 1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is: (based on obj.3 with yellow highlight)

DCHOwithCPAC_PCell = TRRC + TEvent_DU + max(Tmeasure_CHO, Tmeasure_CPA/CPC) + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

Where all components are same as that defined in obj.3 except:

- Tprocessing = 25 ms

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command including target MCG and candidate SCG for CPC/CPA until a condition exists at the measurement reference point which will trigger the CHO and CPC/CPA.

- Tmeasure_CHO is same as Tmeasure for CHO

- Tmeasure_CPA/CPC is same as Tmeasure for CPA/CPC

Similarly, PSCell latency can also be derived based that defined in obj.3 with some modification:

Proposal 2: CPA/CPC delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is: (based on obj.3 with yellow highlight)

DCHOwithPSCell_PSCell = TRRC + TEvent_DU + max(Tmeasure_CHO, Tmeasure_CPA/CPC) + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms,

Where all components are same as that defined in obj.3 except:

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command including target MCG and candidate SCG for CPC/CPA until a condition exists at the measurement reference point which will trigger the CHO and CPC/CPA.

- Tmeasure_CHO is same as Tmeasure for CHO

- Tmeasure_CPA/CPC is same as Tmeasure for CPA/CPC

- If CHO-only or R17 CHO with SCG configuration is provided

Then PCell latency is

- Same as that defined in legacy conditional handover requirement defined in TS38.133 section 6.1.5, if CHO-only is provided.

- Same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

PSCell latency is same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

Proposal 3: PCell latency is same as that defined in legacy conditional handover requirement defined in TS38.133 section 6.1.5, if CHO-only is provided.

Proposal 4: PCell latency is same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

Proposal 5: PSCell latency is same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

Other thing RAN4 needs to discuss is about FR1-FR1 DC based on agreement made in RAN4#106-bis-e [2]:

Issue 3-1-2: frequency range

- Agreements:

  - Both FR1+FR2 and FR1+FR1 NR-DC are in scope of RRM requirements for enhanced CHO configurations.

  - RAN4 shall start from FR1+FR2 NR-DC. Discussion on FR1+FR1 NR-DC will start from RAN4#108.

It is our understanding that difference between FR1+FR2 DC and FR1+FR1 DC is only about Tprocessing from RAN4 requirement perspective. We propose to reuse Tprocessing in handover with PSCell in FR1+FR1 DC from eFeRRM work item.

Observation 1: from RAN4 requirement perspective, Tprocessing is the only difference between FR1+FR2 DC and FR1+FR1 DC.

Proposal 6: reuse Tprocessing in handover with PSCell in FR1+FR1 DC from eFeRRM.

# Conclusion

In this contribution, we provide further discussion on improvement on enhanced CHO configurations. After discussion, the following conclusions are provided:

- If CHO-only or R17 CHO with SCG configuration is NOT provided (depends on RAN2)

Proposal 1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is: (based on obj.3 with yellow highlight)

DCHOwithCPAC_PCell = TRRC + TEvent_DU + max(Tmeasure_CHO, Tmeasure_CPA/CPC) + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

Where all components are same as that defined in obj.3 except:

- Tprocessing = 25 ms

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command including target MCG and candidate SCG for CPC/CPA until a condition exists at the measurement reference point which will trigger the CHO and CPC/CPA.

- Tmeasure_CHO is same as Tmeasure for CHO

- Tmeasure_CPA/CPC is same as Tmeasure for CPA/CPC

Proposal 2: CPA/CPC delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is: (based on obj.3 with yellow highlight)

DCHOwithPSCell_PSCell = TRRC + TEvent_DU + max(Tmeasure_CHO, Tmeasure_CPA/CPC) + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms,

Where all components are same as that defined in obj.3 except:

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command including target MCG and candidate SCG for CPC/CPA until a condition exists at the measurement reference point which will trigger the CHO and CPC/CPA.

- Tmeasure_CHO is same as Tmeasure for CHO

- Tmeasure_CPA/CPC is same as Tmeasure for CPA/CPC

- If CHO-only or R17 CHO with SCG configuration is provided

Proposal 3: PCell latency is same as that defined in legacy conditional handover requirement defined in TS38.133 section 6.1.5, if CHO-only is provided.

Proposal 4: PCell latency is same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

Proposal 5: PSCell latency is same as that agreed in objective 3, if R17 CHO with SCG configuration if provided.

Proposal 6: reuse Tprocessing in handover with PSCell in FR1+FR1 DC from eFeRRM.

Observation 1: from RAN4 requirement perspective, Tprocessing is the only difference between FR1+FR2 DC and FR1+FR1 DC.

# References

- R4-2310064, WF on NR Mobility Enhancements RRM requirements (part 2), Apple.

- R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple