3GPP TSG-RAN WG4 Meeting #108									              R4-2311385

Toulouse, France, August 21 – 25, 2023

Agenda item:	8.25.2.4

Source:	Apple

Title:	Discussion on network B requirements of R18 MUSIM

Document for:	Discussion

# Introduction

Network B requirements of R18 MUSIM have been widely discussed in previous RAN4 meetings. However, there are still some open issues. In this contribution, we provide our view on these issues.

# Discussion

### 2.1 Sub-topic 4-1

Issue 4-1-1: Network B requirements conditions

- Proposals

  - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (vivo CMCC Ericsson Huawei Nokia)

    - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Ericsson Nokia)

  - P2: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements. (vivo)

  - P3: Do not add a condition stating that NW-B requirements only apply if “MUSIM gaps will not collide with other MUSIM gaps (Nokia)

  - P4: Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Agree to continue discussion other conditions during or once NW B requirements are agreed. (Nokia)

  - P5: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm)

In general, we support P1, P1-1 and P2.

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements.

Issue 4-1-2: Network B requirements framework

- Proposals

  - P1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline (vivo CMCC Apple xiaomi ZTE Huawei Qualcomm Ericsson)

    - P2-1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (vivo Apple xiaomi Huawei Qualcomm)

      - P2-1a: The value set of max(DRX cycle, MGRP_max) is {0.32, 0.64, 1.28, 2.56 5.12} (vivo)

    - P2-2: DRX cycle is replaced by max(DRX cycle, MGRP); MGRP is the MGRP of the periodic MUSIM gap with highest priority level (CMCC)

    - P2-3: The NW-B’s requirement should decouple with MUSIM gaps requested by UE.  RAN4 to introduce a relaxed NW-B’s IDLE mode requirement as follow. (Ericsson)

Table 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for NW-B

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 36 x N1 x M2 x R1 | 4 x N1 x M2 x R1 | 16 x N1 x M2 x R1 |
| 0.64 |  | 5 | 8 | 28 x N1 x R1 | 2 x N1 x R1 | 8 x N1 x R1 |
| 1.28 |  | 4 | 6 | 25 x N1 x R1 | 1 x N1 x R1 | 5 x N1 x R1 |
| 2.56 |  | 3 | 5 | 23 x N1 x R1 | 1 x N1 x R1 | 3 x N1 x R1 |
| Note: R1 = 2. |  |  |  |  |  |  |

\

  - P3: P2-1 and P2-2 is not reasonable. The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. (Nokia)

Technically, NR-U framework can also be reused here with similar justification provided in our contribution regarding network A requirement.

One difference here is that for cell reselection in NW B, longer MGRP can be expected and it is not expected to be dropped due to collision with NW A gap, otherwise all the MUSIM gaps would be dropped. Therefore, it makes sense to us that RAN4 can assume no MUSIM gap is dropped in NW B requirements. With this assumption, we are also fine with P1 and P2-1.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline, With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Issue 4-1-3: Requirement when MGRP = 5.12s

- Proposals

  - P1: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined. (vivo CMCC Apple Huawei Qualcomm)

    - P1-1: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s. (vivo Huawei)

  - P2: Not to define NW B measurement/cell reselection requirements in IDLE mode when MUSIM gap’s MGRP=5.12s (MTK)

For completion, we support P1 to introduce requirement for 5.12s MGRP. P1-1 can also be considered since this is the last meeting for core part design.

Proposal 5: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 6: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

# Conclusion

In this contribution, we provide discussion on network B requirements of R18 MUSIM. After discussion, the following conclusions are provided:

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline, With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Proposal 5: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 6: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

# References

- R4-2310165, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo