3GPP TSG-RAN WG4 Meeting #108									              R4-2311392

Toulouse, France, August 21 – 25, 2023

Agenda item:	10.3.4

Source:	Apple

Title:	Discussion on RAN5 LS on RRM test cases with testability issues

Document for:	Discussion

# Introduction

In RAN5#99 an LS [1] was approved to RAN4 on RRM test cases with testability issues:

RAN5 discussed the testability issues affecting LTE/FR1 + FR2 test cases caused by uncalibrated OTA link. The observations are in R5-233696 which is provided as an attachment to this LS. RAN5 would like to bring to RAN4 attention of the extended list of test cases having testability issues. If RAN4 agrees with the RAN5 assessment regarding the list of test cases affected by OTA testability issues, RAN5 kindly requests RAN4 to consider adding the mentioned test cases to the list of tests in TS 38.133 v17.9.0 clause A.3.13A to avoid maintaining 2 separate lists. The list of test cases to be included is also provided in the appendix below.

In this contribution, we discuss the questions asked by RAN5 and provide potential response to their LS.

# Discussion

Criteria for selecting FR1/LTE+FR2 test with OTA testability problem was agreed in RAN4:

## 4.1 Criteria for selecting FR1/LTE+FR2 test with OTA testability problem - FR1/LTE+FR2 test has OTA testability problem if at least one of the following criteria is met: - Tests where any requirement is tested for FR1/LTE, - Tests where UE receives any DL message (e.g. RRC/DCI/MAC-CE configuration message/command etc) on FR1/LTE between the starting point and ending point of the test, and - Tests where UE transmits any UL signal (e.g. measurement report, ACK/NACK, CSI etc) b on FR1/LTE between the starting point and ending point of the test.

Based on this we checked the testability issues identified in RAN5 [2] and provide our comments (highlighted in yellow) in the following table:

Table 2-1: Initial list of TS 38.533 [2] section 5 test cases affected by testability issue caused by uncalibrated LTE/FR1 link [2]

| Test | Test Description | Test Scenario | Testability Issue # | Testability Issue | Comment on RAN5 observation |
| --- | --- | --- | --- | --- | --- |
| A.5.3.3.1 | Handover with PSCell with known FR2 target PSCell | FR1+FR2 | 2 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE receives PSCell change command via uncalibrated LTE link and this marks the start time for the latency check | Valid |
| A.5.5.3.3 | PUCCH SCell Activation and deactivation of known SCell in FR2 | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots n and m, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *LTE PCell/NR PSCell interruption is checked by fully scheduling these uncalibrated cells and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A5.5.3.4 | PUCCH SCell Activation and deactivation of unknown SCell in FR2 | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots n and m, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *LTE PCell/NR PSCell interruption is checked by fully scheduling these uncalibrated cells and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.5.5.3.5 | Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2 | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *LTE PCell/NR PSCell interruption is checked by fully scheduling these uncalibrated cells and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.5.5.3.6 | SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX | FR1+FR2 LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.5.5.3.2 | SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle | FR1+FR2 | 1 | *FR1 cell not guaranteed to be detectable, this can lead to SCell add failure, or inability to report CQI for the SCell | Valid |
| A.5.5.3.5 | SCell Activation and deactivation of SCell in FR2 | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *LTE PCell/NR PSCell interruption is checked by fully scheduling these uncalibrated cells and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slot m, based on which the rest of the timeline check depends *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.5.5.6.1.2 | E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC | LTE+FR2 | 1, 2, 3 | *LTE PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) | Valid |
| A.5.5.6.4.2 | E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *LTE PCell/NR PSCell interruption is checked by fully scheduling these uncalibrated cells and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to receive DCI format 2_6 via uncalibrated FR1 link and this marks slot i and j, based on which the rest of the timeline checks depend | Valid Issue 1 also exists |
| A.5.6.2.5 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used | LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE required to receive RRC config with meas object via uncalibrated LTE *UE required to transmit MR over uncalibrated LTE | Valid MO config can be sent sent via FR1 PSCell MR can be sent via FR1 PSCell |
| A.5.6.2.6 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used | LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE required to receive RRC config with meas object via uncalibrated LTE *UE required to transmit MR over uncalibrated LTE | Valid MO config can be sent sent via FR1 PSCell MR can be sent via FR1 PSCell |
| A.5.6.2.7 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used | LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE required to receive RRC config with meas object via uncalibrated LTE *UE required to transmit MR over uncalibrated LTE | Valid MO config can be sent sent via FR1 PSCell MR can be sent via FR1 PSCell |
| A.5.6.2.8 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used | LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE required to receive RRC config with meas object via uncalibrated LTE *UE required to transmit MR over uncalibrated LTE | Valid MO config can be sent sent via FR1 PSCell MR can be sent via FR1 PSCell |
| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell | LTE+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so test can end early in SCG add failure *UE required to receive RRC config with meas object via uncalibrated LTE *UE required to transmit MR over uncalibrated LTE | Valid MO config can be sent sent via FR1 PSCell MR can be sent via FR1 PSCell |

Table 2-2: Initial list of TS 38.533 [2] section 7 test cases affected by testability issue caused by uncalibrated LTE/FR1 link [2]

| Test | Test Description | Test Scenario | Testability Issue # | Testability Issue | Comment on RAN5 observation |
| --- | --- | --- | --- | --- | --- |
| A.7.3.1.1 | Inter-frequency handover from FR1 to FR2; unknown target cell | FR1+FR2 | 1, 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (HO cmd) via uncalibrated FR1 *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 | Valid |
| A.7.3.1.4 | Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 | FR1+FR2 | 1, 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (HO cmd) via uncalibrated FR1 *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 | Valid |
| A.7.3.1.5 | Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 | FR1+FR2 | 1, 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (HO cmd) via uncalibrated FR1 *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 | Valid |
| A.7.3.1.6 | Handover with PSCell from SA to EN-DC; unknown FR2 target cell | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (in multiple occasions) via uncalibrated FR1 *UE required to trigger event B2, which requires evaluation of uncalibrated FR1 cell vs threshold1 and evaluation of uncalibrated LTE cell vs threshold2 *UE required to transmit MR over uncalibrated FR1 link *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 *UE required to RACH on target LTE PCell | Valid |
| A.7.3.1.7 | HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell | FR1+FR2 LTE+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (in multiple occasions) via uncalibrated FR1 *UE required to trigger event B2, which requires evaluation of uncalibrated FR1 cell vs threshold1 and evaluation of uncalibrated LTE cell vs threshold2 *UE required to transmit MR over uncalibrated FR1 link *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 *UE required to RACH on target LTE PCell | Valid |
| A.7.3.1.8 | NR PSCell change delay in HO with PSCell from NR-DC to NR-DC | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (HO cmd) via uncalibrated FR1 *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 *UE required to RACH on target FR1 PCell | Valid |
| A.7.3.1.11 | Inter-frequency handover from FR1 to FR2-2; unknown target cell | FR1+FR2-2 | 1, 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (HO cmd) via uncalibrated FR1 *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 | Valid |
| A.7.5.3.2 | SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.7.5.3.6 | PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (PUCCH SCell config) via uncalibrated FR1 *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.7.5.3.7 | PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 Pcell | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (SCell config) via uncalibrated FR1 *UE is requested to receive MAC-CE via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.7.5.3.12 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive multiple RRC messages via uncalibrated FR1 link and this marks slot m, based on which the rest of the timeline check depends *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) | Valid |
| A.7.5.3.15 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive multiple RRC messages via uncalibrated FR1 link and this marks slots m and n, based on which the rest of the timeline check depends (for both activation/deactivation, correspondingly) *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) | Valid |
| A.7.5.6.1.2 | NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive DCI_1_1 (BWP swtich) via uncalibrated FR1 and this marks slot i, based on which the rest of the timeline check depends *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) | Valid DCI should be sent via FR2 link. |
| A.7.5.6.4.2 | NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time | FR1+FR2 | 1, 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE is requested to receive DCI_2_6 on uncalibrated FR1 cell, containing SCell dormancy indication for Cells 2 and 3 *FR1 PCell interruption is checked by fully scheduling this uncalibrated cell and then counting DTX (over PUCCH and/or PUSCH) *UE is requested to transmit CSI report for Cells 2 and 3, over the uncalibrated FR1 cell | Valid |
| A.7.5.7.1 | Addition and Release Delay of known NR PSCell in FR2-2 | FR1+FR2-2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.5.7.2 | Addition and Release Delay of unknown NR PSCell in FR2-2 | FR1+FR2-2 | 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 | Valid |
| A.7.5.12.1 | Addition and Release Delay of PSCell | FR1+FR2 | 2 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 | Valid |
| A.7.6.2.5 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.6 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.7 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.8 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.16 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.17 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.18 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.2.19 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1) | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |
| A.7.6.3.6 | Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE is requested to transmit CSI report on uncalibrated FR1 cell, which are checked | Valid |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell | FR1+FR2 | 2, 3 | *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config via uncalibrated FR1 *UE required to transmit MR over uncalibrated FR1 link | Valid |

In summary, most comments from RAN5 are valid and all mentioned tests shall be skipped.

Proposal 1: RAN4 shall update the test cases UE does not have to pass in current version of specification with yellow highlight part with track changes on top of recommendation from RAN5 (based on TS38.133-ha0).

Table A.3.13A.2-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.5.5.2.7 | E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching |
| A.5.5.3.2 | SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle |
| A.5.5.3.5 | SCell Activation and deactivation of SCell in FR2 |
| A.5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| A.5.5.6.4.2 | E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time |
| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| A.5.3.3.1 | Handover with PSCell with known FR2 target PSCell |
| A.5.5.3.9 | PUCCH SCell Activation and deactivation of known SCell in FR2 |
| A.5.5.3.10 | PUCCH SCell Activation and deactivation of unknown SCell in FR2 |
| A.5.5.3.11 | Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2 |
| A.5.5.3.12 | SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX |
| A.5.6.2.5 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used |
| A.5.6.2.6 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used |
| A.5.6.2.7 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used |
| A.5.6.2.8 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used |

Table A.3.13A.3-1: Test cases UE does not have to pass in current version of specification (SA)

| Clause | Test case slogan |
| --- | --- |
| A.7.5.3.2 | SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 |
| A.7.5.3.7 | PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell |
| A.7.5.6.1.2 | NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA |
| A.7.5.6.4.2 | NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time |
| A.7.6.2.5 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.6 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.7 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.8 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| A.7.3.1.1 | Inter-frequency handover from FR1 to FR2; unknown target cell |
| A.7.3.1.4 | Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 |
| A.7.3.1.5 | Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 |
| A.7.3.1.6 | Handover with PSCell from SA to EN-DC; unknown FR2 target cell |
| A.7.3.1.7 | HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell |
| A.7.3.1.8 | NR PSCell change delay in HO with PSCell from NR-DC to NR-DC |
| A.7.3.1.11 | Inter-frequency handover from FR1 to FR2-2; unknown target cell |
| A.7.5.3.6 | PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known |
| A.7.5.3.12 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| A.7.5.3.15 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| A.7.5.7.1 | Addition and Release Delay of known NR PSCell in FR2-2 |
| A.7.5.7.2 | Addition and Release Delay of unknown NR PSCell in FR2-2 |
| A.7.5.12.1 | Addition and Release Delay of PSCell |
| A.7.6.2.16 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.17 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.18 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.19 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.3.6 | Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |

Note that rows from A.7.5.12.1 to A.7.6.3.6 should also be highlighted in yellow.

# Conclusion

In this contribution, we provide discussion on RAN5 LS on RRM test cases with testability issues. After discussion, the following conclusions are provided:

Proposal 1: RAN4 shall update the test cases UE does not have to pass in current version of specification with yellow highlight part with track changes on top of recommendation from RAN5 (based on TS38.133-ha0).

Table A.3.13A.2-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.5.5.2.7 | E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching |
| A.5.5.3.2 | SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle |
| A.5.5.3.5 | SCell Activation and deactivation of SCell in FR2 |
| A.5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| A.5.5.6.4.2 | E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time |
| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| A.5.3.3.1 | Handover with PSCell with known FR2 target PSCell |
| A.5.5.3.9 | PUCCH SCell Activation and deactivation of known SCell in FR2 |
| A.5.5.3.10 | PUCCH SCell Activation and deactivation of unknown SCell in FR2 |
| A.5.5.3.11 | Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2 |
| A.5.5.3.12 | SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX |
| A.5.6.2.5 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used |
| A.5.6.2.6 | EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used |
| A.5.6.2.7 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used |
| A.5.6.2.8 | EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used |

Table A.3.13A.3-1: Test cases UE does not have to pass in current version of specification (SA)

| Clause | Test case slogan |
| --- | --- |
| A.7.5.3.2 | SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 |
| A.7.5.3.7 | PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell |
| A.7.5.6.1.2 | NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA |
| A.7.5.6.4.2 | NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time |
| A.7.6.2.5 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.6 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.7 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.8 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| A.7.3.1.1 | Inter-frequency handover from FR1 to FR2; unknown target cell |
| A.7.3.1.4 | Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 |
| A.7.3.1.5 | Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 |
| A.7.3.1.6 | Handover with PSCell from SA to EN-DC; unknown FR2 target cell |
| A.7.3.1.7 | HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell |
| A.7.3.1.8 | NR PSCell change delay in HO with PSCell from NR-DC to NR-DC |
| A.7.3.1.11 | Inter-frequency handover from FR1 to FR2-2; unknown target cell |
| A.7.5.3.6 | PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known |
| A.7.5.3.12 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| A.7.5.3.15 | PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| A.7.5.7.1 | Addition and Release Delay of known NR PSCell in FR2-2 |
| A.7.5.7.2 | Addition and Release Delay of unknown NR PSCell in FR2-2 |
| A.7.5.12.1 | Addition and Release Delay of PSCell |
| A.7.6.2.16 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.17 | SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.18 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.19 | SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.3.6 | Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |

Note that rows from A.7.5.12.1 to A.7.6.3.6 should also be highlighted in yellow.

# References

- R4-2311012, LS on RRM test cases with testability issues, RAN5

- R5-233696, Discussion on affected list of RRM test cases with testability issues, QC