3GPP TSG-RAN WG4 Meeting # 108	R4-2311411

Toulouse, France, Aug. 21 – Aug. 25, 2023

Agenda Item: 8.30.3.1

Source: MediaTek Inc.

Title: Discussion on RRM core requirements for R18 NR SL CA

Document for: Discussion

- Introduction

RAN4#107 had some discussion on the RRM requirements for NR SL CA [1]. There are still some open issues. We will further discuss the open issues in this contribution.

- Discussion

In last meeting, RAN4 agreed to define interruption requirements on WAN when NR SL UE performs SL carrier addition/release and had no consensus on how to yet.

Issue 1-1: Interruption requirements when NR SL UE performs SL carrier addition/release - Agreements - Define interruptions to WAN carriers when NR SL UE performs SL carrier addition/release - Do not define interruptions to SL carriers when NR SL UE performs SL carrier addition/release - Interruptions to WAN - Interruption are defined with granularity of the NR slot length - FFS on interruption ratio and length Issue 1-2: Delay requirements of carrier addition/release for NR SL CA - Agreements - The delay requirements for SL component carrier addition/release are not needed for Mode 2 operation and up to UE implementation.

Regarding the interruption length at each SL carrier addition/release, we propose to use 1ms which is also the time assumed in NR CA. Considering that Uu and SL may be asynchronized, one more slot interruption should be allowed. The interruption length will be as following

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |

Proposal 1: The interruption length at each SL carrier addition/release is

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |

In last meeting, RAN4 also discussed how to define the requirements for Selection / Reselection of Synchronization Reference Source under NR SL CA operation. In general, we are fine to reuse the requirements defined for LTE V2X. As RAN1 has not started their work on SL CA and RAN2 had no agreement on Selection / Reselection of Synchronization Reference Source under NR SL CA operation yet, we think we can keep the status of this issue as it is and further discuss after there is more progress in RAN1/2.

Issue 1-3: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation FFS: - Option 1 (Huawei, LGE, MTK): - Wait for RAN1/RAN2 progress. - Option 2 (OPPO): - If N synchronization carriers can be configured based on RAN2 conclusion, the corresponding detection and measurement delay requirements should be scaled by N compared to that for the scenario when a single synchronization carrier is configured. - Option 3 (Ericsson): - The LTE V2X requirements defined in section 13.10 in TS 36.133 for selection/reselection of V2X SyncRefUE are reused for NR SL selection/reselection of SyncRefUE when supporting NR SL CA

Proposal 2: Regarding requirements on Selection / Reselection of Synchronization Reference Source under NR SL CA operation, wait more progress in RAN1/2 and then further discuss in RAN4.

- Summary

In this paper, we have some discussion on R18 SL CA. We have the following proposals:

Proposal 1: The interruption length at each SL carrier addition/release is

|  | NR Slot length (ms) of victim cell | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |

Proposal 2: Regarding requirements on Selection / Reselection of Synchronization Reference Source under NR SL CA operation, wait more progress in RAN1/2 and then further discuss in RAN4.

4 Reference

- R4-2310061, WF on NR_SL_enh2_part2, OPPO, RAN4#107 meeting, May 22th – 26th, 2023.