3GPP TSG-RAN WG4 Meeting # 108	R4-2311425

Toulouse, FR, 21st – 25th August 2023

Title:	TP to TR 36.718-02-01 Addition of CA_1-8 and CA_8-11

Source:	SoftBank

Agenda item:	9.1.2.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 36.718-02-01[1] to include CA_1-8 and CA_8-11 as requested in RAN4#107.

2. Reference

[1] TR36.718-02-01, LTE-A intra-band/inter-band Carrier Aggregation for x (x<=6) bands DL with y bands (y=1, 2) UL (Release 18) V0.0.4

Text Proposal

-- Start of TP --

### 5.2.x	CA_1-8

#### 5.2.x.1	Channel bandwidths per operating band for CA

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8B | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

#### 5.2.x.2	Co-existence studies

Coexistence requirements for CA_1-8 already exist in TS 36.101.

#### 5.2.x.3	∆TIB and ∆RIB values

The following ∆TIB and ∆RIB values are drawn from CA_1-8 in TS36.101:

Table 5.2.x.3-1: IB,c

| CA_1-8 | 1 | 0.3 |
| --- | --- | --- |
|  | 8 | 0.3 |

Table 5.2.x.3-2: R IB,c

| CA_1-8 | 1 | 0 |
| --- | --- | --- |
|  | 8 | 0 |

#### 5.2.x.4	REFSENS Requirements

No additional REFSENS requirements needed when compared to CA_1A-8A.

### 5.2.x	CA_8-11

#### 5.2.x.1	Channel bandwidths per operating band for CA

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8B-11A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 30 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |

#### 5.2.x.2	Co-existence studies

Coexistence requirements for CA_8-11 already exist in TS 36.101.

#### 5.2.x.3	∆TIB and ∆RIB values

The following ∆TIB and ∆RIB values are drawn from CA_8-11 in TS36.101:

Table 5.2.x.3-1: IB,c

| CA_8-11 | 8 | 0.3 |
| --- | --- | --- |
|  | 11 | 0.4 |

Table 5.2.x.3-2: R IB,c

| CA_8-11 | 8 | 0 |
| --- | --- | --- |
|  | 11 | 0 |

#### 5.2.x.4	REFSENS Requirements

No additional REFSENS requirements needed when compared to CA_8A-11A.

-- End of TP --