3GPP TSG-RAN WG4 Meeting # 108		R4-2311433

Toulouse, France, August 21 – August 25, 2023

Agenda item:		8.29.3.1

Source:		Samsung

Title:	Further discussion on general RRM impacts on Rel-18 MIMO evolution

Document for:		Discussion

- Introduction

In previous RAN4 meetings, we discussed the RRM impacts on each objective and had consensus on some objectives for RRM impacts. However, RAN4 had not reached agreement on RRM impacts on TDCP and part of SRS enhancement.

In this contribution, we continue to discuss the open issues.

- Discussion

CSI reporting enhancement - TDCP

TDCP: TRS-based TDCP (time-domain channel properties) reporting

During the discussion in RAN4#107 meeting, it is suggested to further check RAN1 progress then make the further decision.

It is captured in [1] as below:

Issue 1-1-1: Whether to specify RRM requirements for TDCP reporting? Agreements - FFS to define TDCP measurement accuracy requirements - FFS to define TDCP measurement delay requirements

In RAN1 last meeting RAN1#113, RAN1 finished the discussion for TDCP and achieve several agreements. By our understanding, there is no further RAN1 conclusion is needed to input to RAN4 to determine whether there are RRM impacts or not.

After checking RAN1 progress, our understanding is still that there is no RRM related impact. The definition of TDCP is to report the time-domain channel properties. It used the correlation from TRS symbols. For the reporting, it contains the quantization for the amplitude values and phase values.

For the quantization for the amplitude, the agreements from RAN1#113 are captured as below:

Agreement For the Rel-18 TRS-based TDCP reporting, regarding the alphabet for the quantization of wideband normalized amplitude value, support only (Alt3) N=2Q where Q=4, s=½ - Note: This does not preclude an “invalid” autocorrelation value report.

For the quantization for the phase, the agreements from RAN1#113 are captured as below:

Agreement For the Rel-18 TRS-based TDCP reporting, regarding the alphabet for the quantization of phase value, (Alt3) a given correlation phase value θ(D) is quantized to θ(D) based on the 4-bit (16-PSK) uniform quantization (full reuse of Rel-16 eType-II W2 phase quantization).

And for the time delay to apply the correlation, the agreements from RAN1#113 are captured as below:

Agreement For the Rel-18 TRS-based TDCP reporting, for a configured value of Y and a set of configured delay values {D1, …, DY}, for the n-th delay Dn (n=1, …, Y), the respective TDCP calculation is defined as wideband normalized correlation between two TRS symbols separated by Dn symbols Send a LS to RAN4 to solicit their inputs on whether additional description/definition is needed, e.g. averaging across RX ports.

The LS is sent to RAN4 to include such definition in [2].

From the definition, it can be understood that the reporting reflects to channel properties. The dedicated delay for multiple TRS symbols is configured by network. and the network will use the reporting to perform MIMO scheduling such as precoding. The performance reflects by the BLER and throughput curve in common. There is no RRM such as mobility or beam management. We think the new introduced channel TD correlation profile is not used for RRM. No need to define TDCP measurement delay and accuracy requirements. When demodulation discussion starts, it depends on the demodulation discussion whether to introduce the minimum requirements for TDCP.

For the question from RAN1 of RX ports, for FR2, one receiver branch is enough to be used for TDCP. We think no further definition is needed.

Proposal 1: Do not define TDCP measurement delay and accuracy requirements for RRM.

Proposal 2: To the question in the RAN1 LS, one receiver branch is enough to be used for TDCP in FR2. No further definition is needed on top of RAN1 definition.

SRS enhancement for 8TX UL

Issue 1-2-1: Whether to specify RRM requirements for Rel-17 Full slot SRS transmission? Conclusion: No consensus to specify RRM requirements for Rel-17 Full slot SRS transmission feature in the scope of Rel-18 MIMO evolution WI Issue 1-2-2: Whether to specify RRM requirements for Rel-18 SRS enhancement for 8TX UL? - FFS: - Reuse legacy SRS switching RRM requirements for 8TX UL

In previous meeting conclusion, RAN4 will not introduce Rel-17 full slot SRS transmission in this WI. For the 8TX SRS with TDM, the TDM factor s and number of SRS symbol can be 2 or 4 to support 8TX SRS feature. In this case, legacy SRS switching RRM requirements can be reused for 8TX UL. The spec should be updated to capture 8TX. And reuse the legacy requirements of Interruptions at NR SRS antenna port switching.

In last meeting, company suggested to use only 8 SRS symbols because of the cell coverage. But we think the similar situation happened for 2 TX and 4TX. There is no rule to restrict only 2 symbols for 2TX or 4 symbols for 4TX. The coverage is different. By understanding for RAN1 power control, the uplink power is ramping to the upper limit.

Proposal 3: The TDM factor s and number of SRS symbol can be 2 or 4 to support 8TX SRS feature. In this case, reuse the legacy requirements of Interruptions at NR SRS antenna port switching. The spec should be updated to capture 8TX.

- Conclusion

In this contribution, we provide our further consideration of RRM impacts for TDCP and SRS and our proposals are:

Proposal 1: Do not define TDCP measurement delay and accuracy requirements for RRM.

Proposal 2: To the question in the RAN1 LS, one receiver branch is enough to be used for TDCP in FR2. No further definition is needed on top of RAN1 definition.

Proposal 3: The TDM factor s and number of SRS symbol can be 2 or 4 to support 8TX SRS feature. In this case, reuse the legacy requirements of Interruptions at NR SRS antenna port switching. The spec should be updated to capture 8TX.

- Reference

[1] R4-2310178, WF on NR MIMO evolution RRM requirements, Samsung

[2] R4-2310471, LS to RAN4 on TDCP Agreement for Rel-18 MIMO, RAN1