3GPP TSG-RAN WG4 Meeting #108                              R4-2311441

Toulouse, France, August 21 – August 25, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS and RSD of n71 symmetric UL/DL 35MHz

Agenda Item:	7.26.2.1	Adding new channel bandwidth(s) support to existing NR bands [NR_bands_R18_BWs-Core]

Document for:	Approval

Introduction

In the RAN Plenary#100 meeting, the symmetric UL/DL 35MHz CBW of n71 was added to the basket of WID [1]. And in WF [2], it was agreed to keep the relevant PC2 work of new bandwidth of n71 within the “Adding new channel BWs support to existing NR bands” WI. In this paper, we analyzed the impact of UL CBW extending on Rx noise and proposed REFSENS of PC3 operation and RSD for PC2 operation.

Discussion

Rx band noise analysis

In the last RAN4 meeting discussion on symmetrical UL/DL 25MHz and 30MHz, we explained that the Rx noise increases significantly with the UL CBW extension due to the IMD3 product of the Tx signal and the IQ image falling into the Rx band [3]. And for UL 35MHz, the IMD3 and LO leakage of the Tx signal also fall into the Rx band, not just the IMD3 and IQ image of the Tx signal, as shown in the figure below. Therefore, Rx noise will further increase.

Using the traditional Rx band noise measurement method used for REFSENS analysis in the past, we found that the Rx noise of symmetric UL/DL 35MHz increased by 21dB for PC3 1Tx and 21.7dB for PC2 1Tx compared to asymmetric UL/DL.

Figure 1. Tx output image

Observation:

- For UL 35MHz, Rx noise further increase due to IMD products caused by Tx signal, IQ image and LO leakage.

- Compared to asymmetric UL/DL 35MHz, the Rx noise of symmetric UL/DL 35MHz increased by 21dB for PC3 1Tx and 21.7dB for PC2 1Tx.

REFSENS for PC3 and RSD for PC2

FE architecture

The two kinds of FE architecture shown in the Figure 2 are used to analyse REFSENS and RSD. For 2Tx/2Rx architecture, the mutual influence between primary Tx and diversity Tx was considered.

Figure 2. FE architecture

Calculation conditions

Calculation conditions shown in the Table 1 are used to RSD estimation.

Table 1. Calculation conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 23 dBm | For 1Tx PC3, at ANT1 |
|  | 26 dBm | For 1Tx PC2, at ANT1 |
|  | 23 dBm | For 2Tx PC2, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1 - ANT2 |
| Duplexer rejection | 55 | Tx to Rx, at Tx band |
|  | 55 | Tx to Rx, at Rx band |
|  | 45 | Tx to ANT, at Rx band |
| BPF | 45 | ANT to Rx, at Tx band |
| MRC | ANT1&ANT2 | Correlated |

Results

The calculation results are shown in Table 2.

Compared with existing REFSENS of asymmetric UL/DL, the degradation of REFSENS is around 19.9dB.

Table 2. REFSENS for PC3 and RSD for PC2

|  |  |  | Asymmetric | Symmetric |
| --- | --- | --- | --- | --- |
|  | DL CBW | MHz | 35 | 35 |
| PC3 | Scaling REFSENS | dBm | -88.4 | -88.4 |
|  | Desense | dB | 7.9 | 27.8 |
|  | REFSENS | dBm | -80.5 | -60.6 |
| PC2 | RSD 1Tx | dB | 2.7 | 3.9 |
|  | RSD 2Tx | dB | 6.8 | 7.8 |

Proposal:

The REFSENS for PC3 and RSD for PC2 are proposed in bold in Table 2.

Conclusion

In this contribution, we presented the Rx band noise analysis of symmetric UL/DL 35MHz for n71 and proposed the calculation results of the REFSENS for PC3 and the RSD for PC2.

Observation:

- For UL 35MHz, Rx noise further increase due to IMD products caused by Tx signal, IQ image and LO leakage.

- Compared to asymmetric UL/DL 35MHz, the Rx noise of symmetric UL/DL 35MHz increased by 21dB for PC3 1Tx and 21.7dB for PC2 1Tx.

Proposal:

The REFSENS for PC3 and RSD for PC2 are proposed in bold in Table 2.

Table 2. REFSENS for PC3 and RSD for PC2

|  |  |  | Asymmetric | Symmetric |
| --- | --- | --- | --- | --- |
|  | DL CBW | MHz | 35 | 35 |
| PC3 | Scaling REFSENS | dBm | -88.4 | -88.4 |
|  | Desense | dB | 7.9 | 27.8 |
|  | REFSENS | dBm | -80.5 | -60.6 |
| PC2 | RSD 1Tx | dB | 2.7 | 3.9 |
|  | RSD 2Tx | dB | 6.8 | 7.8 |

References

[1] RP-231010 “Basket WID on adding channel bandwidth support to existing NR bands”, Ericsson, 3GPP TSG RAN Meeting #100, Taipei, June 12 – 14, 2023

[2] R4-2306544 “WF on FDD HPUE Basket WIs”, China Unicom, 3GPP TSG-RAN4 Meeting #106-bis-e, Online, 17th – 26th April 2023

[3] R4-2308456 “REFSENS and RSD of n71 symmetrical UL/DL 25MHz and 30MHz”, Murata, 3GPP TSG-RAN4 Meeting #107, Incheon, May 22 – 26, 2023