3GPP RAN WG4 Meeting #108		R4-2311445

Toulouse, FR, Aug 21 – 25th, 2023

Agenda item:	7.34.1

Source:	CAICT

Title:	Discussions on NR NTN 30MHz bandwidth

WI/SI:	NR_NTN_channel_30MHz

Release:	Rel-18

Document for:	Discussion

1	Introduction

During the RAN#100 meeting, the work item “Introduction of the satellite L-/S-band” had been approved[1]. The objectives can be seen as follow

The objective of the core part is to:

- Define 30 MHz channel bandwidth for n256 and n255 defined for 5G NR-NTN;

- Introduce the corresponding SAN and UE RF core requirements, limited to clause 5 (Operating Bands and Channel Arrangement) requirements for these specifications.

2	Disscussion

2.1	Channel arrangement

Since the work is limited in clause 5, so the changes needed to be discussed are Maximum transmission bandwidth configuration、Minimum guardband and transmission bandwidth configuration、Channel bandwidths for each NTN satellite band, the rest of clause 5 can stay unchanged.

One of the objectives of the core part of this WID is to define 30MHz bandwidth for n255 and n256. Since band n256 is 1980-2010MHz for UL and 2170-2200MHz for DL. The total operating UL/DL bandwidth of band n256 is 30MHz, so the defined channel bandwidth equals to the operating band. For n255, the UL/DL operating bandwidth is 34MHz, 4MHz can be used as guardband to the adjacent channel, so the channel config can be set as table1 to table3.

Table 1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Table 2: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

Table 3: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n256 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n255 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |

Proposal 1:	Specify the channel configuration of n255 and n256 as Table1 to Table3.

3	Conclusions

Proposal 1:	Specify the channel configuration of n255 as Table1 to Table3.

Table 1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Table 2: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

Table 3: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n255 | 30 |  | 10 | 15 | 20 | 30 |

.

4	References

- RP-221499, 30 MHz Channel Bandwidth for NR NTN in FR1, THALES.