3GPP TSG-RAN WG4 Meeting #108                      					R4-2311458

Toulous, France, 21– 25 Aug, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 on BS RF requirements

Agenda Item:	8.13.3

Document for:	Approval

Introduction

In the previous meetings, RAN4 has made progress on BS RF requirements. This paper presents a text proposal on ATG BS RF requirements for TR 38.876 based on [1].

TP for TR 38.876

<Start of Text Proposal>

- 7.2.1 ATG BS class and BS type

This WI study focus on FR1 bands only. After some discussion, the three BS FR1 types are specified for ATG BS, i.e. ATG BS type type1-C, 1-H and 1-O are defined.

< Non-changed part is omitted >

- <<Next Change>>

#### 7.2.2.3 Unwanted emission requirements

Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 (=0.5%) of the total mean transmitted power. For ATG BS, it is agreed to reuse the same requirement defined in TS38.104 [5] which is following Recommendation ITU-R SM.328.

Adjacent Channel Leakage Power Ratio (ACLR)

Based on the coexistence simulation results, it is sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

< Non-changed part is omitted >

- <<Next Change>>

#### 7.2.3.5 Receiver intermodulation

Regarding the Rx intermodulation,  although it is not likely to have the coexistence scenario in which ATG BS is subject to intermodulation interferers with the power levels defined in FR1 WA Rx intermodulation requirement, Rx intermodulation still dimension some aspects of receiver design (e.g. receiver linearity to minimize the interfering signal intermodulation fall into the wanted signal). Therefore, it’ s still useful to keep the receiver intermodulation requirements and it is agreed to reuse the Rx IM requirement for FR1 WA BS for ATG BS.

<End of Text Proposal>

Reference

[1] 3GPP TR 38.876 V0.4.0, 2023.5