3GPP TSG-RAN WG4 Meeting #108                                                      R4-2311462

Toulous, France, 21– 25 Aug, 2023

Source:	Huawei, Hisilicon

Title:	TP for TR 38.718-02-01 on correct CA_n3A-n75A

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

The title of the Table 5.35.1.1-1 and Table 5.35.1.2-1 are incorrect for CA_n3A-n75A in TR 38.718-02-01 V0.6.0.The uplink configuration are incorrect for CA_n3A-n75A since n75 has no uplink. This TP is to correct them.

Text Proposal

<TP for TR 38.718-02-01>

<<Start of Change>>

5.35	  CA_n3-n75

5.35.1	Common for 1 band UL and 2 bands UL CA

5.35.1.1	Operating bands for CA

Table 5.35.1.1-1:  CA band combination of band n3+n75

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n75 | NA |  |  | 1432 MHz | – | 1517 MHz | SDL |

5.35.1.2	Channel bandwidths per operating band for CA

Table 5.35.1.2-1: Supported bandwidths per CA band combination of band n3+n75

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n75A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

<<End of Change>>