3GPP TSG-RAN WG4 Meeting #108                                                    	R4-2311466

Toulous, France, 21– 25 Aug., 2023

Source:	Huawei, Hisilicon, Rogers

Title:	TP to TR 37.718-21-11 Addition of DC_2A-5A_n41A

Agenda item:		7.4.2

Document for:	Approval

# 1. Introduction

The new basket WID for DC_R18_2BLTE_1BNR_3DL2UL was approved in RAN #99 meeting [1]. This contribution provides a TP for TR 37.718-21-11 to finish the requirements for DC_2A-5A_n41A.

2 Discussion

We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1.

Figure 1 Main/diversity antenna topology

References

[1] RP-230473, WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)，Huawei, HiSilicon ，RAN#99，Mar.  2023

Text Proposal

<TP for TR 37.718-21-11>

<Start of Text Proposal>

5.x	DC_2-5_n41

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A_n41A | DC_2A_n41A DC_5A_n41A |

5.x.2	Co-existence studies

Table 5.x.2-1 lists the UL configuration of DC_2_n41 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: UL DC_2_n41 harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 586 | 840 | 4346 | 4600 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1010 | 1324 | 3082 | 3530 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6196 | 6510 | 6842 | 7290 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2860 | 3234 | 5578 | 6220 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8046 | 8420 | 9338 | 9980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1680 | 1172 | 8692 | 9200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8910 | 8074 | 5144 | 4710 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4370 | 3668 | 738 | 170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11834 | 12670 | 9896 | 10330 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11188 | 11890 | 10542 | 11110 |

There is no harmonic or IMD impact from UL DC_2_n41 affecting DL band 2, 5 and n41.

Table 5.x.2-2 lists the UL configuration of DC_5_n41 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-2: DC_5_n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1647 | 1866 | 3320 | 3539 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1042 | 798 | 4143 | 4556 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4144 | 4388 | 5816 | 6229 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 218 | 51 | 6639 | 7246 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4968 | 5237 | 8312 | 8919 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3732 | 3294 | 6640 | 7078 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9936 | 9135 | 900 | 606 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6422 | 5790 | 2445 | 2908 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10808 | 11609 | 5792 | 6086 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9136 | 9768 | 7464 | 7927 |

The 3rd order harmonics of band 5 may affect DL band n41.

The IMD3 from UL_5_n41 may affect DL band 5.

The IMD5 from UL_5_n41 may affect DL band n41.

There is no IMD impact from UL_5_n41 affecting DL band 2.

5.x.3	∆TIB and ∆RIB values

For DC_2-5_n41, the TIB,c and RIB,c values are reused from CA_n2-n5-n41 and given in the tables below.

Table 5.x.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-5_n41 | 0.5 | 0.6 | 0.41 / 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.x.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-5_n41 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.x.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

<End of Text Proposal>