3GPP TSG-RAN WG4 Meeting #108                                                    	R4-2311472

Toulous, France, 21– 25 Aug., 2023

Source:	Huawei, Hisilicon, Rogers

Title:	TP for TR 38.718-02-01: CA_n2(2A)-n7A

Agenda item:			7.11.2

Document for:	Approval

# 1. Introduction

The new basket WID for NR_CADC_R18_2BDL_xBUL was approved in RAN #100 meeting [1]. This contribution provides a TP for TR 38.718-02-01 to finish the requirements for CA_n2(2A)-n7A.

References

[1] RP-231115, Revised WID Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2),  ZTE，June 2023

Text Proposal

<TP for TR 38.718-02-01>

<Start of Text Proposal>

5.x	CA_n2(2A)-n7A

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n2+n7

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n2+n7

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2(2A)-n7A | CA_n2(2A)-n7A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |

5.x.1.3	UE co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n2(2A)-n7. It is shown that there are no harmonic issues to consider.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |

Based on above table, there is no uplink harmonic issues.

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9950 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |

Based on above table, there is no harmonic mixing issues.

5.x.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Table 5.x.1.3.1-1 gives a non-contiguous uplink IMD interference analysis

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 5 | 5 | 50 | 1850 | 1910 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 1850 | 1860 | 1910 | 1910 | 1900 | 1850 |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 10 | 60 | 3710 | 3810 | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 1790 | 1970 | 5560 | 5720 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 20 | 120 | 3640 | 3880 | 7410 | 7630 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 1730 | 2030 | 5490 | 5790 | 9260 | 9540 |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 30 | 180 | 3580 | 3940 | 7340 | 7700 |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1670 | 2090 | 5430 | 5850 | 9190 | 9610 |

There are no IMD products affecting n7 DL.

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n7, The TIB,c and RIB have been specified in current spec.

5.x.1.5	REFSENS requirements

According to the co-existence study, no additional MSD requirements should be defined.

5.x.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2(2A)-n7A | 23 | +2/-3 |

5.x.2.2	UE co-existence studies

Table 5.x.2.2-1 lists Band n2 + Band n7 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n2 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1130 | 1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9620 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1180 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8430 | 8090 | 5140 | 4830 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4010 | 3680 | 730 | 410 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12190 | 9900 | 10210 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

There is no IMD issue for CA_n2-n7 with 2UL.

5.x.2.3	REFSENS requirements

Based on co-existence studies on 5.x.2.2, no need to define exceptional REFSENS requirements.

<End of Text Proposal>