3GPP TSG-RAN WG4 Meeting#108      	R4-2311483

Toulouse, 21 – 25 Aug, 2023

Title:	V2X con-current operation UEtoUE co-ex requirement

Agenda item:	8.1.3

Source:	Nokia

Document for:	Approval

# 1	Background

In RAN4#106 LTE 2UL interband CA UEtoUE co-ex requirement was simplified [1][2][3]. In RAN4#[107] similar simplification was agreed to NR [4][5][6] and EN-DC [7][8][9].

# 2 	Discussion

While preparing to RAN4#108 we noticed that V2X con-current operation would benefit also from UEtoUE co-ex requirement simplification.

In a process of creating the CR we noticed that in V2X con-current operation the E-UTRA band and NR band n47 do not have common protected bands list therefore simplification was not possible.

We are asking from RAN4 should E-UTRA band and NR band n47 have common protected bands list? We think they should and when examining the Table 6.5E.3.2.2-1: Requirements for inter-band con-current V2X operation this is mostly the case indeed, at least one difference was found as V2X_40_n47 protects band 27

| V2X_40_n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 27, 28, 34, 39, 41, 42, 44, 45, 65, 68, 72, 73 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |

But n47 do not, we take this as an error.

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR Band n71, n77, n78, n79, n105 | FDL_low | - | FDL_high | -50 | 1 |  |

Second question is that is V2X_20_n38 valid configuration as it does not have band 47 or n47.

| V2X_20_n38 | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |

So in this contribution we are proposing to perform the UEtoUE co-ex requirement simplification for Table 6.5E.3.2.2-1: Requirements for inter-band con-current V2X operation in 38.101-3 and for Table 6.5E.3.3-1: Requirements for inter-band con-current V2X operation in 38.101-1 and Table 6.6.3G-0: Requirements for inter-band con-current V2X operation in 36.101. Starting from REL16.

# 3	Conclusion

Proposal: Perform the UEtoUE co-ex requirement simplification for Table 6.5E.3.2.2-1: Requirements for inter-band con-current V2X operation in 38.101-3 and for Table 6.5E.3.3-1: Requirements for inter-band con-current V2X operation in 38.101-1 and Table 6.6.3G-0: Requirements for inter-band con-current V2X operation in 36.101. Starting from REL16.

# 4	References

[1] R4-2303509	LTE interband 2UL CA co-ex simplication R16,

[2] R4-2303510	LTE interband 2UL CA co-ex simplication R17

[3] R4-2303511	LTE interband 2UL CA co-ex simplication R18

[4] R4-2308826	NR interband 2UL CA co-ex simplication R16

[5] R4-2307862	NR interband 2UL CA co-ex simplication R17

[6] R4-2307863	NR interband 2UL CA co-ex simplication R18

[7] R4-2310373	EN-DC interband 2UL co-ex simplication R16

[8] R4-2310374	EN-DC interband 2UL co-ex simplication R17

[9] R4-2310375	EN-DC interband 2UL co-ex simplication R18