3GPP TSG-RAN4 Meeting #108	R4-2311486

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5995 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 36.101: uplink inter-band carrier aggregation UEtoUE co-ex table cell border correction R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Cell borders are not correct in Table |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell borders are corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are ambiquous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 6.6.3.2A-0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

Table 6.6.3.2A-0: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| CA_1-5 |  | Frequency range |  | 859 |  |  |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
| CA_1-8 |  | Frequency range |  | 860 |  |  |  | - |  | 890 |  | -40 |  | 1 |  | 3, 11 |  |
| CA_1-11 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_1-18 |  | Frequency range |  | 758 |  |  |  | - |  | 799 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 799 |  |  |  | - |  | 803 |  | -40 |  | 1 |  | 3 |  |
|  |  | Frequency range |  | 860 |  |  |  | - |  | 890 |  | -40 |  | 1 |  |  |  |
|  |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2545 |  |  |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_1-19 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-20 | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_1-21 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_1-26 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 703 |  |  |  | - |  | 799 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 799 |  |  |  | - |  | 803 |  | -40 |  | 1 |  | 3 |  |
| CA_1-28 |  | Frequency range |  | 470 |  |  |  | - |  | 694 |  | -42 |  | 8 |  | 3, 22 |  |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
| CA_2-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_2-13 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_2-14 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_3-5 |  | Frequency range |  | 859 |  |  |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
| CA_3-8 |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 4, 11 |  |
|  |  | Frequency range |  | 860 |  |  |  | - |  | 890 |  | -40 |  | 1 |  | 3,11,17 |  |
| CA_3-11 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-18 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-19 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-20 | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_3-21 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  |  | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_3-26 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_3-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_4-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_4-13 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_4-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  |  | 803 | -50 | 1 |  |
| CA_5-7 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-12 |  | Frequency range |  | 859 |  |  |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
| CA_5-17 |  | Frequency range |  | 859 |  |  |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
|  |  | Frequency range |  | 859 |  |  |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
| CA_5-40 |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 4 |  |
| CA_7-26 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  |  |  |
| CA_7-28 | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  |  | Frequency range |  | 773 |  |  |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
| CA_8-20 |  | Frequency range |  | 758 |  |  |  | - |  | 788 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
| CA_8-28 | Frequency range | 470 |  | - | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5, 11 |
| CA_11-18 | Frequency range | 860 |  | - | 890 | -40 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_11-26 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_13-66 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-30 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-66 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  |  | Frequency range |  | 799 |  |  |  | - |  | 805 |  | -35 |  | 0.00625 |  | 3 |  |
| CA_18-28 |  | Frequency range |  | 470 |  |  |  | - |  | 710 |  | -26.2 |  | 6 |  | 23 |  |
|  |  | Frequency range |  | 758 |  |  |  | - |  | 773 |  | -32 |  | 1 |  | 3 |  |
|  |  | Frequency range |  | 773 |  |  |  | - |  | 799 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 799 |  |  |  | - |  | 803 |  | -40 |  | 1 |  | 3 |  |
|  |  | Frequency range |  | 860 |  |  |  | - |  | 890 |  | -40 |  | 1 |  |  |  |
|  |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  | 3 |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 4 |  |
|  |  | Frequency range |  | 2545 |  |  |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_19-21 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19-42 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_21-28 |  | Frequency range |  | 470 |  |  |  | - |  | 710 |  | -26.2 |  | 6 |  | 23 |  |
|  |  | Frequency range |  | 773 |  |  |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 4, 5 |  |
|  |  | Frequency range |  | 2545 |  |  |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  |  |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| CA_21-42 |  | Frequency range |  | 945 |  |  |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_26-46 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_26-48 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_28-40 |  | Frequency range |  | 758 |  | - |  |  |  | 773 |  | -32 |  | 1 |  |  |  |
|  |  | Frequency range |  | 773 |  | - |  |  |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 4 |  |
| CA_28-41 |  | Frequency range |  | 470 |  | - |  |  |  | 694 |  | -42 |  | 8 |  | 3, 22 |  |
|  | Frequency range | 470 | - |  | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 | - |  | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - |  | 1915.7 | -41 | 0.3 | 4, 5, 18 |
| CA_28-42 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_40-42 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_41-42 | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: Void NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE10: Void NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: Void NOTE13: Void. NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: Void NOTE 21: Void NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48.NOTE 24: Void NOTE 25: Void |  |  |  |  |  |  |  |  |

***************************** End of changes ************************************