3GPP TSG-RAN4 Meeting #108 	R4-2311497

# Toulouse, France, 21st August 2023 - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_RF_FR1-Core] Correction to EVM measurement point for DFTs-OFDM DM-RS Type 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DFTs-OFDM DMRS Type-2 to achieve Low-PAPR by Pi/2 shift BPSK is newly defined from 38.211 Release 16. Conventional DMRS has its modulated symbols mapped directly to the frequency domain in a UE, but DFTs-OFDM DM-RS Type 2 has its modulated symbols mapped to the frequency domain after DFT instead. This is the same process as DFT-s-OFDM PUSCH and PUCCH. Calculating the EVM of DFTs-OFDM DM-RS Type 2 according to the current EVM measurement block diagram in Figure F.1-1 cannot provide a correct EVM result, the constellation of modulated symbols (Pi/2 shift BPSK) being incorrectly calculated prior to IDFT. It is appropriate to correct the EVM measurement block diagram to calculate EVM by the constellation after IDFT, similarly to DFT-s-OFDM PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figure F.1-1 was corrected as follows: - “DFT-s-OFDM DM-RS Type 2” was added to the input of DFT in DUT section. - “DM-RS” at the input of Tone map in DUT section was replaced by “CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1”. - “DFT-s-OFDM DM-RS Type 2” was added to the output of IDFT in Test equipment section. - “DM-RS” at the output of Tx-Rx chain equalizer in Test equipment section was replaced by “CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper EVM can be calculated as used symbols are fully corrupted (due to missing IDFT block in the TE section of the block diagram in Figure F.1-1). May lead to the wrong interpretation by readers that a DFT block for DFTs-OFDM DM-RS Type 2 is not used by UEs (as missing in the DUT section of the block diagram in Figure F.1-1). . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

# F.1	Measurement Point

Figure F.1-1 shows the measurement point for the unwanted emission falling into non-allocated RB(s) and the EVM for the allocated RB(s).

DFT

IFFT

TX

Front

-

-end

Channel

RF

correction

FFT

Tx

-

Rx chain

equalizer

In

-

band

emissions

meas.

0

0

IDFT

DUT

Test equipment

EVM meas. of

CP-OFDM PUSCH, PUCCH, and CP-OFDM DM-RS, DFT-s-OFDM DMRS Type 1

CP-OFDM PUSCH, PUCCH, and CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1

Tone  map

DFT-s-OFDM PUSCH, PUCCH, and DFT-s-OFDM DM-RS Type 2

EVM meas. of DFT-s-OFDM PUSCH, PUCCH, and DFT-s-OFDM DM-RS Type 2

DFT

IFFT

TX

Front

-

-end

Channel

RF

correction

FFT

Tx

-

Rx chain

equalizer

In

-

band

emissions

meas.

DFT-s-OFDM PUSCH

0

0

IDFT

DUT

Test equipment

CP-OFDM PUSCH, PUCCH and   DM-RS

CP-OFDM PUSCH, PUCCH and DM-RS

Tone  map

DFT-s-OFDM PUSCH, PUCCH

Figure F.1-1: EVM measurement points

## <<End of change>>